GREY STRUCTURE MATERIAL ESTIMATE

Double-storey house with mumty (2nd-floor room) - Plot No. 79/K

Quantities to procure (upper estimate, including wastage)

| Cement 1,560 bags (50 kg) | Bricks 125,000 nos | Steel (Grade 60) 8.08 tons | Crush 4,200 cft | Sand (all) 7,750 cft |
| --- | --- | --- | --- | --- |

| Client | Mr. Saleem |
| --- | --- |
| Architect | Faisal Associates, WAPDA City, Faisalabad - working drawings sheets 01-24 (16-08-2026) |
| Structural engineer | Fakhar Associates - M. Fakhar Iqbal Sheikh (PEC Civil 11697), drawings 2026/L-20/S-01 to S-09 (14-08-2026) |
| Covered area (as per drawings) | Ground 1,898 sft + First 1,844 sft + Second 362 sft = 4,104 sft |
| Scope | Complete grey structure from excavation to roof parapet: foundations, plinth, brickwork, all R.C.C, steel, plaster, sub-floors, boundary walls, septic tank, manholes, overhead water tank, sewerage / rain-water / water-supply / gas pipes and electrical conduits |
| Estimate basis | Upper estimate - quantities measured from the vector drawing files, upper end of standard coefficient ranges, standard wastage added |
| Date | 30 September 2026 |

# 1. Final material requirement (what to buy)

The quantities below are the total for the complete grey structure described on the cover, including standard wastage. Where a standard gives a range, the upper value has been used, as requested. "Net" is the measured quantity; "To procure" is the net quantity plus wastage, rounded up. Fill in your local rates to get the cost.

| # | Material | Net qty | Waste | To procure | Unit | Remarks / purchase guide | Rate (Rs) | Amount (Rs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cement (OPC, 50 kg bag) | 1,481 | 5% | 1,560 | bags | Buy fresh stock per stage; do not store more than 4-6 weeks |  |  |
| 2 | Bricks - 1st class burnt clay (9" x 4½" x 3") | 118,495 | 5% | 125,000 | nos | Standard 13.5 bricks per cft of brickwork |  |  |
| 3a | Steel Grade 60 deformed - 10 mm (#3) | 4,406 | 5% | 4,630 | kg | Laps & hooks already included in net |  |  |
| 3b | Steel Grade 60 deformed - 12 mm (#4) | 2,165 | 5% | 2,280 | kg | Laps & hooks already included in net |  |  |
| 3c | Steel Grade 60 deformed - 20 mm (#6) | 976 | 5% | 1,030 | kg | Laps & hooks already included in net |  |  |
| 3d | Steel Grade 60 deformed - 6 mm (#2) - rings/stirrups | 132 | 5% | 140 | kg | Laps & hooks already included in net |  |  |
| 3 | Steel - TOTAL | 7,679 | 5% | 8,080 | kg | = 8.08 metric tons |  |  |
| 4 | Binding wire (annealed, 18 gauge) | 77 | - | 85 | kg | About 10 kg per ton of steel |  |  |
| 5 | Crush / coarse aggregate (¾" down) | 3,984 | 5% | 4,200 | cft | ≈ 42 tractor-trolleys of 100 cft |  |  |
| 6 | Sand for concrete (Chenab / Lawrencepur) | 2,014 | 10% | 2,250 | cft | Coarse clean sand for RCC & PCC |  |  |
| 7 | Sand for masonry mortar & plaster (Ravi) | 4,415 | 10% | 4,900 | cft | Fine sand; sieve for plaster |  |  |
| 8 | Fill sand under ground floors (4") | 590 | incl. | 600 | cft | Any clean sand / Ravi sand |  |  |
| 9 | Earth filling (plinth + porches, compacted) | 6,405 | incl. 20% | 6,450 | cft | Re-use ~3,200 cft surplus excavated earth; import about 3,250 cft |  |  |
| 10 | Bitumen (hot, vertical DPC 2 coats) | 304 | 5% | 320 | kg | Inner face of external plinth walls |  |  |
| 11a | Sewerage (UPVC) 5" | 156 | 10% | 172 | ft | ≈ 9 lengths of 20 ft |  |  |
| 11b | Sewerage (UPVC) 4" | 128 | 10% | 141 | ft | ≈ 8 lengths of 20 ft |  |  |
| 11c | Sewerage (UPVC) 3" | 218 | 10% | 240 | ft | ≈ 12 lengths of 20 ft |  |  |
| 11d | Rain water (UPVC) 3" | 112 | 10% | 124 | ft | ≈ 7 lengths of 20 ft |  |  |
| 11e | Water supply (PPR) 1"-1¼" | 118 | 10% | 130 | ft | ≈ 7 lengths of 20 ft |  |  |
| 11f | Water supply (PPR) ¾" | 370 | 10% | 407 | ft | ≈ 21 lengths of 20 ft |  |  |
| 11g | Water supply (PPR) ½" | 140 | 10% | 154 | ft | ≈ 8 lengths of 20 ft |  |  |
| 11h | Hot water (PPR) ¾" | 360 | 10% | 397 | ft | ≈ 20 lengths of 20 ft |  |  |
| 11i | Hot water (PPR) ½" | 80 | 10% | 88 | ft | ≈ 5 lengths of 20 ft |  |  |
| 11j | Gas (GI/PPR-gas) ½"-¾" | 190 | 10% | 209 | ft | ≈ 11 lengths of 20 ft |  |  |
| 12a | Electrical conduit (PVC) ¾" | 2,900 | 10% | 3,191 | ft | ≈ 320 lengths of 10 ft |  |  |
| 12b | Electrical conduit (PVC) 1" | 1,390 | 10% | 1,530 | ft | ≈ 153 lengths of 10 ft |  |  |
| 12c | Electrical conduit (PVC) 1½" | 85 | 10% | 94 | ft | ≈ 10 lengths of 10 ft |  |  |

Pipe lengths are for pipes only; fittings, manhole covers, boxes and accessories are listed in Sections 7 and 8. Optional items (roof insulation with brick tiles, underground water tank, external paving) are NOT included above - see Section 9.

# 2. Stage-wise requirement (order material in phases)

Buying material stage by stage reduces theft, damage and cement ageing. Quantities include wastage.

| Stage | Cement (bags) | Bricks (nos) | Steel (kg) | Crush (cft) | Concrete sand (cft) | Mortar/plaster & fill sand (cft) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Foundation & plinth | 255 | 25,798 | 1,397 | 893 | 468 | 737 |
| 2. Ground-floor walls & GF roof | 299 | 28,556 | 2,754 | 914 | 479 | 731 |
| 3. First-floor walls & FF roof | 307 | 31,403 | 2,702 | 881 | 461 | 801 |
| 4. Mumty (2F), parapets & roof tank | 149 | 14,974 | 907 | 344 | 180 | 405 |
| 5. Plaster, sub-floors & floor bases | 358 | 0 | 0 | 850 | 445 | 2,029 |
| 6. External works (boundary, septic, manholes) | 187 | 23,689 | 302 | 302 | 183 | 802 |
| TOTAL (incl. wastage) | 1,555 | 124,420 | 8,062 | 4,183 | 2,216 | 5,506 |

# 3. Project data read from the drawings

| Item | Data used in this estimate |
| --- | --- |
| Structural system | Load-bearing brick masonry (9" external & main walls, 4½" partitions) with 5½" R.C.C slabs; concealed beams CB-1 (12" x 9½") and CB-2 (9" x 9½") carry first-floor walls that do not sit on ground-floor walls; one R.C.C column at the car-porch front corner |
| Plot / building | Plot about 37' wide x 65' deep (left boundary skewed); building 37'-3" x 51'-7" on ground floor; corner plot with main gate (front) and side gate (left) |
| Levels | Road ±0; car porch +1'-3"; finished floor +2'-9"; floor-to-floor 12'-0" (11'-0" clear + 1'-0" slab band); mumty 9'-0" clear; terrace parapet 2'-6"; mumty parapet 1'-6". Natural surface level "as per site" - taken 1'-6" below road as drawn |
| Specifications (S-01 notes) | Deformed steel Grade 60; slab & beam R.C.C 1:2:4 (3000 psi); column R.C.C 1:1.5:3 (4000 psi); covers: slab ¾", beams, columns & foundation 1½"; foundation masonry 1:6; P.C.C 1:4:8; D.P.C 1½" 1:2:4 with vertical DPC (¾" plaster 1:3 + 2 coats hot bitumen) |
| Foundation types (S-02) | 1-1 external property-line wall (eccentric): P.C.C 2'-9" x 3", R.C.C footing 2'-6" x 6"; 2-2 external walls: P.C.C 3'-0", footing 2'-6"; 3-3 internal 9" walls: P.C.C 3'-6", footing 3'-0"; 4-4 4½" walls: P.C.C 2'-6", footing 2'-0". Footing steel #3@6"/#3@8". Brick steps 22½" (6"), 18" (6"), 13½" (12"), 9" wall; plinth beam 9"x9" 2#4+2#4, #2@10" rings; depth 2'-6" below NSL |
| Measured foundation lengths | Type 1-1: 40.3 ft; type 2-2: 133.4 ft; type 3-3: 86.2 ft; type 4-4: 110.8 ft; total centre-line 370.7 ft (38 wall runs - Appendix A) |
| Slabs (S-06/07/08) | 5½" thick. GF roof 1,705.0 sft; FF roof 1,650.7 sft; mumty roof 353.2 sft (net of stair openings, open shaft and terrace) |
| Concealed beams | CB-1: GF roof 81.7 ft (5 beams) + FF roof 15.5 ft (1); CB-2: GF roof 48.5 ft (5) + FF roof 102.0 ft (7) + mumty roof 30.3 ft (2); 23 bed plates 9"x3'x3" (3#4) |
| Stair (S-03 + sheet 06/07) | Zig-zag (dog-legged), 2 flights per storey, 22 risers of 6.57", treads 10½", width 3'-6", 6" waist with #4@5" main bars, #3@9"/12" distribution, 6" landing, base footing |
| Lintels (S-04/S-05) | DL-1 9"x9" 4#4 #3@6"; DL-2 4½"x9" 4#4 #2@6"; WL-1 9"x9" 4#4; WL-2 9"x9" 2#4+4#4; WL-3 4½"x9"; WL&DL-1 9"x9"; VL-1 9"x9" 4#3 #3@10". Key plans show 21 (GF), 24 (FF) and 6 (2F) lintels |
| Wall heights used | GF 11'-0" (DPC to slab soffit); FF 11'-6.5"; mumty 9'-6.5"; terrace parapet 3'-0.5"; mumty parapet 2'-0.5" (from slab top, incl. roof build-up) |
| Services on drawings | Sewerage in 3"/4"/5" UPVC with 9 manholes and septic tank connected to main sewer; 4 rain-water pipes in 9"x6" ducts; cold/hot water & gas lines (sheets 21-24); electrical layout with legend (sheets 13-16); overhead water tank on mumty roof |

# 4. How the estimate was prepared

- All 40 sheets were studied: 9 structural sheets (foundation layout & sections, stair, lintel key plans & sections, three slab reinforcement plans, beam sections) and 31 architectural/services sheets (plans, elevations, wall section, boundary walls, bath details, electrical, sewerage, sanitary).

- The structural PDFs are AutoCAD vector files, so wall lines were measured directly from the file geometry instead of scaling paper prints. Scale was calibrated on the 13'-0" x 13'-0" Bed-1 (1 ft = 33.053 drawing units); 9" and 4½" walls measured 9.0" and 4.5". The hand-built wall network (Appendix A) matches the raster-measured wall area within 1%.

- Foundation volumes use exact polygon unions of every layer (PCC, footing, each brick step, plinth), so overlaps at the many T-junctions are not double counted (unions are about 12% less than simple length x width).

- First-floor walls were measured from the lintel key plan (S-04) and openings added back; mumty walls from the 2nd-floor plan. Slab outlines, stair openings, beams and bed plates were measured on the slab sheets.

- Slab steel: every bar set on S-06/S-07/S-08 (spacing label, its distribution dimension line and bar length) was listed individually - 44 sets (GF roof), 40 sets (FF roof), 11 sets (mumty roof) - see Appendix B.

- Sewer, water and gas pipe runs were measured on the colour-coded services sheets (sheet scale calibrated separately); vertical stacks, risers and drops were added. Electrical conduits are based on a point count of the electrical plans.

## 4.1 Standard coefficients used (upper end of range)

| Coefficient | Value | Remarks |
| --- | --- | --- |
| Bricks per cft of brickwork | 13.5 nos | Standard local brick with ⅜" joints |
| Mortar per cft of brickwork | 0.30 cft wet = 0.40 cft dry | Range 0.25-0.30 wet; upper value used |
| Dry volume factor - concrete | 1.57 | Range 1.52-1.57 |
| Dry volume factor - mortar | 1.33 |  |
| Plaster extra for joints/uneven surface | +25% | Range 20-30% |
| Cement bag | 50 kg = 1.25 cft |  |
| Steel weights (lb/ft) | #2 0.167, #3 0.376, #4 0.668, #6 1.502 | 1 lb = 0.4536 kg |
| Wastage | Cement 5%, sand 10%, crush 5%, bricks 5%, steel 5%, pipes 10% | Laps, hooks, cranks, chairs are inside the net steel |
| Mortar ratios | Foundation 1:6 (drawing); 9" walls 1:5; 4½" walls, parapets, pillars, tanks 1:4; plaster 1:4 (walls), 1:3 (ceilings, water-retaining) | Superstructure ratios not given on drawings - assumed |

## 4.2 Assumptions (please confirm on site)

- Natural surface level taken 1'-6" below road (drawings: "as per site"). Every 6" higher/lower NSL changes earth filling by about 820 cft and foundation brickwork slightly.

- Foundation R.C.C footing concrete taken 1:2:4 (mix not stated).

- Septic tank enlarged to a standard 6'-0" x 4'-0" x 5'-6" deep (internal, 2 chambers) - the drawing shows only a 4'-10" x 3'-5" symbol, small for 8 WCs.

- Overhead water tank taken as R.C.C 1,000 gallons (8'x5'x4'-6" internal) on the mumty roof - drawings show a symbol only. If you use plastic tanks, delete Section I (saves ~34 bags cement, ~230 kg steel).

- Back boundary wall (37'-3") and short right-side walls at the back passage and driveway are not detailed but included (9" x 6'-6" to 7'-0"). Delete if neighbours' walls already exist.

- Boundary wall foundation (not detailed) taken as 3" P.C.C + three brick steps + 9"x6" R.C.C plinth band + 3" R.C.C coping.

- A 5% brickwork allowance is added for front-elevation frames, fins, piers and profile-light reveals that are not shown in plan.

- The car-porch corner (13½" x 13½") is taken as an R.C.C column (4#6, #3@6" ties) as the notes specify column concrete - costlier than a brick pillar.

- An 8'-0" lintel is added over the 6'-6" dining opening (not shown on the lintel key plan).

- A 3" P.C.C 1:4:8 sub-base is included over the first-floor and mumty floors to level the 4" upstand beams and cover conduits.

## 4.3 Not included (exclusions)

Labour; shuttering/formwork and scaffolding (contractor's items); curing water and construction electricity; all finishing works (tiles/marble, paint, woodwork, aluminium, glass, grills, railings, gates); sanitary fixtures; electrical cables, switches, breakers and DBs (only conduits and boxes included); geyser; termite treatment; and the optional items in Section 9.

# 5. Detailed bill of quantities with material breakdown (net, before wastage)

Each line shows the measured quantity and the materials it consumes. Section sub-totals are net; wastage is applied only once, in Sections 1 and 2.

| Item | Description | Quantity | Unit | Cement (bags) | Sand (cft) | Crush (cft) | Bricks (nos) | Steel (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. Foundations & Plinth |  |  |  |  |  |  |  |  |
| A1 | Excavation for wall foundations (PCC footprint x 2'-6" deep below NSL), incl. 10% working space [PCC footprint 969.6 sft (types 1-1..4-4 from S-01/S-02)] | 2,666.3 | cft | - | - | - | - | - |
| A2 | P.C.C 1:4:8, 3" thick under footings (widths 2'-9"/3'-0"/3'-6"/2'-6") | 242.4 | cft | 23.4 | 117 | 234 | - | - |
| A3 | R.C.C 1:2:4 wall footing 6" thick (2'-6"/2'-6"/3'-0"/2'-0" wide) with #3@6" transverse + #3@8" longitudinal | 416.8 | cft | 74.8 | 187 | 374 | - | 673 |
| A4 | Brick masonry in foundation 1:6 - stepped footing (22½", 18", 13½") + 9" wall below plinth beam | 1,075.4 | cft | 49.0 | 368 | - | 14,518 | - |
| A5 | Brick masonry 1:6 - 9" plinth wall from plinth beam (road level) to DPC/FFL (2'-9") | 744.6 | cft | 34.0 | 255 | - | 10,052 | - |
| A6 | R.C.C 1:2:4 plinth beam 9"x9" under all walls, 2#4 top + 2#4 bottom, #2@10" rings [plinth beam centre-line length 370.7 ft] | 203.1 | cft | 36.4 | 91 | 182 | - | 569 |
| A7 | D.P.C 1½" thick 1:2:4 at FFL over plinth walls | 33.8 | cft | 6.1 | 15 | 30 | - | - |
| A8 | Vertical DPC: ¾" plaster 1:3 + 2 coats hot bitumen on inner face of external plinth walls [bitumen approx 304 kg (0.25 kg/sft/coat)] | 607.9 | sft | 12.6 | 47 | - | - | - |
| A9 | R.C.C column 1:1.5:3 at car-porch front corner 13½"x13½" (17.0 ft high), 4#6 + #3@6" ties | 21.5 | cft | 4.9 | 9 | 18 | - | 77 |
| A10 | Column footing 3'x3'x1' R.C.C 1:2:4 (#4@6" both ways) + 3" P.C.C 1:4:8 | 9.0 | cft | 1.9 | 6 | 11 | - | 12 |
| A11 | Earth filling inside plinth (NSL to underside of sand), compacted in 6" layers - incl. 20% compaction [floor area 1363 sft x 3.54 ft; NSL assumed 1'-6" below road (drawings say "as per site")] | 5,791.0 | cft | - | - | - | - | - |
| A12 | Back-filling excavated earth around foundations (re-use) | 965.0 | cft | - | - | - | - | - |
|  | Sub-total A |  |  | 243.2 | 1,095 | 850 | 24,570 | 1,330 |
| B. Ground-floor sub-floor |  |  |  |  |  |  |  |  |
| B1 | Sand filling 4" under floors (rooms + car porches) | 589.7 | cft | - | 590 | - | - | - |
| B2 | P.C.C 1:4:8, 3" thick sub-floor (rooms 1363 + porches 246 sft) | 402.1 | cft | 38.8 | 194 | 388 | - | - |
| B3 | Earth filling under car porches (to porch level +1'-3") | 614.2 | cft | - | - | - | - | - |
| B4 | Upper-floor sub-base: 3" P.C.C 1:4:8 over FF floor & mumty floor (levels 4" upstand CB beams, covers conduits) | 436.3 | cft | 42.2 | 211 | 422 | - | - |
|  | Sub-total B |  |  | 81.0 | 995 | 810 | 0 | 0 |
| C. Brick masonry (superstructure) |  |  |  |  |  |  |  |  |
| C-GF9 | Ground floor 9" walls, 1:5 - gross 2128.5 cft (plan 193.5 sft x 11.00 ft) less openings 416.9, lintels 56.9, bed plates | 1,649.6 | cft | 87.8 | 549 | - | 22,270 | - |
| C-GF45 | Ground floor 4½" walls, 1:4 - gross 427.3 cft (plan 38.8 sft x 11.00 ft) less openings 54.4, lintels 8.1 | 364.9 | cft | 23.3 | 116 | - | 4,926 | - |
| C-FF9 | First floor 9" walls, 1:5 - gross 2017.9 cft (plan 174.8 sft x 11.54 ft) less openings 373.4, lintels 55.2, bed plates | 1,582.6 | cft | 84.2 | 526 | - | 21,365 | - |
| C-FF45 | First floor 4½" walls, 1:4 - gross 737.8 cft (plan 63.9 sft x 11.54 ft) less openings 91.9, lintels 13.1 | 632.8 | cft | 40.4 | 202 | - | 8,542 | - |
| C-SF9 | Second floor (mumty/room) 9" walls, 1:5 - gross 414.7 cft (plan 43.5 sft x 9.54 ft) less openings 51.0, lintels 8.9, bed plates | 353.7 | cft | 18.8 | 118 | - | 4,775 | - |
| C-SF45 | Second floor (mumty/room) 4½" walls, 1:4 - gross 237.2 cft (plan 24.9 sft x 9.54 ft) less openings 48.4, lintels 7.2 | 181.6 | cft | 11.6 | 58 | - | 2,451 | - |
| C-P1 | Terrace parapets on FF roof (4½", one 9" run) 125.6 ft x 3.04 ft high, 1:4 | 154.4 | cft | 9.9 | 49 | - | 2,084 | - |
| C-P2 | Parapet on mumty roof 4½" 94.2 ft x 2.04 ft, 1:4 | 72.1 | cft | 4.6 | 23 | - | 974 | - |
| C-E | Elevation features allowance 5% (front frames, fins, piers, profile-light reveals, planters) 1:4 | 249.6 | cft | 15.9 | 80 | - | 3,369 | - |
|  | Sub-total C |  |  | 296.4 | 1,721 | 0 | 70,757 | 0 |
| D. R.C.C work (1:2:4 unless noted) |  |  |  |  |  |  |  |  |
| D-SGF | Roof slab 5½" - GF roof: outline 1739.1 + porch cantilever 38.7 - stair opening 72.8 (1705.0 sft) | 781.5 | cft | 140.2 | 351 | 701 | - | - |
| D-SFF | Roof slab 5½" - FF roof: outline 1725.0 - stair opening 74.3 (excl. open shaft & terrace) (1650.7 sft) | 756.6 | cft | 135.8 | 339 | 679 | - | - |
| D-SSF | Roof slab 5½" - Mumty roof: 16.85x7.75 + 17.63x12.63 (353.2 sft) | 161.9 | cft | 29.0 | 73 | 145 | - | - |
| D-CB-1-GF | CB-1 concealed beam 12" x 9½" in GF roof - 5 nos, 81.7 ft (4" upstand above slab; 3#6 bottom, 3#4 top, #3@7" rings) | 27.2 | cft | 4.9 | 12 | 24 | - | 374 |
| D-CB-1-FF | CB-1 concealed beam 12" x 9½" in FF roof - 1 nos, 15.5 ft (4" upstand above slab; 3#6 bottom, 3#4 top, #3@7" rings) | 5.2 | cft | 0.9 | 2 | 5 | - | 71 |
| D-CB-2-GF | CB-2 concealed beam 9" x 9½" in GF roof - 5 nos, 48.5 ft (4" upstand above slab; 2#6 top & bottom, #3@6" rings) | 12.1 | cft | 2.2 | 5 | 11 | - | 225 |
| D-CB-2-FF | CB-2 concealed beam 9" x 9½" in FF roof - 7 nos, 102.0 ft (4" upstand above slab; 2#6 top & bottom, #3@6" rings) | 25.5 | cft | 4.6 | 11 | 23 | - | 443 |
| D-CB-2-SF | CB-2 concealed beam 9" x 9½" in SF roof - 2 nos, 30.3 ft (4" upstand above slab; 2#6 top & bottom, #3@6" rings) | 7.6 | cft | 1.4 | 3 | 7 | - | 131 |
| D-LGF | Door/window/vent lintels GF - 23 nos (DL-1, DL-2, WL-1, WL-2, WL-3, VL-1 per S-05) | 64.9 | cft | 11.7 | 29 | 58 | - | 264 |
| D-LFF | Door/window/vent lintels FF - 26 nos (DL-1, DL-2, WL-1, WL-2, WL-3, VL-1 per S-05) | 68.3 | cft | 12.3 | 31 | 61 | - | 287 |
| D-LSF | Door/window/vent lintels SF - 9 nos (DL-1, DL-2, WL-1, WL-2, WL-3, VL-1 per S-05) | 16.1 | cft | 2.9 | 7 | 14 | - | 73 |
| D-BP-GF | Bed plates 9" x 3'-0" x 3" (1:2:4, 3#4) under beam ends, GF roof - 9 nos | 5.1 | cft | 0.9 | 2 | 5 | - | 22 |
| D-BP-FF | Bed plates 9" x 3'-0" x 3" (1:2:4, 3#4) under beam ends, FF roof - 12 nos | 6.8 | cft | 1.2 | 3 | 6 | - | 30 |
| D-BP-SF | Bed plates 9" x 3'-0" x 3" (1:2:4, 3#4) under beam ends, SF roof - 2 nos | 1.1 | cft | 0.2 | 1 | 1 | - | 5 |
| D-ST-GF | Zig-zag stair GF to FF: 6" waist (flights 11.65 + 9.59 ft), 6" landing 8'-3"x3'-6", 21 concrete steps; riser 6.57", tread 10½", width 3'-6" | 69.2 | cft | 12.4 | 31 | 62 | - | 283 |
| D-ST-FF | Zig-zag stair FF to 2nd floor: 6" waist (flights 11.65 + 9.59 ft), 6" landing 8'-3"x3'-6", 21 concrete steps; riser 6.57", tread 10½", width 3'-6" | 69.2 | cft | 12.4 | 31 | 62 | - | 283 |
| D-SF | Stair base footing + stem (8" slab, #4@6" both ways; stem #4@5") | 12.5 | cft | 2.2 | 6 | 11 | - | 55 |
| D-SP | P.C.C 1:4:8 under stair footing | 4.4 | cft | 0.4 | 2 | 4 | - | - |
| D-SH-GF | R.C.C sunshade (chajja) GF: 12.5 ft long x 1'-3" projection, 4"-3" thick, #3@6" main | 5.2 | cft | 0.9 | 2 | 5 | - | 19 |
| D-SH-FF | R.C.C sunshade (chajja) FF: 8.0 ft long x 1'-3" projection, 4"-3" thick, #3@6" main | 3.3 | cft | 0.6 | 1 | 3 | - | 12 |
| D-SH-SF | R.C.C sunshade (chajja) SF: 6.0 ft long x 1'-3" projection, 4"-3" thick, #3@6" main | 2.5 | cft | 0.4 | 1 | 2 | - | 9 |
| D-PR | Provisional: front elevation canopy / RCC fins & projections | 20.0 | cft | 3.6 | 9 | 18 | - | 26 |
| D-CP | Parapet coping 3" x 9" (1:2:4) with 2#3 | 41.2 | cft | 7.4 | 18 | 37 | - | 79 |
|  | Sub-total D |  |  | 388.5 | 972 | 1,945 | 0 | 2,691 |
| E. Slab reinforcement |  |  |  |  |  |  |  |  |
| E-GF | Slab steel GF roof (44 bar sets incl. cranked, extra top, trimmer 2#6/2#4 & chairs) - 0.84 kg/sft | 1,705.0 | sft | - | - | - | - | 1,435 |
| E-FF | Slab steel FF roof (40 bar sets incl. cranked, extra top, trimmer 2#6/2#4 & chairs) - 0.88 kg/sft | 1,650.7 | sft | - | - | - | - | 1,447 |
| E-SF | Slab steel SF roof (11 bar sets incl. cranked, extra top, trimmer 2#6/2#4 & chairs) - 0.73 kg/sft | 353.2 | sft | - | - | - | - | 259 |
|  | Sub-total E |  |  | 0.0 | 0 | 0 | 0 | 3,141 |
| F. Plaster (grey-structure finish) |  |  |  |  |  |  |  |  |
| F-IGF | Internal plaster ½" 1:4 - GF walls (gross 6044 sft, less openings, plus reveals) | 5,233.7 | sft | 58.0 | 290 | - | - | - |
| F-EGF | External plaster ¾" 1:4 - GF outer faces incl. exposed plinth | 1,595.2 | sft | 26.5 | 133 | - | - | - |
| F-IFF | Internal plaster ½" 1:4 - FF walls (gross 7262 sft, less openings, plus reveals) | 6,393.1 | sft | 70.9 | 354 | - | - | - |
| F-EFF | External plaster ¾" 1:4 - FF outer faces | 1,304.0 | sft | 21.7 | 108 | - | - | - |
| F-ISF | Internal plaster ½" 1:4 - SF walls (gross 1472 sft, less openings, plus reveals) | 1,252.1 | sft | 13.9 | 69 | - | - | - |
| F-ESF | External plaster ¾" 1:4 - SF outer faces | 679.8 | sft | 11.3 | 57 | - | - | - |
| F-P | Parapet plaster ¾" 1:4 both faces + coping | 1,313.6 | sft | 21.8 | 109 | - | - | - |
| F-C | Ceiling plaster ⅜" 1:3 (slab soffits, stair soffits, shades) | 3,442.2 | sft | 35.8 | 134 | - | - | - |
|  | Sub-total F |  |  | 259.8 | 1,255 | 0 | 0 | 0 |
| G. Boundary walls, gates pillars & external works |  |  |  |  |  |  |  |  |
| G1 | Excavation for boundary wall & pillar foundations 2'-3" wide x 2'-6" deep (141.9 ft run) | 878.0 | cft | - | - | - | - | - |
| G2 | P.C.C 1:4:8 3" under boundary wall foundation | 79.8 | cft | 7.7 | 39 | 77 | - | - |
| G3 | Brick masonry 1:6 stepped foundation (22½", 18", 13½" x 6" each) | 319.3 | cft | 14.6 | 109 | - | 4,310 | - |
| G4 | R.C.C 1:2:4 plinth band / DPC beam 9"x6" at plinth level, 4#3 + #2@9" rings | 53.2 | cft | 9.5 | 24 | 48 | - | 128 |
| G5 | Brick masonry 1:5 boundary wall 9" (122.0 ft of panels, 6'-0" to 7'-3" above road) | 646.3 | cft | 34.4 | 215 | - | 8,726 | - |
| G6 | Brick pillars 1:4 - 7 nos 13½"x13½" + 3 nos 4'-0"x13½" feature pillars, 6'-6" high | 150.9 | cft | 9.6 | 48 | - | 2,037 | - |
| G7 | R.C.C coping 3" on wall panels + pillar caps (1:2:4), 2#3 | 36.1 | cft | 6.5 | 16 | 32 | - | 44 |
| G8 | Boundary wall plaster ¾" 1:4 (both faces road side, inner face on neighbour side, pillars, top) | 2,055.2 | sft | 34.2 | 171 | - | - | - |
| G9 | Lawn toe walls 9" (approx. 15 ft x 2'-6") 1:5 + 3" PCC base | 28.1 | cft | 2.0 | 12 | 5 | 380 | - |
| G10 | Entrance / porch / rear-door steps (brick 1:5, provisional) [incl. 20 cft PCC 1:4:8 treads base] | 40.0 | cft | 4.1 | 23 | 19 | 540 | - |
|  | Sub-total G |  |  | 122.6 | 657 | 182 | 15,993 | 172 |
| H. Septic tank & manholes |  |  |  |  |  |  |  |  |
| H1 | Septic tank excavation (internal 6'-0" x 4'-0" x 5'-6" deep, 2 chambers) [drawing shows ~4'-10" x 3'-5" overall; enlarged to standard size for ~12 users (upper estimate)] | 372.9 | cft | - | - | - | - | - |
| H2 | P.C.C 1:4:8 3" base | 12.0 | cft | 1.2 | 6 | 12 | - | - |
| H3 | R.C.C 1:2:4 base slab 6" (#3@6" both ways) | 20.6 | cft | 3.7 | 9 | 19 | - | 29 |
| H4 | Brick masonry 1:4 - 9" walls + two 4½" baffle walls | 140.0 | cft | 8.9 | 45 | - | 1,890 | - |
| H5 | R.C.C 1:2:4 cover slab 5" with 2 manhole openings (#3@6" both ways + trimmers) | 13.9 | cft | 2.5 | 6 | 12 | - | 33 |
| H6 | Plaster ¾" 1:3 with water-proofing compound inside septic tank | 206.0 | sft | 4.3 | 16 | - | - | - |
| H7 | Manholes / inspection chambers - 9 nos 2'x2' internal, avg 3'-6" deep: excavation | 774.6 | cft | - | - | - | - | - |
| H8 | Manholes: P.C.C 1:4:8 4" base | 36.7 | cft | 3.5 | 18 | 35 | - | - |
| H9 | Manholes: 9" brick walls 1:4 | 346.5 | cft | 22.1 | 111 | - | 4,678 | - |
| H10 | Manholes: benching/channels 1:2:4 | 13.5 | cft | 2.4 | 6 | 12 | - | - |
| H11 | Manholes: inside plaster ½" 1:3 | 288.0 | sft | 4.0 | 15 | - | - | - |
| H12 | Manholes: R.C.C cover slabs 3" (#3@6" both ways) | 17.0 | cft | 3.1 | 8 | 15 | - | 54 |
|  | Sub-total H |  |  | 55.7 | 239 | 105 | 6,568 | 116 |
| I. Overhead water tank (R.C.C, 1000 gallons) |  |  |  |  |  |  |  |  |
| I1 | R.C.C 1:1.5:3 overhead tank internal 8'x5'x4'-6" (1000 imp. gal): 6" base, 6" walls, 4" top [drawings show tank position only (5'-4"x3'-1" symbol) on mumty roof; engineer to confirm support] | 106.7 | cft | 24.4 | 46 | 91 | - | 228 |
| I2 | Brick support walls/piers under tank 9" 1:4 (provisional) | 45.0 | cft | 2.9 | 14 | - | 608 | - |
| I3 | Tank plaster: inside ¾" 1:3 water-proof (197 sft) + outside ½" 1:4 (204 sft) | 401.0 | sft | 6.4 | 27 | - | - | - |
|  | Sub-total I |  |  | 33.6 | 87 | 91 | 608 | 228 |
|  | GRAND TOTAL (net) |  |  | 1,480.9 | 7,020 | 3,984 | 118,495 | 7,679 |

# 6. Reinforcement steel schedule (kg, net)

| Component | 6 mm (#2) | 10 mm (#3) | 12 mm (#4) | 20 mm (#6) | Total (kg) |
| --- | --- | --- | --- | --- | --- |
| A. Foundations & Plinth | 79 | 697 | 502 | 53 | 1,330 |
| D. R.C.C work (1:2:4 unless noted) | 27 | 713 | 1,083 | 868 | 2,691 |
| E. Slab reinforcement | - | 2,506 | 580 | 55 | 3,141 |
| G. Boundary walls, gates pillars & external works | 26 | 145 | - | - | 172 |
| H. Septic tank & manholes | - | 116 | - | - | 116 |
| I. Overhead water tank (R.C.C, 1000 gallons) | - | 228 | - | - | 228 |
| TOTAL net | 132 | 4,406 | 2,165 | 976 | 7,679 |
| TOTAL with 5% wastage (to procure) | 140 | 4,630 | 2,280 | 1,030 | 8,080 |

Bar lengths are generally available in 40 ft (12 m) lengths. Approximate number of bars to buy (with wastage): 10 mm (#3): 679 bars, 12 mm (#4): 189 bars, 20 mm (#6): 38 bars, 6 mm (#2): 47 bars. Per-component detail: footings 673 kg, plinth beams 569 kg, slabs 3,141 kg, concealed beams 1,244 kg, lintels 624 kg, stairs 622 kg.

# 7. Sewerage, rain-water, water-supply and gas pipes

Horizontal runs were measured on the colour-coded sewerage (sheets 17-20) and sanitary (sheets 21-24) drawings; vertical soil/waste stacks, vent pipes, rain-water pipes, risers and fixture drops were added.

| Service | Size | Run / basis | Net (ft) | +10% (ft) | 20-ft lengths |
| --- | --- | --- | --- | --- | --- |
| Sewerage (UPVC) | 5" | Main sewer: rear manholes -> side passage -> septic tank -> road sewer; car-porch MH -> septic tank | 156 | 172 | 9 |
| Sewerage (UPVC) | 4" | WC connections GF/FF/2F (horizontal 47.8 ft) + soil stacks 3x13 ft (FF), 1x25 ft (2F) + 4 vent pipes x 4 ft | 128 | 141 | 8 |
| Sewerage (UPVC) | 3" | Floor traps, basins, showers, sinks (horizontal 116.1 ft) + waste stacks 4x13 ft + 2x25 ft | 218 | 240 | 12 |
| Rain water (UPVC) | 3" | 4 rain-water down pipes in 9"x6" ducts (38+29+29+16 ft) | 112 | 124 | 7 |
| Water supply (PPR) | 1"-1¼" | Tank down-take to GF (38 ft) + pump rising main to tank (40 ft) + floor mains (40 ft) | 118 | 130 | 7 |
| Water supply (PPR) | ¾" | Cold water branch lines (measured on sanitary drawings) | 370 | 407 | 21 |
| Water supply (PPR) | ½" | Cold fixture drops 35 points x 4 ft | 140 | 154 | 8 |
| Hot water (PPR) | ¾" | Hot water lines (measured) + geyser risers 36 ft | 360 | 397 | 20 |
| Hot water (PPR) | ½" | Hot fixture drops 20 points x 4 ft | 80 | 88 | 5 |
| Gas (GI/PPR-gas) | ½"-¾" | Gas lines to kitchens, geyser, heaters (measured) + riser from meter 30 ft | 190 | 209 | 11 |

## 7.1 Manholes and septic tank

- 9 manholes / inspection chambers (3 in rear passage, 3 in left side passage, 1 front-left lawn corner, 1 patio, 1 car porch): 2'-0" x 2'-0" internal, average 3'-6" deep, 9" brick walls in 1:4 on 4" P.C.C, benching, ½" plaster 1:3 inside, 3" R.C.C cover slab. Buy 9 manhole covers (C.I. or R.C.C, 20"x20").

- Septic tank (front lawn, connected to main sewer): internal 6'-0" x 4'-0" x 5'-6", two chambers with 4½" baffle walls, 6" R.C.C base, 9" brick walls 1:4, 5" R.C.C top with two 2'x2' openings, ¾" water-proof plaster. Buy 2 covers and 2 x 5" UPVC tees (inlet/outlet).

- Rain water: 4 roof sumps (1'-6" x 1'-6") and 3" pipes in 9" x 6" wall ducts with 1½" tile cover as drawn.

## 7.2 Fittings (approximate counts)

| Group | Fittings |
| --- | --- |
| UPVC sewer 5" | Bends 8, Y-tees 6, sockets 9, septic inlet/outlet tees 2 |
| UPVC 4" (soil) | WC connectors 8, bends 20, Y-tees 10, sockets 8, vent cowls 4, clean-out plugs 4 |
| UPVC 3" (waste & rain) | Floor traps (P-traps) 14, gully traps 3, bends 40, Y-tees 20, sockets 15, roof outlets 4, RWP shoes 4 |
| PPR water (cold/hot) | Elbows ~120, tees ~60, sockets ~80, male/female adaptors ~55, unions ~20, ball valves ~20, gate valves 4 |
| Gas | Elbows ~20, tees ~8, gas cocks 8, meter connection set 1 |
| Consumables | UPVC solvent cement 4 kg, PTFE tape, pipe clamps ~120 |

# 8. Electrical conduits and boxes

| Floor | Light / fan / fancy / profile / CCTV points | Socket points | A.C points | Switch boards |
| --- | --- | --- | --- | --- |
| Ground floor | 53 | 41 | 5 | 18 |
| First floor | 47 | 30 | 4 | 16 |
| Mumty & terrace | 15 | 8 | 1 | 5 |
| Total | 115 | 79 | 10 | 39 |

| PVC conduit | Basis | Net (ft) | +10% (ft) | 10-ft pipes |
| --- | --- | --- | --- | --- |
| ¾" | Light/fan points 115 x 12 ft + socket points 79 x 8 ft + 25 low-voltage runs x 30 ft (TV, net, CCTV, intercom, bells) + 10% UPS circuits | 2,900 | 3,191 | 320 |
| 1" | DB to 39 switch-boards x 25 ft + 10 A.C points (385 ft) + earthing 30 ft | 1,390 | 1,530 | 153 |
| 1½" | Meter to main DB + risers to FF & 2F DBs | 85 | 94 | 10 |

- Back boxes: 39 switch-board boxes (assorted 3"x3" to 12"x3"), ~79 socket boxes, 10 A.C boxes, 13 fan boxes with hooks, ~110 round light junction boxes, 3 DB enclosures (main 12-way GF, 8-way FF, 4-way mumty).

- Accessories: ~300 bends ¾", ~100 bends 1", ~350 couplers ¾", ~150 couplers 1", PVC solvent, GI draw wire.

- Points were counted from the electrical plans (sheets 13-15, legend sheet 16); per-point conduit allowances: 12 ft per light/fan point, 8 ft per socket, 25 ft per switch-board feeder, 30-45 ft per A.C circuit, 30 ft per low-voltage run (TV, internet, CCTV, intercom, bells), +10% for UPS circuits.

# 9. Optional items (NOT included in the totals)

| Item | Description | Cement (bags) | Sand (cft) | Crush (cft) | Bricks / tiles | Steel (kg) | Other |
| --- | --- | --- | --- | --- | --- | --- | --- |
| O1 | Roof insulation/treatment on terrace & mumty roof (1651 sft): polythene sheet, 4" earth (mud) compacted, 1½" brick tiles in 1" cement mortar 1:4, pointing 1:3 | 37 | 170 | - | 5,720 | - | Polythene 1816 sft; earth 660 cft |
| O2 | Underground water tank 1,500 gallons (internal 8'x6'x5'): R.C.C 1:1.5:3 walls/base/top ~170 cft, PCC 12 cft, #3/#4 steel ~400 kg, excavation ~520 cft, inside water-proof plaster | 44 | 94 | 157 | - | 400 | Excavation ~520 cft |
| O3 | External paving base: side & rear passages + driveway (~430 sft) 3" P.C.C 1:4:8 over 3" sand | 10 | 159 | 104 | - | - |  |

Roof treatment (item O1) is often counted as part of the grey structure in Faisalabad contracts - add it if your contractor's scope includes it.

# 10. Points to confirm with your architect / engineer

These observations came up while measuring the drawings. None changes the estimate materially, but each should be settled before work starts.

- Natural surface level vs road level is marked "as per site". Have it surveyed; it controls excavation depth and earth filling (about 820 cft per 6").

- Ground-floor M-Bed is 14'-0" x 12'-0" on the structural layout but labelled 14'-3" x 12'-0" on the architectural plan (first floor M-Bed is 14'-3" x 14'-0"). The estimate follows the structural geometry.

- The septic tank on sheet 17 is only about 4'-10" x 3'-5" overall. For 8 WCs a larger tank (estimated 6'-0" x 4'-0" x 5'-6" internal) is advisable.

- The overhead water tank is shown only as a symbol (5'-4" x 3'-1") on the mumty roof; its support (walls/beams under it) is not designed. Confirm R.C.C vs plastic tanks.

- The 6'-6" dining opening has no lintel on the lintel key plan (S-04); an 8'-0" DL-1 has been included.

- The second stair flight (steps 14-22) passes over the rear car porch (headroom about 7'). The first-floor stair-hall rear wall there is 4½" thick and sits on CB-1 beams cast with the ground-floor roof - make sure these beams and the stair are cast together as detailed.

- The car-porch front corner (13½" square, hatched on S-01) has no column or footing detail. The estimate assumes an R.C.C column with 4#6 bars on a 3'x3'x1' footing.

- The ground-floor patio (4'-0" wide) and the first-floor "open 3'-6" wide" shaft are open on the neighbour (property-line) side - confirm whether a privacy/boundary wall is required there (not included).

- The powder-room wall on the property line is only 4½" thick with a concentric 2'-0" footing (section 4-4) that would cross the property line; consider an eccentric footing like section 1-1.

- Boundary wall foundations and the back boundary wall are not detailed; confirm the assumed details (Section 4.2).

- Superstructure mortar ratios are not specified; the estimate assumes 1:5 for 9" and 1:4 for 4½" walls.

- Sunken slabs (+0'-6" depressed) are noted under first-floor baths - ensure the slab steel is detailed through the drops.

# 11. Practical checklist for a first project

- Order in stages (Section 2) and keep a stock register; count bricks and weigh steel on every delivery.

- Bricks: first class, uniform size and colour, clear ringing sound, no cracks; soak before laying.

- Cement: check the manufacturing date (use within about a month), store on a raised dry platform, not more than 10 bags high.

- Sand: do a silt (bottle) test - reject if silt is more than about 8%. Chenab sand for concrete, Ravi sand for mortar and plaster.

- Steel: Grade 60 rolling marks on every bar; check weight per foot against the standard (10 mm = 0.376 lb/ft, 12 mm = 0.668 lb/ft, 20 mm = 1.502 lb/ft).

- Measure concrete and mortar with gauge boxes (farma), never by shovels; use a vibrator for slabs and beams.

- Before every slab pour, have the engineer check bar sizes, spacing, cranked bars, extra top bars, covers (¾" slab, 1½" beams) and chairs.

- Place electrical conduits, fan boxes and sewer/water sleeves before casting slabs and before plastering.

- Cure slabs and beams for at least 10-14 days, brickwork and plaster for 7 days.

# Appendix A - Measured foundation / ground-floor wall schedule

Coordinates in feet from the rear-left corner of the building (x to the right, y towards the road), measured from structural drawing S-01. Section = foundation type from S-01/S-02.

| Wall | Description | Thickness | Section | Length (ft) | Centre-line coordinates (ft) |
| --- | --- | --- | --- | --- | --- |
| R1 | Back wall (Bed-1 & S-T lobby) facing rear passage | 9" | 2-2 | 19.50 | (17.38, 0.38) to (36.88, 0.38) |
| R2 | Back wall step (S-T lobby, rear) | 9" | 2-2 | 3.31 | (17.38, 0.38) to (17.38, 3.69) |
| R3 | S-T lobby / Bed-1 partition | 9" | 3-3 | 13.56 | (23.13, 0.38) to (23.13, 13.94) |
| R4 | S-T lobby / lobby partition | 9" | 3-3 | 5.75 | (17.38, 9.25) to (23.13, 9.25) |
| R5 | Bath-Dress & M-Bed / lobby partition | 9" | 3-3 | 17.94 | (17.38, 7.56) to (17.38, 25.5) |
| R6 | Left outer wall of stair & bath | 9" | 2-2 | 9.06 | (3.9, 3.69) to (3.9, 12.75) |
| R6b | Left outer wall jog | 9" | 2-2 | 1.27 | (2.63, 12.75) to (3.9, 12.75) |
| R7 | M-Bed left outer wall | 9" | 2-2 | 12.75 | (2.63, 12.75) to (2.63, 25.5) |
| R8 | M-Bed / kitchens partition | 9" | 3-3 | 17.25 | (1.0, 25.5) to (18.25, 25.5) |
| R9 | Raw kitchen left outer wall | 9" | 2-2 | 11.75 | (1.38, 25.5) to (1.38, 37.25) |
| R10 | Raw kitchen / TV lounge partition | 9" | 3-3 | 8.75 | (0.38, 37.25) to (9.12, 37.25) |
| R11 | TV lounge left outer wall | 9" | 2-2 | 13.84 | (0.38, 37.25) to (0.38, 51.09) |
| R12 | TV lounge front splayed wall | 9" | 2-2 | 3.25 | (0.38, 51.09) to (3.44, 50.0) |
| R13 | TV lounge front wall | 9" | 2-2 | 12.69 | (3.44, 50.0) to (16.13, 50.0) |
| R14 | TV lounge / entrance wall | 9" | 2-2 | 8.50 | (16.13, 41.5) to (16.13, 50.0) |
| R15 | Entrance front wall | 9" | 2-2 | 7.00 | (16.13, 45.25) to (23.13, 45.25) |
| R16a | Lobby / dining-drawing partition | 9" | 3-3 | 22.94 | (23.13, 19.31) to (23.13, 42.25) |
| R16b | Lobby / car-porch wall | 9" | 2-2 | 3.75 | (23.13, 42.25) to (23.13, 46.0) |
| R17 | Drawing room front wall (car porch) | 9" | 2-2 | 13.75 | (23.13, 42.25) to (36.88, 42.25) |
| R18a | Right property-line wall (rear part) | 9" | 1-1 | 19.13 | (36.88, 0.38) to (36.88, 19.5) |
| R18b | Right property-line wall (front part) | 9" | 1-1 | 21.19 | (36.88, 29.31) to (36.88, 50.5) |
| R19 | Patio top wall | 9" | 2-2 | 4.00 | (32.88, 19.5) to (36.88, 19.5) |
| R20 | Patio left wall | 9" | 2-2 | 4.75 | (32.88, 19.5) to (32.88, 24.25) |
| R21 | Patio bottom / P-W top wall | 9" | 2-2 | 4.19 | (32.88, 24.25) to (37.07, 24.25) |
| G1 | Stair hall rear wall (to back car porch) | 4½" | 4-4 | 13.48 | (3.9, 3.69) to (17.38, 3.69) |
| G2 | Stair / bath-dress partition | 4½" | 4-4 | 13.48 | (3.9, 7.56) to (17.38, 7.56) |
| G3 | Bath-dress / M-Bed partition | 4½" | 4-4 | 13.48 | (3.9, 12.94) to (17.38, 12.94) |
| G4 | Bath / dress partition | 4½" | 4-4 | 5.38 | (12.32, 7.56) to (12.32, 12.94) |
| G5 | Bed-1 / dress-bath partition | 4½" | 4-4 | 13.75 | (23.13, 13.94) to (36.88, 13.94) |
| G6 | Lobby / dress partition | 4½" | 4-4 | 5.38 | (22.94, 13.94) to (22.94, 19.31) |
| G7 | Dress-bath / dining partition | 4½" | 4-4 | 9.94 | (22.94, 19.31) to (32.88, 19.31) |
| G8 | Dress / bath partition | 4½" | 4-4 | 5.38 | (27.32, 13.94) to (27.32, 19.31) |
| G9 | Raw kitchen / open kitchen partition | 4½" | 4-4 | 11.75 | (8.94, 25.5) to (8.94, 37.25) |
| G10 | P-W left wall | 4½" | 4-4 | 5.06 | (32.69, 24.25) to (32.69, 29.31) |
| G11 | P-W front wall | 4½" | 4-4 | 4.19 | (32.69, 29.31) to (36.88, 29.31) |
| G12 | P-W right (property-line) wall | 4½" | 4-4 | 5.06 | (37.07, 24.25) to (37.07, 29.31) |
| G13 | Front elevation projection | 4½" | 4-4 | 1.12 | (3.5, 50.56) to (4.62, 50.56) |
| G14 | Front elevation projection | 4½" | 4-4 | 3.37 | (13.13, 50.56) to (16.5, 50.56) |
|  | Total centre-line length |  |  | 370.71 |  |

| Foundation layer | Plan area (sft, union of all walls) | Thickness | Volume (cft) |
| --- | --- | --- | --- |
| P.C.C 1:4:8 | 969.6 | 3" | 242.4 |
| R.C.C footing | 833.6 | 6" | 416.8 |
| Brick step 22½" | 478.0 | 6" | 239.0 |
| Brick step 18" | 523.2 | 6" | 261.6 |
| Brick step 13½" | 398.9 | 12" | 398.9 |
| 9" plinth strip (plinth beam, DPC, plinth wall) | 270.8 | 9" / 1½" / 2'-9" | 203.1 / 33.8 / 744.6 |

# Appendix B - Slab reinforcement bar sets (from S-06, S-07, S-08)

B = bottom bars (alternately straight and cranked, +0.6 ft per bar for hooks/cranks); T = extra top bars (+0.3 ft). No. of bars = distribution width / spacing + 1. Trimmer bars (2#6, 2#4) and chairs are added to each floor total.

## Ground-floor roof (S-06) - slab area 1,705.0 sft

| Set | Bar | Spacing | Distribution width (ft) | Bar length (ft) | Type | No. of bars | Total length (ft) | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | 10 mm | 8" | 14.59 | 13.40 | Bottom | 23 | 322.0 | 54.9 |
| B2 | 10 mm | 9" | 13.33 | 14.79 | Bottom | 19 | 292.4 | 49.9 |
| B3 | 10 mm | 7" | 14.84 | 8.25 | Bottom | 27 | 238.9 | 40.8 |
| B4 | 10 mm | 10" | 8.58 | 14.67 | Bottom | 12 | 183.2 | 31.3 |
| B5 | 10 mm | 8" | 13.59 | 16.14 | Bottom | 22 | 368.3 | 62.8 |
| B6 | 10 mm | 6" | 16.33 | 13.54 | Bottom | 34 | 480.8 | 82.0 |
| B7 | 10 mm | 10" | 4.58 | 46.13 | Bottom | 7 | 327.1 | 55.8 |
| B8 | 10 mm | 8" | 43.71 | 5.75 | Bottom | 67 | 425.4 | 72.6 |
| B9 | 10 mm | 6" | 46.83 | 13.75 | Bottom | 95 | 1,363.2 | 232.5 |
| B10 | 10 mm | 10" | 12.58 | 13.57 | Bottom | 17 | 240.9 | 41.1 |
| B11 | 10 mm | 10" | 12.58 | 5.56 | Bottom | 17 | 104.7 | 17.9 |
| B12 | 10 mm | 8" | 12.58 | 14.14 | Bottom | 20 | 294.8 | 50.3 |
| B13 | 10 mm | 10" | 12.58 | 8.61 | Bottom | 17 | 156.6 | 26.7 |
| B14 | 12 mm | 10" | 12.96 | 9.29 | Bottom | 17 | 168.1 | 50.9 |
| T1 | 10 mm | 8" | 14.59 | 4.16 | Extra top | 23 | 102.6 | 17.5 |
| T2 | 12 mm | 10" | 15.32 | 6.50 | Extra top | 20 | 136.0 | 41.2 |
| T3 | 12 mm | 10" | 13.44 | 5.17 | Extra top | 18 | 98.5 | 29.8 |
| T4 | 10 mm | 8" | 8.67 | 6.83 | Extra top | 15 | 107.0 | 18.2 |
| T5 | 10 mm | 8" | 4.25 | 4.50 | Extra top | 8 | 38.4 | 6.5 |
| T6 | 10 mm | 10" | 8.58 | 4.50 | Extra top | 12 | 57.6 | 9.8 |
| T7 | 10 mm | 8" | 8.67 | 13.17 | Extra top | 15 | 202.1 | 34.5 |
| T8 | 12 mm | 10" | 13.59 | 7.25 | Extra top | 18 | 135.9 | 41.2 |
| T9 | 12 mm | 10" | 3.96 | 4.58 | Extra top | 6 | 29.3 | 8.9 |
| T10 | 10 mm | 10" | 11.58 | 4.16 | Extra top | 15 | 66.9 | 11.4 |
| T11 | 10 mm | 8" | 16.42 | 12.84 | Extra top | 26 | 341.6 | 58.3 |
| T12 | 10 mm | 10" | 12.32 | 4.91 | Extra top | 16 | 83.4 | 14.2 |
| T13 | 10 mm | 8" | 6.96 | 6.25 | Extra top | 12 | 78.6 | 13.4 |
| T14 | 10 mm | 10" | 5.96 | 3.33 | Extra top | 9 | 32.7 | 5.6 |
| T15 | 10 mm | 10" | 17.96 | 3.92 | Extra top | 23 | 97.1 | 16.6 |
| T16 | 10 mm | 10" | 3.15 | 3.33 | Extra top | 5 | 18.1 | 3.1 |
| T17 | 10 mm | 8" | 11.71 | 6.25 | Extra top | 19 | 124.5 | 21.2 |
| T18 | 10 mm | 10" | 22.96 | 3.92 | Extra top | 29 | 122.4 | 20.9 |
| T19 | 10 mm | 10" | 12.58 | 5.26 | Extra top | 17 | 94.5 | 16.1 |
| T20 | 10 mm | 10" | 12.58 | 8.17 | Extra top | 17 | 144.0 | 24.6 |
| T21 | 10 mm | 10" | 12.58 | 7.17 | Extra top | 17 | 127.0 | 21.7 |
| T22 | 12 mm | 10" | 12.96 | 7.54 | Extra top | 17 | 133.3 | 40.4 |
|  |  |  |  |  | Bar sets total |  |  | 1,344.3 |
|  |  |  |  |  | Floor total incl. trimmers & chairs |  |  | 1,435.4 |

## First-floor roof (S-07) - slab area 1,650.7 sft

| Set | Bar | Spacing | Distribution width (ft) | Bar length (ft) | Type | No. of bars | Total length (ft) | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | 10 mm | 8" | 14.96 | 14.31 | Bottom | 24 | 357.8 | 61.0 |
| B2 | 10 mm | 9" | 13.58 | 14.78 | Bottom | 20 | 307.6 | 52.5 |
| B3 | 10 mm | 7" | 14.84 | 8.25 | Bottom | 27 | 238.9 | 40.8 |
| B4 | 10 mm | 10" | 8.58 | 14.67 | Bottom | 12 | 183.2 | 31.3 |
| B5 | 10 mm | 8" | 13.59 | 16.15 | Bottom | 22 | 368.5 | 62.8 |
| B6 | 10 mm | 6" | 16.33 | 13.54 | Bottom | 34 | 480.8 | 82.0 |
| B7 | 10 mm | 7" | 18.33 | 11.72 | Bottom | 33 | 406.6 | 69.3 |
| B8 | 10 mm | 8" | 18.33 | 7.10 | Bottom | 29 | 223.3 | 38.1 |
| B9 | 10 mm | 10" | 11.58 | 17.88 | Bottom | 15 | 277.2 | 47.3 |
| B10 | 10 mm | 10" | 5.21 | 19.50 | Bottom | 8 | 160.8 | 27.4 |
| B11 | 10 mm | 8" | 14.33 | 12.00 | Bottom | 23 | 289.8 | 49.4 |
| B12 | 10 mm | 8" | 13.21 | 14.25 | Bottom | 21 | 311.8 | 53.2 |
| B13 | 10 mm | 6" | 19.58 | 14.83 | Bottom | 41 | 632.6 | 107.9 |
| B14 | 10 mm | 8" | 13.20 | 19.00 | Bottom | 21 | 411.6 | 70.2 |
| T1 | 10 mm | 8" | 14.96 | 5.70 | Extra top | 24 | 144.0 | 24.6 |
| T2 | 10 mm | 10" | 13.58 | 4.50 | Extra top | 18 | 86.4 | 14.7 |
| T3 | 12 mm | 10" | 13.58 | 9.17 | Extra top | 18 | 170.5 | 51.6 |
| T4 | 12 mm | 10" | 14.96 | 6.50 | Extra top | 19 | 129.2 | 39.1 |
| T5 | 12 mm | 10" | 8.58 | 5.17 | Extra top | 12 | 65.6 | 19.9 |
| T6 | 12 mm | 10" | 10.33 | 7.83 | Extra top | 14 | 113.8 | 34.5 |
| T7 | 12 mm | 10" | 13.59 | 7.25 | Extra top | 18 | 135.9 | 41.2 |
| T8 | 12 mm | 10" | 4.58 | 4.70 | Extra top | 7 | 35.0 | 10.6 |
| T9 | 10 mm | 10" | 11.33 | 4.16 | Extra top | 15 | 66.9 | 11.4 |
| T10 | 10 mm | 10" | 12.32 | 4.92 | Extra top | 16 | 83.5 | 14.2 |
| T11 | 12 mm | 10" | 4.58 | 7.50 | Extra top | 7 | 54.6 | 16.5 |
| T12 | 12 mm | 10" | 12.08 | 8.84 | Extra top | 16 | 146.2 | 44.3 |
| T13 | 10 mm | 10" | 18.33 | 3.67 | Extra top | 23 | 91.3 | 15.6 |
| T14 | 10 mm | 8" | 18.33 | 6.00 | Extra top | 29 | 182.7 | 31.2 |
| T15 | 10 mm | 8" | 18.33 | 6.41 | Extra top | 29 | 194.6 | 33.2 |
| T16 | 10 mm | 10" | 17.96 | 5.67 | Extra top | 23 | 137.3 | 23.4 |
| T17 | 10 mm | 10" | 6.21 | 9.55 | Extra top | 9 | 88.7 | 15.1 |
| T18 | 10 mm | 10" | 13.20 | 4.33 | Extra top | 17 | 78.7 | 13.4 |
| T19 | 12 mm | 10" | 18.33 | 7.16 | Extra top | 23 | 171.6 | 52.0 |
| T20 | 10 mm | 8" | 13.44 | 7.38 | Extra top | 22 | 169.0 | 28.8 |
| T21 | 10 mm | 8" | 5.73 | 4.16 | Extra top | 10 | 44.6 | 7.6 |
| T22 | 12 mm | 8" | 14.90 | 5.00 | Extra top | 24 | 127.2 | 38.5 |
| T23 | 10 mm | 10" | 13.20 | 5.67 | Extra top | 17 | 101.5 | 17.3 |
|  |  |  |  |  | Bar sets total |  |  | 1,392.1 |
|  |  |  |  |  | Floor total incl. trimmers & chairs |  |  | 1,446.9 |

## Mumty roof (S-08) - slab area 353.2 sft

| Set | Bar | Spacing | Distribution width (ft) | Bar length (ft) | Type | No. of bars | Total length (ft) | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1a | 10 mm | 8" | 16.28 | 7.75 | Bottom | 26 | 217.1 | 37.0 |
| B1b | 10 mm | 8" | 16.67 | 11.87 | Bottom | 27 | 336.7 | 57.4 |
| B2a | 10 mm | 10" | 10.71 | 17.25 | Bottom | 14 | 249.9 | 42.6 |
| B2b | 10 mm | 10" | 6.58 | 16.70 | Bottom | 9 | 155.7 | 26.6 |
| T1 | 10 mm | 10" | 16.06 | 3.33 | Extra top | 21 | 76.2 | 13.0 |
| T2 | 10 mm | 10" | 16.06 | 3.34 | Extra top | 21 | 76.4 | 13.0 |
| T3 | 10 mm | 10" | 15.71 | 3.50 | Extra top | 20 | 76.0 | 13.0 |
| T4 | 10 mm | 10" | 15.71 | 3.50 | Extra top | 20 | 76.0 | 13.0 |
| T5 | 10 mm | 10" | 10.71 | 4.67 | Extra top | 14 | 69.6 | 11.9 |
| T6 | 10 mm | 10" | 10.71 | 6.50 | Extra top | 14 | 95.2 | 16.2 |
| T7 | 10 mm | 10" | 6.58 | 4.75 | Extra top | 9 | 45.4 | 7.8 |
|  |  |  |  |  | Bar sets total |  |  | 251.4 |
|  |  |  |  |  | Floor total incl. trimmers & chairs |  |  | 259.0 |

# Appendix C - Openings and lintels

## Ground floor - openings deducted from brickwork

| Opening | Width | Height | Wall | Nos | Wall type |
| --- | --- | --- | --- | --- | --- |
| S-T lobby rear door | 3'-6" | 8'-0" | 9" | 1 | External |
| S-T lobby to lobby door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| Bed-1 door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| M-Bed door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| Drawing room door (to porch) | 3'-6" | 8'-0" | 9" | 1 | External |
| Dining arch opening | 6'-6" | 8'-0" | 9" | 1 | Internal |
| Raw-kitchen side door | 3'-0" | 8'-0" | 9" | 1 | External |
| Main entrance glazed door (5'-1½" x 20'-6", GF part) | 5'-1.5" | 11'-0" | 9" | 1 | External |
| Bed-1 dress door | 2'-10.5" | 8'-0" | 4½" | 1 | Internal |
| Bed-1 bath door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| M-Bed dress door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| M-Bed bath door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| Under-stair store door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| Raw to open kitchen door | 3'-0" | 8'-0" | 4½" | 1 | Internal |
| Powder room door | 2'-3" | 8'-0" | 4½" | 1 | Internal |
| Bed-1 window | 7'-0" | 7'-0" | 9" | 1 | External |
| M-Bed window | 7'-0" | 7'-0" | 9" | 1 | External |
| Drawing window | 6'-0" | 7'-0" | 9" | 1 | External |
| Dining window (patio) | 3'-3" | 7'-0" | 9" | 1 | External |
| Raw-kitchen window | 3'-0" | 4'-6" | 9" | 1 | External |
| TV-lounge front glazing 8'-6" x 20' (GF part) | 8'-6" | 10'-6" | 9" | 1 | External |
| Ventilators 2'-6" x 2' | 2'-6" | 2'-0" | 9" | 3 | External |
| Exhaust fan openings 1'x1' | 1'-0" | 1'-0" | 9" | 3 | External |

## First floor - openings deducted from brickwork

| Opening | Width | Height | Wall | Nos | Wall type |
| --- | --- | --- | --- | --- | --- |
| S-T lobby door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| Bed-1 door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| M-Bed door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| Front bed door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| Store door | 3'-0" | 8'-0" | 9" | 1 | Internal |
| Kitchen door | 3'-0" | 8'-0" | 9" | 1 | Internal |
| Bed-1 dress door | 2'-10.5" | 8'-0" | 4½" | 1 | Internal |
| Bed-1 bath door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| M-Bed dress door | 3'-6" | 8'-0" | 4½" | 1 | Internal |
| M-Bed bath door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| Front bed dress door | 3'-0" | 8'-0" | 4½" | 1 | Internal |
| Front bed bath door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| Kitchen door/window to terrace 6x8 (sill 6") | 6'-0" | 7'-6" | 4½" | 1 | External |
| Bed-1 window | 7'-0" | 7'-0" | 9" | 1 | External |
| M-Bed window | 7'-0" | 7'-0" | 9" | 1 | External |
| Front bed side window | 7'-0" | 7'-0" | 9" | 1 | External |
| Front bed glazing (upper part of 20' glazing) | 8'-6" | 8'-6" | 9" | 1 | External |
| TV-lounge window (to open shaft) | 8'-0" | 7'-0" | 9" | 1 | External |
| Entrance glazing (upper part) | 5'-1.5" | 8'-6" | 9" | 1 | External |
| Stair-hall rear windows 3x7 | 3'-0" | 7'-0" | 4½" | 2 | External |
| Stair-hall vertical glazing slot 2' wide | 2'-0" | 11'-6.5" | 4½" | 1 | External |
| Ventilators 2'-6" x 2' | 2'-6" | 2'-0" | 9" | 3 | External |
| Exhaust fan openings 1'x1' | 1'-0" | 1'-0" | 9" | 4 | External |

## Mumty (2nd floor) - openings deducted from brickwork

| Opening | Width | Height | Wall | Nos | Wall type |
| --- | --- | --- | --- | --- | --- |
| Stair-hall door to terrace | 3'-6" | 8'-0" | 4½" | 1 | External |
| Room door | 3'-6" | 8'-0" | 9" | 1 | Internal |
| Bath door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| Laundry door | 2'-6" | 8'-0" | 4½" | 1 | Internal |
| Room window 5x7 | 5'-0" | 7'-0" | 9" | 1 | External |
| Stair-hall rear windows 3x7 | 3'-0" | 7'-0" | 4½" | 2 | External |
| Stair-hall vertical glazing slot | 2'-0" | 9'-6.5" | 4½" | 1 | External |
| Bath ventilator | 2'-6" | 2'-0" | 9" | 1 | External |

## Lintel schedule (length = opening + bearing)

| Floor | Type | Length | Nos | Section & steel (S-05) |
| --- | --- | --- | --- | --- |
| GF | DL-1 | 5'-0" | 5 | 9"x9", 4#4, #3@6" |
| GF | DL-1 | 6'-7.6" | 1 | 9"x9", 4#4, #3@6" |
| GF | DL-1 | 8'-0" | 1 | 9"x9", 4#4, #3@6" |
| GF | DL-2 | 4'-4.5" | 1 | 4½"x9", 4#4, #2@6" |
| GF | DL-2 | 4'-0" | 4 | 4½"x9", 4#4, #2@6" |
| GF | DL-2 | 4'-6" | 1 | 4½"x9", 4#4, #2@6" |
| GF | DL-2 | 3'-9" | 1 | 4½"x9", 4#4, #2@6" |
| GF | WL&DL-1 | 9'-4.4" | 1 | 9"x9", 4#4, #3@6" |
| GF | WL-1 | 8'-8.3" | 1 | 9"x9", 4#4, #3@6" |
| GF | WL-1 | 8'-10.1" | 1 | 9"x9", 4#4, #3@6" |
| GF | WL-1 | 7'-6" | 1 | 9"x9", 4#4, #3@6" |
| GF | WL-1 | 4'-9" | 1 | 9"x9", 4#4, #3@6" |
| GF | WL-2 | 10'-4.1" | 1 | 9"x9", 2#4+4#4, #3@6" |
| GF | VL-1 | 4'-0" | 3 | 9"x9", 4#3, #3@10" |
| FF | DL-1 | 5'-0" | 4 | 9"x9", 4#4, #3@6" |
| FF | DL-1 | 4'-6" | 2 | 9"x9", 4#4, #3@6" |
| FF | DL-1 | 5'-0" | 1 | 9"x9", 4#4, #3@6" |
| FF | DL-2 | 4'-4.5" | 1 | 4½"x9", 4#4, #2@6" |
| FF | DL-2 | 4'-0" | 3 | 4½"x9", 4#4, #2@6" |
| FF | DL-2 | 5'-0" | 1 | 4½"x9", 4#4, #2@6" |
| FF | DL-2 | 4'-6" | 1 | 4½"x9", 4#4, #2@6" |
| FF | WL-1 | 8'-10" | 3 | 9"x9", 4#4, #3@6" |
| FF | WL-1 | 6'-7.6" | 1 | 9"x9", 4#4, #3@6" |
| FF | WL-1 | 8'-8.3" | 1 | 9"x9", 4#4, #3@6" |
| FF | WL-2 | 10'-4.1" | 1 | 9"x9", 2#4+4#4, #3@6" |
| FF | WL-3 | 3'-7.2" | 1 | 4½"x9", 4#4, #2@6" |
| FF | WL-3 | 4'-6" | 2 | 4½"x9", 4#4, #2@6" |
| FF | WL-3 | 8'-1.8" | 1 | 4½"x9", 4#4, #2@6" |
| FF | VL-1 | 4'-0" | 3 | 9"x9", 4#3, #3@10" |
| SF | DL-1 | 5'-0" | 1 | 9"x9", 4#4, #3@6" |
| SF | DL-2 | 5'-0" | 1 | 4½"x9", 4#4, #2@6" |
| SF | DL-2 | 4'-0" | 2 | 4½"x9", 4#4, #2@6" |
| SF | WL-1 | 6'-9.6" | 1 | 9"x9", 4#4, #3@6" |
| SF | WL-3 | 3'-7.2" | 1 | 4½"x9", 4#4, #2@6" |
| SF | WL-3 | 4'-6" | 2 | 4½"x9", 4#4, #2@6" |
| SF | VL-1 | 4'-0" | 1 | 9"x9", 4#3, #3@10" |