House No. 79/K, WAPDA City, Faisalabad

Grey Structure MATERIAL Estimate, Revision 1  |  Final Decisions Register applied  |  3 October 2026

# 1. One-page summary

| Total material cost Rs 9,189,795 purchase quantities incl. wastage | Covered area 4,104 sft GF 1,898 + FF 1,844 + mumty 362 | Cost per sft Rs 2,239 material only, no labour | Steel (sariya) 7,390 kg Grade 60, incl. 4% wastage |
| --- | --- | --- | --- |

Material only (no labour) at Faisalabad prices of October 2026; dimensions from the architect's drawings, RCC sizes and steel from the structural drawings, every line revised to the Final Decisions Register. Rev 0 was not priced, so it is compared by quantity. Recommended extras (Section 9) are NOT in the total.

| Material (purchase quantity incl. wastage) | Buy | Unit | Trolley loads* | Cost (Rs) | Rev 0 qty |
| --- | --- | --- | --- | --- | --- |
| Cement OPC 50 kg (Lucky / Bestway / Maple Leaf) | 1,544 | bags |  | 2,408,640 | 1,560 |
| Bricks, awwal (first class) 9" x 4½" x 3" | 133,000 | nos | 67 | 2,128,000 | 125,000 |
| Steel Grade 60 (sariya): #2 170, #3 3,940, #4 2,260, #6 1,020 kg | 7,390 | kg |  | 1,937,540 | 8,080 |
| Binding wire (10 kg per ton of steel) | 75 | kg |  | 30,000 | 85 |
| Crush ¾" Sargodha (bajri) | 4,000 | cft | 40 | 440,000 | 4,200 |
| Chenab sand (concrete, screed) | 2,550 | cft | 26 | 165,750 | 2,250 |
| Ravi sand (mortar, plaster, blinding) | 5,400 | cft | 54 | 216,000 | 4,900 + 600 fill sand |
| Khangar (stone ballast) 1½"-2½" | 700 | cft | 7 | 63,000 | none |
| Earth (mitti) to buy for filling | 4,200 | cft | 42 | 105,000 | 3,250 |
| Waterproofing: bitumen 500 kg, membrane 230 m2, polythene, compound, coatings, primer, LPG, water-stop |  |  |  | 498,720 | bitumen 320 kg |
| Bhatta roof tiles 9" x 4½" x 1½" | 5,300 | nos |  | 106,000 | none |
| Anti-termite (12 L) and chicken mesh (18 rolls) with nails |  |  |  | 147,750 | none |
| Sewer, water and gas pipes, fittings and covers |  |  |  | 627,075 | pipes only |
| Electrical conduits, boxes, DBs, earth pits, solar-plinth bolts |  |  |  | 316,320 | conduits only |
| TOTAL MATERIAL COST |  |  |  | 9,189,795 |  |

* Tractor-trolley load = 100 cft (sand, crush, khangar, earth) or 2,000 bricks. Purchase = quantity with wastage rounded up to buying units (Section 5).

## Ten biggest changes from Rev 0

- Foundation now exactly as sheet 05 (C1-C3): the S-02 RCC strip footing (417 cft concrete, 673 kg steel) is deleted; PCC 1:4:8 at -4'-0", brick steps and 13½" wall to road level; foundation brickwork 1,075 → 1,717 cft.

- Plinth beam 9" x 9" moved to floor level with a proper DPC on top (B4, B5): 1½" 1:2:4 + waterproofing compound + 2 coats hot bitumen; top of DPC = FFL +2'-9", 18" above the highest outside level.

- Raised levels and full filling (A3-A5): house 5,655 cft, both car porches 1,060 cft, passages and ramps filled; earth to buy 3,250 → 4,185 cft.

- New moisture-proof sub-floor (B2, B3): 4" khangar + 1" sand + 0.2 mm polythene + 3" PCC 1:4:8, no roṛi: 665 cft khangar, 2,185 sft polythene.

- Roof waterproofing now in the total (F3): slope screed, primer, 4 mm torch-on membrane, gola and bhatta tiles on 1,659 sft - Rs 561,331.

- Water storage (I1-I4): RCC overhead tank deleted; watertight ~515-gallon underground tank added at the back; RCC platform for a 500-gallon plastic roof tank.

- Plaster (F1, F2, D3): ceiling plaster deleted (-3,442 sft); soffits a separate line; boundary walls plastered both faces; chicken mesh on all junctions and chases (18 rolls).

- Anti-termite treatment added (B1): 12 L of bifenthrin 10% EC concentrate for trenches, plinth fill and the outside perimeter.

- Electrical civil works expanded (J1-J8): own 1" conduit to each of 10 A.C points, data and CCTV conduits, backup DBs, EV conduit, 3 earth pits, 34 solar plinths.

- Standards (D1, K2, E4): 9" walls in 1:6, lower wastage (cement 3%, sand 8%, steel 4%, pipes 7%): steel 8,080 → 7,390 kg and cement 1,560 → 1,544 bags despite the added work.

# 2. Basis of the estimate

## 2.1 Documents, order of authority and scope

| Project | House No. 79/K, WAPDA City, Faisalabad, for Mr. Saleem. Ground floor, first floor and mumty (2nd-floor stair tower). Covered area 4,104 sft. |
| --- | --- |
| This revision | Rev 1, 3 October 2026. Replaces Rev 0 (30 September 2026) completely. |
| Architect's working drawings | Faisal Associates, sheets 01-24 (16-08-2026), with 3D views and five bath-detail pages. They govern ALL dimensions and levels. |
| Structural drawings | Fakhar Associates (M. Fakhar Iqbal Sheikh, PEC Civil 11697), 2026/L-20/S-01 to S-09 (14-08-2026). They govern ONLY the size and reinforcement of superstructure RCC members. |
| Owner's decisions | Final Decisions Register A1-K3 (50 lines). Order of authority: (1) Register, (2) questionnaire answers, (3) earlier assumptions. The filled questionnaire itself was not attached, so the Register was used as the record of the answers. |
| Scope (K1) | Grey-structure MATERIALS only. No labour, shuttering, scaffolding or finishes (Section 8). |
| Prices | Faisalabad market, October 2026. Each rate has its date and source in 2.9; rates with no published source are marked "estimated". Rates and wastage can be changed in the Excel file (Rates sheet). |
| Design approach | Sound, not over-built. Sheet 05 is followed with only the DPC added. Nothing is added "just in case": extra items are listed in Section 9 with cost and reason and are not in the total. No compromise on damp-proofing, termite protection, watertight tanks or superstructure RCC. |

## 2.2 Materials and conversion factors (K3, as Rev 0)

| Item | Specification / factor used |
| --- | --- |
| Cement | Ordinary Portland Cement (OPC) 50 kg bag - Lucky, Bestway, Maple Leaf or DG Khan. Soil is not saline and there is no ground water (A2), so sulphate-resisting cement is not needed. |
| Sand | Chenab sand for concrete and roof screed; Ravi sand for mortar, plaster, blinding and under paving. |
| Crush (bajri) | ¾" down, Sargodha. |
| Stone ballast (khangar) | 1½"-2½" crushed stone, hand packed (no brick ballast / roṛi). |
| Steel (sariya) | Grade 60 deformed bars - Amreli, Mughal or FF Steel. Weights: #2 (6 mm) 0.167, #3 (10 mm) 0.376, #4 (12 mm) 0.668, #6 (20 mm) 1.502 lb/ft. |
| Bricks | Awwal (first class) burnt clay 9" x 4½" x 3"; 13.5 bricks per cft of brickwork including joints. |
| Pipes | UPVC sewer pipes Dadex or Beta; PPR PN-16 cold / PN-20 hot - Popular, Dadex or Master; SNGPL-approved medium-class GI for gas. |
| Concrete | Dry volume = wet volume x 1.57; 1 bag = 1.25 cft; mixes 1:1.5:3 (column, water tank), 1:2:4 (RCC), 1:4:8 (PCC). |
| Mortar and plaster | Dry volume = wet x 1.33; 0.30 cft wet mortar per cft of brickwork; plaster +25% for uneven brick surface. |
| Steel laps and binding wire | Laps, hooks, cranks and chairs included in net steel (laps 5%, as Rev 0); binding wire 10 kg per ton of gross steel (E4). |
| Waterproofing compound | 1 kg per bag of cement in DPC, vertical DPC, septic plaster and water-tank concrete. |
| Earth (mitti) | Excavated earth bulks 25% (loose = 1.25 x dug); filling quantities include 20% for compaction in 6" layers (A4). |
| Trolley loads | One tractor-trolley = 100 cft of sand, crush, khangar or earth, or 2,000 bricks. |

## 2.3 Wastage (K2)

| Material | Rev 0 | Rev 1 | Note |
| --- | --- | --- | --- |
| Cement | 5% | 3% | buy fresh stock per stage |
| Sand (Chenab and Ravi) | 10% | 8% |  |
| Crush, khangar | 5% | 5% |  |
| Bricks | 5% | 5% | includes bats used in the work |
| Steel | 5% | 4% | cut lengths planned from the bar schedule |
| Pipes and conduits | 10% | 7% | fittings counted separately (no wastage) |
| Bitumen, membrane, coatings, primer, mesh, tiles | - | 5% | membrane laps are inside the quantity |
| Earth, binding wire, fittings, boxes, covers | - | 0% | counted items / compaction already allowed |

## 2.4 Level table - foundation to finished floor (B5)

| Level (in) | Level | What is at this level | Register |
| --- | --- | --- | --- |
| -48 | -4'-0" | Bottom of PCC under house footings (2'-6" below NSL; ~6" into the firm stratum) | C2 |
| -45 | -3'-9" | Top of PCC 3" / bottom of 27" (22½") brick step | C3 |
| -42 | -3'-6" | Firm natural soil starts (about 2'-0" below NSL) | A2 |
| -27 | -2'-3" | Top of three 6" brick steps; 13½" foundation wall (9" under 4½" walls) starts | C1 |
| -18 | -1'-6" | Natural surface level (NSL) | A1 |
| 0 | +0'-0" | Road crown; 13½" wall steps to 9" plinth wall | A1, C1 |
| 12 | +1'-0" | Finished level of side and rear passages (assumed, sloping 1:60 to drains) | G4 |
| 14 | +1'-2" | Finished lawn level (lawn filling excluded) | A3, A5 |
| 15 | +1'-3" | Finished level of both car porches; highest adjoining outside level | A3 |
| 22½ | +1'-10½" | Underside of 9"x9" RCC plinth beam | B4 |
| 23½ | +1'-11½" | Top of compacted earth fill inside the house | A3, A5 |
| 27½ | +2'-3½" | Top of 4" khangar (stone ballast) | B2 |
| 28½ | +2'-4½" | Top of 1" sand blinding; 0.2 mm polythene laid here | B2 |
| 31½ | +2'-7½" | Top of 3" PCC sub-floor = top of plinth beam = underside of DPC | B2, B4 |
| 33 | +2'-9" | Top of DPC = finished floor level (top of tile, +2'-9") | B4, B5 |
|  |  | B5 check: top of DPC = FFL +2'-9"; highest outside level = car porch +1'-3"; clearance 18" (minimum 9") - PASS | B5 |

Floor build-up above the compacted fill: khangar 4" + sand 1" + PCC 3" + DPC 1½" = 9½" (+1'-11½" to +2'-9"). The 1½" DPC concrete is laid continuous over the plinth beam and wall tops; the floor tile and its bed sit on the PCC inside the rooms so that the finished tile level equals the top of the DPC.

## 2.5 Roof build-up check (F3): must fit within 6½"

| Roof | Rain-water outlets | Max. screed | Membrane + bed + tile | Max. build-up | Limit | Result |
| --- | --- | --- | --- | --- | --- | --- |
| Terrace (first-floor roof) | 2 (as drawn) | 5.4" | 0.16 + ¾ + 1½" | 7.8" | 6½" | Too thick - used in total |
| Terrace (first-floor roof) | 4 (recommended) | 3.4" | 0.16 + ¾ + 1½" | 5.8" | 6½" | Fits - Section 9 |
| Mumty roof | 1 (as drawn) | 4.9" | 0.16 + ¾ + 1½" | 7.3" | 6½" | Too thick - used in total |
| Mumty roof | 2 (recommended) | 3.0" | 0.16 + ¾ + 1½" | 5.5" | 6½" | Fits - Section 9 |

Screed slope 1:100 (within the 1:80-1:100 range), minimum ¾" at outlets, thickness found on a 6" grid by outlet catchment. The total uses the drawn outlets (more screed); approving the extra outlets keeps the build-up within 6½" and saves screed (Section 9, point 4 in Section 11).

## 2.6 Edge and retaining walls (C6)

| No. | Edge wall (where the outside level changes) | Length | Section |
| --- | --- | --- | --- |
| 1 | Toe wall: lawn / front car porch (sheet 04) | 9.2 ft | D-D |
| 2 | Toe wall across left passage at TV-lounge front (sheet 04) | 4.9 ft | D-D |
| 3 | Rear car porch / left passage edge | 3.1 ft | D-D |
| 4 | Rear car porch / rear passage edge | 5.0 ft | D-D |
| 5 | Main-gate threshold: front porch / ramp | 12.8 ft | D-D |
| 6 | Side-gate threshold: rear porch / ramp | 7.7 ft | D-D |
|  | Total length | 42.7 ft |  |

Built to sheet-05 section D-D on PCC 2'-3" below NSL, 9" wall to the porch level with a DPC (items 4.7-4.10, 16.1, 16.10).

## 2.7 Drawing register and where each sheet is used

| Sheet | Content | Used for (BOQ items) |
| --- | --- | --- |
| Pages 1-2 | 3D front views | Elevation features 7.9; elevation RCC 8.23 |
| 01 | Ground floor plan (furnished) | Rooms and openings: walls 7.1-7.2; plaster 10.3, 10.6 |
| 02 | First floor plan (furnished) | Walls 7.3-7.4; plaster 10.4, 10.7 |
| 03 | Second floor (mumty) plan (furnished) | Walls 7.5-7.6; plaster 10.5, 10.8 |
| 04 | Ground floor plan with site levels | Levels A3; porch filling 6.3-6.4; edge walls 4.7-4.10; passages and ramps 16.3-16.8 |
| 05 | Foundation sections A-A to D-D | Excavation 2.1-2.3; footings 4.1-4.10; plinth beam and DPC 5.1-5.3 |
| 06 | Ground floor working plan | Walls 7.1-7.2; GF lintels 8.10; rear car porch 6.4 |
| 07 | First floor working plan | Walls 7.3-7.4; FF lintels 8.11; front balcony roof 9.1-9.6 |
| 08 | Second floor working plan | Walls and parapets 7.5-7.8; lintels 8.12 |
| 09-10 | Elevations | Elevation allowance 7.9, 8.23; parapets 7.7-7.8; external plaster 10.6-10.9 |
| 11 | Front boundary wall | Boundary walls 15.1-15.11 (heights G2) |
| 12 | Left-side boundary wall | Boundary walls 15.1-15.11 (heights G2) |
| 5 pages | Bath details | Sunken baths 8.28-8.30; water points 13.17-13.19; floor traps 11.13 |
| 13-15 | Electrical plans GF, FF, 2F | Conduits, boxes, DBs 14.1-14.18 |
| 16 | Electrical legend | Box types 14.17 |
| 17 | Ground floor sewerage (manholes, septic tank) | Manholes and sewers 11.1-11.13; septic tank 12.1-12.9 |
| 18-19 | First and second floor sewerage | Soil and waste stacks 11.10-11.11 |
| 20 | Roof drainage | Rain-water pipes 11.12; outlets and screed 9.1 |
| 21-23 | Water supply and gas plans | PPR and GI pipes 13.14-13.21 |
| 24 | Top roof plan (water tank) | Tank platform 13.11-13.13; roof waterproofing 9.1-9.6; solar plinths 14.21 |
| S-01 | Foundation layout plan and notes | Wall centre-lines (geometry); specifications 2.2 |
| S-02 | Foundation sections 1-1 to 4-4, plinth beam | Plinth-beam steel 5.1 only (RCC strip footing not used - C1) |
| S-03 | Stair plan and reinforcement | Stairs 8.16-8.19 |
| S-04 | Lintel key plans GF, FF, 2F | Lintels 8.10-8.12 |
| S-05 | Door and window lintel details | Lintels 8.10-8.12 |
| S-06 | Ground floor roof slab | Slab 8.2, beams 8.5, 8.7, bed plates 8.13, slab steel 8.25 |
| S-07 | First floor roof slab | Slab 8.3, beams 8.6, 8.8, bed plates 8.14, slab steel 8.26 |
| S-08 | Second floor (mumty) roof slab | Slab 8.4, beam 8.9, bed plates 8.15, slab steel 8.27 |
| S-09 | Roof slab beam sections CB-1, CB-2 | Concealed beams 8.5-8.9 |

## 2.8 Building elements and where they are in the BOQ

| Element | On the drawings | BOQ items |
| --- | --- | --- |
| Rooms and walls | All rooms of GF, FF and mumty; 9" and 4½" walls; parapets | 7.1-7.8 (masonry), 10.1-10.9 (mesh and plaster) |
| Openings and lintels | 58 lintels (S-04 key plans + openings without marks, incl. 8'-0" dining lintel) | 8.10-8.12; openings deducted in 7.1-7.6 |
| Slabs | 3 roof slabs (GF, FF, mumty) + porch cantilever; 4 sunken bath areas | 8.2-8.4, 8.25-8.27, 8.28-8.31 |
| Beams, bed plates, chajjas | CB-1, CB-2 concealed beams; 23 bed plates; 3 chajjas; parapet coping | 8.5-8.9, 8.13-8.15, 8.20-8.22, 8.24 |
| Stairs | Two zig-zag stairs (GF-FF, FF-mumty) with base footing | 8.16-8.19 |
| Column | Car-porch corner column 13½" with footing | 2.2, 4.5-4.6, 8.1, 16.2 |
| Tanks | Septic tank; underground water tank; roof-tank platform | 12.1-12.9; 13.1-13.10; 13.11-13.13 |
| Manholes | 9 manholes (7 RCC covers + 2 cast-iron covers) | 11.1-11.8 |
| Boundary walls | 6 stretches, 122.0 ft of panels, 10 pillars, 2 gates | 15.1-15.11 |
| External works | Edge walls, passages, ramps, steps | 4.7-4.10, 16.1-16.10 |

## 2.9 Rates used - Faisalabad, October 2026 (rate, date, source)

| Material | Unit | Rate (Rs) | Date | Source / note |
| --- | --- | --- | --- | --- |
| Cement |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | bag | 1,560 | Sep 2026 | ARY News "Cement Price in Pakistan Today", 15 Aug 2026 (Rs 1,500-1,610/bag); Sep 2026 brand list (Lucky 1,500-1,540, Bestway 1,430-1,570, Maple Leaf 1,570) - Faisalabad estimate |
| Bricks |  |  |  |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | nos | 16 | Jan-Oct 2026 | The News (PBS) Jan 2026: Lahore Rs 17,463/1,000; Pak Observer: Faisalabad DC-fixed A-class Rs 12,000/1,000 - market estimate Rs 16,000/1,000 |
| Steel |  |  |  |  |
| Steel Grade 60 deformed, #2 (6 mm) rings | kg | 270 | Sep 2026 | Grade 60 Rs 258-265/kg (Daily Ausaf 19 Sep 2026 / Daily Qudrat 26 Sep 2026 price reports) + small-bar premium (estimated) |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | kg | 262 | Sep 2026 | Grade 60 Rs 258-265/kg (Daily Ausaf 19 Sep 2026 / Daily Qudrat 26 Sep 2026 price reports) |
| Steel Grade 60 deformed, #4 (12 mm) | kg | 262 | Sep 2026 | as #3 |
| Steel Grade 60 deformed, #6 (20 mm) | kg | 262 | Sep 2026 | as #3 |
| Binding wire, annealed 18 gauge | kg | 400 | Oct 2026 | estimated |
| Sand |  |  |  |  |
| Chenab sand (concrete, screed) | cft | 65 | Jul 2026 | toolsfluent.com Pakistan construction cost guide 2026: river sand Rs 65/cft |
| Ravi sand (mortar, plaster, blinding, fill) | cft | 40 | Oct 2026 | estimated (about 60% of Chenab sand rate) |
| Crush |  |  |  |  |
| Crush ¾" down, Sargodha (bajri) | cft | 110 | Jul 2026 | toolsfluent.com 2026 guide: crush Rs 105/cft + delivery to Faisalabad (estimated) |
| Stone ballast |  |  |  |  |
| Stone ballast (khangar) 1½"-2½" | cft | 90 | Oct 2026 | estimated |
| Earth |  |  |  |  |
| Earth (mitti) for filling, delivered | cft | 25 | Oct 2026 | estimated (~Rs 2,500 per 100-cft tractor-trolley); 20% compaction already in quantity |
| Waterproofing |  |  |  |  |
| Polythene sheet 0.2 mm (800 gauge) | kg | 550 | Oct 2026 | estimated |
| Hot bitumen 60/70 (drum) | kg | 280 | 2025-26 | Argus Media: NRL 60/70 bitumen FOB Karachi $380-420/t (Sep-Nov 2025) + local drum and dealer margin (estimated) |
| Integral waterproofing compound (powder), 1 kg per bag of cement | kg | 350 | Oct 2026 | estimated |
| Bitumen primer | L | 900 | Oct 2026 | estimated |
| 4 mm APP/SBS modified-bitumen membrane (1 m x 10 m roll) | m2 | 700 | Oct 2026 | estimated, material only |
| LPG for torching (11.8 kg cylinder) | kg | 300 | Oct 2026 | estimated (often supplied by applicator) |
| Polymer-modified cementitious waterproof coating (2-component) | kg | 450 | Oct 2026 | estimated |
| Food-safe (potable-water) cementitious coating | kg | 550 | Oct 2026 | estimated |
| PVC water-stop 150 mm | ft | 650 | Oct 2026 | estimated |
| Anti-termite |  |  |  |  |
| Termiticide: bifenthrin 10% EC (1 L makes 200 L emulsion) | L | 3,500 | Oct 2026 | estimated |
| Roof tiles |  |  |  |  |
| Bhatta (brick) roof tiles 9" x 4½" x 1½" | nos | 20 | Oct 2026 | estimated |
| Chicken mesh |  |  |  |  |
| Galvanised chicken mesh, roll 3 ft x 100 ft | roll | 5,500 | Oct 2026 | estimated |
| Concrete nails with washers for mesh | kg | 450 | Oct 2026 | estimated |
| Sewer pipes |  |  |  |  |
| UPVC sewer pipe 5" (Dadex / Beta) | ft | 420 | Oct 2026 | estimated |
| UPVC sewer pipe 4" | ft | 300 | Oct 2026 | estimated |
| UPVC pipe 3" (waste / vent) | ft | 200 | Oct 2026 | estimated |
| UPVC rain-water pipe 3" | ft | 200 | Oct 2026 | estimated |
| UPVC fittings 5" (bends, tees, sockets) | nos | 950 | Oct 2026 | estimated |
| UPVC fittings 4" | nos | 600 | Oct 2026 | estimated |
| UPVC fittings 3" incl. floor / gully traps, roof outlets | nos | 380 | Oct 2026 | estimated |
| Vent cowl 3" | nos | 600 | Oct 2026 | estimated |
| PVC drain pipe ¾" (20 mm) for A.C condensate | ft | 35 | Oct 2026 | estimated |
| Stainless-steel mesh cap for A.C drain outlet | nos | 350 | Oct 2026 | estimated |
| Covers |  |  |  |  |
| Heavy-duty CI manhole cover with frame 20" x 20" | nos | 15,000 | Oct 2026 | estimated |
| Lockable steel / CI tank cover 2' x 2' | nos | 12,000 | Oct 2026 | estimated |
| Water pipes |  |  |  |  |
| PPR pipe 1¼" (40 mm) PN-16 (Popular / Dadex / Master) | ft | 270 | May 2026 | icons.com.pk PPRC price list 25 May 2026 (32 mm Rs 463-751/m) - 40 mm estimated |
| PPR pipe 1" (32 mm) PN-16 | ft | 175 | May 2026 | icons.com.pk 25 May 2026: 32 mm Rs 463-751/m |
| PPR pipe ¾" (25 mm) PN-16, cold | ft | 110 | May 2026 | icons.com.pk 25 May 2026: 25 mm Rs 288-462/m |
| PPR pipe ½" (20 mm) PN-16, cold | ft | 70 | May 2026 | icons.com.pk 25 May 2026 (from Rs 186/m) - estimated |
| PPR pipe ¾" (25 mm) PN-20, hot | ft | 135 | May 2026 | icons.com.pk 25 May 2026 upper range - estimated |
| PPR pipe ½" (20 mm) PN-20, hot | ft | 85 | May 2026 | estimated |
| PPR fittings (average) | nos | 120 | Oct 2026 | estimated |
| Ball / gate valves (average) | nos | 900 | Oct 2026 | estimated |
| Float valve ¾" for underground tank | nos | 2,500 | Oct 2026 | estimated |
| Gas pipes |  |  |  |  |
| GI pipe medium class ½"-¾" for gas (SNGPL-approved) | ft | 260 | Oct 2026 | estimated |
| GI fittings ½"-¾" | nos | 250 | Oct 2026 | estimated |
| Electrical |  |  |  |  |
| PVC conduit ¾" heavy gauge | ft | 25 | Oct 2026 | estimated |
| PVC conduit 1" heavy gauge | ft | 38 | Oct 2026 | estimated |
| PVC conduit 1¼" heavy gauge | ft | 56 | Oct 2026 | estimated |
| PVC conduit 1½" heavy gauge | ft | 75 | Oct 2026 | estimated |
| Conduit bend ¾" | nos | 35 | Oct 2026 | estimated |
| Conduit bend 1" | nos | 55 | Oct 2026 | estimated |
| Conduit bend 1¼" | nos | 90 | Oct 2026 | estimated |
| Conduit bend 1½" | nos | 120 | Oct 2026 | estimated |
| Conduit coupler ¾" | nos | 15 | Oct 2026 | estimated |
| Conduit coupler 1" | nos | 25 | Oct 2026 | estimated |
| Conduit coupler 1¼" | nos | 40 | Oct 2026 | estimated |
| Conduit coupler 1½" | nos | 55 | Oct 2026 | estimated |
| PVC solvent cement (tin) | nos | 600 | Oct 2026 | estimated |
| Switch-board back box (assorted 3"x3" to 12"x3") | nos | 250 | Oct 2026 | estimated |
| Socket back box | nos | 120 | Oct 2026 | estimated |
| A.C point box | nos | 150 | Oct 2026 | estimated |
| Fan box with hook | nos | 180 | Oct 2026 | estimated |
| Round light junction box | nos | 60 | Oct 2026 | estimated |
| Ceiling box for Wi-Fi access point | nos | 150 | Oct 2026 | estimated |
| CCTV camera junction box | nos | 150 | Oct 2026 | estimated |
| NVR / router enclosure box | nos | 400 | Oct 2026 | estimated |
| Bell push box | nos | 120 | Oct 2026 | estimated |
| DB enclosure 12-way (empty) | nos | 4,500 | Oct 2026 | estimated |
| DB enclosure 8-way (empty) | nos | 3,200 | Oct 2026 | estimated |
| DB enclosure 6-way (empty) | nos | 2,800 | Oct 2026 | estimated |
| DB enclosure 4-way (empty) | nos | 2,000 | Oct 2026 | estimated |
| EV charger isolator enclosure | nos | 2,500 | Oct 2026 | estimated |
| PVC sleeve 2" | ft | 120 | Oct 2026 | estimated |
| Earthing |  |  |  |  |
| Copper-bonded earth rod 5/8" x 10 ft | nos | 9,000 | Oct 2026 | estimated |
| Earthing compound, 25 kg bag | bag | 3,500 | Oct 2026 | estimated |
| Earth-pit inspection chamber with cover | nos | 3,000 | Oct 2026 | estimated |
| Solar plinths |  |  |  |  |
| M12 x 250 mm galvanised J-bolt with nut and washer | nos | 250 | Oct 2026 | estimated |

72 of 80 rates are wholly or partly estimated and are marked "estimated" (local market judgement, mostly small items). The main cost items - cement, bricks, steel, Chenab sand, crush and PPR pipes - are based on 2026 published reports and make up about 79% of the total.

# 3. Revision log - what changed from Rev 0 and why

## 3.1 Quantity changes

| Item | Rev 0 | Rev 1 | Change | Register | Why |
| --- | --- | --- | --- | --- | --- |
| Foundations |  |  |  |  |  |
| Excavation of house trenches (cft) | 2,666.3 | 2,550.4 | -116.0 | C1, C2 | sheet-05 widths 3'-0" / 2'-6"; PCC bottom 2'-6" below NSL |
| PCC 1:4:8 3" under footings (cft) | 242.4 | 231.9 | -10.5 | C1, C3 | narrower sheet-05 PCC |
| RCC strip footing 6" (cft) - S-02 | 416.8 | 0.0 | -416.8 | C1 | removed; its 673 kg of #3 steel removed too |
| Foundation brickwork 1:6 below road (cft) | 1,075.4 | 1,717.2 | +641.9 | C1 | steps 27" / 22½" / 18" + 13½" wall up to road level |
| Plinth brickwork above road (cft) | 744.6 | 507.3 | -237.3 | B4 | now from road level to underside of beam at floor level |
| RCC plinth beam 9" x 9" (cft) | 203.1 | 202.9 | -0.1 | B4 | moved from road level to floor level (sheet 05); steel per S-02 |
| DPC 1½" 1:2:4 (cft) | 33.8 | 33.8 | nil | B4, B5 | now with waterproofing compound + 2 coats bitumen; top = FFL |
| Vertical DPC (sft) | 607.9 | 716.5 | +108.6 | B6 | height 4'-1½" (was 3'-6"); bitumen at 0.2 kg/sft/coat (was 0.25) |
| Car-porch column (cft) | 21.5 | 20.9 | -0.6 | C5 | height 16'-6" with new levels |
| Edge / toe walls (cft of brickwork) | 28.1 | 196.2 | +168.1 | C6 | 6 edge walls 42'-9" (D-D) replace 15 ft lawn toe wall |
| Earthwork |  |  |  |  |  |
| Back-filling of house trenches (cft) | 965.0 | 1,142.8 | +177.9 | A3 | from the sheet-05 profile |
| Earth filling inside house (cft) | 5,791.0 | 5,654.8 | -136.3 | A3, A5 | floor build-up now 9½" |
| Earth filling under car porches (cft) | 614.2 | 1,060.3 | +446.1 | A5 | both porches full area (451 sft, was 246) |
| Earth to buy (cft) | 3,250.0 | 4,184.8 | +934.8 | A3-A5, G4 | porches, passages and ramps now filled |
| Anti-termite |  |  |  |  |  |
| Termiticide concentrate (L) | 0.0 | 11.3 | +11.3 | B1 | new: trenches, plinth fill, perimeter |
| Sub-floor |  |  |  |  |  |
| Sand fill 4" under floors (cft) | 589.7 | 0.0 | -589.7 | B2 | replaced by khangar + sand blinding + polythene |
| Khangar 4" (cft) | 0.0 | 665.1 | +665.1 | B2 | new capillary break |
| Sand blinding 1" (cft) | 0.0 | 188.9 | +188.9 | B2 | new |
| Polythene 0.2 mm (sft) | 0.0 | 2,185.2 | +2,185 | B2, B3 | new ("graphite sheet") |
| PCC 3" sub-floor, rooms + porches (cft) | 402.1 | 453.4 | +51.4 | A5, B2 | porches full area |
| Upper-floor 3" PCC sub-base (cft) | 436.3 | 388.5 | -47.8 | E3 | less sunken bath areas |
| Masonry |  |  |  |  |  |
| 9" walls GF+FF+mumty: cement (bags) | 190.8 | 163.5 | -27.3 | D1 | mortar 1:6 (was 1:5); brick volume unchanged |
| RCC |  |  |  |  |  |
| Superstructure RCC (slabs, beams, lintels, stairs, chajjas, coping) | as Rev 0 | as Rev 0 | nil | E1, E2 | unchanged; dining-opening 8'-0" lintel kept |
| Sunken bath slabs: extra concrete (cft) | 0.0 | 25.5 | +25.5 | E3 | new: + steel, coating and brick-bat fill |
| Roof |  |  |  |  |  |
| Roof waterproofing and tiles (sft of roof) | optional (not in total) | 1,658.8 | now in total | F3 | was optional item O1 (not in total); now screed + membrane + gola + bhatta tiles |
| Plaster |  |  |  |  |  |
| Ceiling plaster (sft) | 3,442.2 | 0.0 | -3,442 | F1 | deleted (false ceilings) |
| External soffit plaster (sft) | 0.0 | 240.8 | +240.8 | F1 | new separate line |
| GF external plaster (sft) | 1,595.2 | 1,428.5 | -166.8 | A3, F1 | exposed plinth 2'-0" (was 3'-3") |
| Chicken mesh (rolls 3' x 100') | 0.0 | 16.4 | +16.4 | D3 | new: all junctions and chases |
| Boundary |  |  |  |  |  |
| Boundary excavation (cft) | 878.0 | 790.2 | -87.8 | G3 | PCC bottom 2'-3" below NSL |
| Boundary brickwork below DPC (cft) | 319.3 | 618.2 | +298.9 | G3 | measured from natural ground (adds wall below band) |
| Boundary wall above DPC (cft) | 646.3 | 484.8 | -161.5 | D1, G1, G2 | mortar 1:6 (was 1:5); own back and right walls |
| Boundary DPC on plinth band (cft) | 0.0 | 13.3 | +13.3 | G3 | new |
| Boundary plaster (sft) | 2,055.2 | 2,089.6 | +34.4 | F1 | both faces on all sides |
| Septic tank |  |  |  |  |  |
| Septic tank internal size | 6'-0" x 4'-0" x 5'-6", 2 chambers | 4'-10" x 3'-5" x 5'-0", 3 chambers | changed | H1 | drawing size, read as internal |
| Sewerage |  |  |  |  |  |
| Manhole covers | 9 RCC | 7 RCC + 2 cast iron | changed | H2 | heavy covers in car porches |
| Manhole 9" brickwork (cft) | 346.5 | 259.9 | -86.6 | H2 | arithmetic corrected: 9 x centre-line 11 ft x 9" x 3'-6" (Rev 0 left out the 9" thickness) |
| A.C condensate drains ¾" (ft) | 0.0 | 200.0 | +200.0 | H3 | new, with stainless mesh caps |
| Water |  |  |  |  |  |
| RCC overhead tank 1,000 gal (cft RCC) | 106.7 | 0.0 | -106.7 | I4 | deleted; plastic tank (finishing) on RCC platform |
| Roof-tank platform (cft RCC) | 0.0 | 8.3 | +8.3 | I4 | new |
| Underground water tank (cft RCC) | 0.0 | 75.9 | +75.9 | I1, I2 | was optional O2; now ~515 gal, at back |
| PPR ¾" cold incl. society supply line (ft) | 369.7 | 430.0 | +60.3 | I3, I5 | +60 ft supply to UGT |
| PPR 1"-1¼" mains (ft) | 118.0 | 131.0 | +13.0 | I3 | pump at back |
| Electrical |  |  |  |  |  |
| ¾" conduit (ft) | 2,900.0 | 2,662.0 | -238.0 | J2, J8 | no TV / intercom / +10% UPS; data + CCTV added |
| 1" conduit (ft) | 1,390.0 | 1,575.0 | +185.0 | J1, J5, J7 | earth pits, pump, spare roof AC |
| 1¼" conduit (ft) | 0.0 | 172.0 | +172.0 | J4, J5 | inverter, backup DBs, DC spare |
| 1½" conduit (ft) | 85.0 | 120.0 | +35.0 | J6 | EV charger run added |
| DB enclosures (nos) | 3.0 | 6.0 | +3.0 | J5 | 3 backup (essential-load) DBs added + EV box |
| Earth pits (nos) | 0.0 | 3.0 | +3.0 | J7 | new |
| Solar mounting plinths (nos) | 0.0 | 34.0 | +34.0 | J3 | new, 23 panels x 650 W |
| External |  |  |  |  |  |
| Passage paving base (sft) | 0.0 | 265.0 | +265.0 | G4 | was optional O3 |
| Ramps at both gates (sft) | 0.0 | 183.3 | +183.3 | G4 | new |
| Standards |  |  |  |  |  |
| Wastage cement / sand / steel / pipes | 5% / 10% / 5% / 10% | 3% / 8% / 4% / 7% | changed | K2 | crush and bricks stay 5% |
| Binding wire (kg) | 85.0 | 75.0 | -10.0 | E4 | 10 kg per ton of steel |

## 3.2 Register compliance - every line A1 to K3, how it is applied and where

| Line | Decision (Register) | How it is applied in Rev 1 | BOQ items |
| --- | --- | --- | --- |
| A1 | Road crown = ±0; natural surface level (NSL) 1'-6" below road. | All levels in this estimate are from the road crown; NSL -1'-6" (level table 2.4). | 1.1, 2.1 |
| A2 | Firm soil about 2'-0" below NSL; not saline; no ground water; use OPC. | PCC bottom about 6" into firm soil; OPC throughout; no sulphate-resisting cement or dewatering. | all concrete and mortar |
| A3 | FFL +2'-9", car porches +1'-3", lawn +1'-2"; recalculate excavation, back-filling and filling. | Excavation, back-filling and filling recomputed for the new levels. | 1.1, 2.1, 6.1-6.2 |
| A4 | Re-use clean excavated earth, buy the balance, 6" layers, 20% compaction. | Dug 4,890 cft, back-filled 2,348 cft, surplus re-used 3,177 cft (loose); fill 7,362 cft incl. 20%; earth to buy 4,185 cft (live formula in Excel). | 6.1-6.2, 6.9 |
| A5 | Fill house and both porches (full area) to sub-floor; passages, driveway and ramps to underside of paving; lawn not filled. | House 5,655, porches 1,060, passages 583, ramps 64 cft; lawn excluded. | 6.2-6.4, 6.9, 16.3 |
| B1 | Termiticide (bifenthrin or imidacloprid) on trenches, plinth fill and outside perimeter; give litres of concentrate. | Bifenthrin 10% EC at 1:200: trenches 6.5 L, plinth fill 4.2 L, perimeter 0.6 L = 11.3 L (buy 12 L). | 3.1-3.3 |
| B2 | Sub-floor: compacted earth, 4" khangar, 1" sand, polythene 0.2 mm (6" laps, turned up), 3" PCC 1:4:8; no roṛi. | Khangar 665 cft, sand 189 cft, polythene 2,185 sft, PCC 453 cft. | 6.5-6.8 |
| B3 | "Graphite sheet" means the polythene. | 0.2 mm (800 gauge) polythene, 37 kg. | 6.7 |
| B4 | Architect's beam at floor level plus 1½" 1:2:4 DPC with waterproofing admixture and 2 coats hot bitumen over all walls. | Plinth beam at floor level; DPC on house walls, edge walls and boundary walls. | 4.4, 4.10, 5.1-5.2, 15.6 |
| B5 | Top of DPC = FFL (top of tile), at least 9" above the highest outside level; give a level table. | DPC top +2'-9"; porch +1'-3"; clearance 18" - PASS (table 2.4). | 5.2 |
| B6 | Vertical DPC: ¾" 1:3 plaster with waterproofing compound + 2 coats bitumen at 0.2 kg/sft per coat. | 716 sft on the fill side of the outer plinth walls (NSL to top of sub-floor). | 5.3 |
| C1 | Sheet 05 footing; no RCC strip footing. | Sheet-05 brick footing; S-02 RCC strip footing (417 cft, 673 kg steel) deleted. | 2.1, 4.1-4.4 |
| C2 | PCC bottom 2'-6" below NSL (-4'-0" from road). | PCC bottom -4'-0" under all house walls. | 2.1 |
| C3 | PCC 1:4:8, 3" thick; foundation brickwork 1:6. | As decided in all footing items. | 4.1-4.2, 4.7 |
| C4 | Eccentric footings on all property-line walls, including the powder-room wall. | Footing bands cut at the property line (right side incl. powder room); all boundary-wall footings eccentric. | 2.1, 15.3 |
| C5 | Porch column 13½", 4 #6, #3 ties @ 6", 1:1.5:3, on 3' x 3' x 1' footing with #4 @ 6", on 3" PCC. | As decided; column height 16'-6" with the new levels. | 2.2, 4.5-4.6, 8.1, 16.2 |
| C6 | Identify and list edge / retaining walls. | 6 walls, 42.7 ft (table 2.6), to section D-D. | 2.3, 4.7-4.10, 16.1, 16.10 |
| D1 | 9" walls 1:6; 4½" walls, parapets, pillars 1:4; boundary walls 1:6 with pillars 1:4. | Mortar ratios as decided in every wall item. | 4.9, 7.1-7.8, 15.4, 15.7-15.8 |
| D2 | Awwal bricks at 13.5/cft; heights, parapets and elevation allowance as before; no privacy walls. | As decided; elevation allowance 5% kept; no privacy walls. | 7.1-7.9, 8.31 |
| D3 | Chicken mesh strips on all junctions and chases; give running feet and rolls. | 12" strips 3,131 rft + 9" strips 1,805 rft = 16.4 rolls (buy 18). | 10.1-10.2 |
| E1 | Superstructure RCC as before. | Slabs, beams, lintels, stairs, chajjas, coping as Rev 0 (S-03 to S-09). | 8.1-8.27 |
| E2 | 8'-0" dining lintel; 9" bearing; no lintel band. | DL-1 8'-0" in GF lintels; 9" bearings; no lintel band. | 8.10 |
| E3 | 6" sunken slabs under FF and mumty baths: extra concrete and steel, coating, lightweight fill. | 191 sft sunk: concrete 25.5 cft + 101 kg #3; coating 69 kg; brick-bat fill 143 cft. | 8.28-8.31 |
| E4 | Laps as before; binding wire 10 kg/ton; steel by size and member group. | Laps 5% as Rev 0; binding wire 75 kg; schedule in Section 6. | 8.25-8.27 |
| F1 | Plaster all walls, parapets and boundary walls on both faces on all sides; no ceiling plaster; soffits separate. | Ceiling plaster deleted; soffits 241 sft separate (10.10); boundary walls both faces. | 10.3-10.10, 15.10, 16.10 |
| F2 | Internal ½" 1:4; external ¾" 1:4. | Internal 12,879 sft; external 3,412 sft; parapets 1,314 sft. | 10.3-10.8 |
| F3 | Roof: screed 1:80-1:100, primer, 4 mm torch membrane (4" laps, 12" upturn into groove), gola, bhatta tiles in 1:4; check 6½"; insulation only as a recommendation. | 1,659 sft of roof; build-up check 2.5; extra outlets and insulation in Section 9. | 9.1-9.6 |
| G1 | Own boundary walls on all sides: back 37'-3" x 7'-0" (gali), right walls on open stretches. | Back wall and right-side walls (9'-0" and 5'-5") added. | 15.1, 15.7 |
| G2 | Street wall heights per drawing. | Front 6'-0", left long panel 7'-3" (sheets 11-12). | 15.7-15.8 |
| G3 | Boundary footing from NSL, PCC bottom about 2'-3" below NSL; DPC at plinth-band level. | As decided; DPC on the plinth band (15.6). | 15.1-15.6, 15.9, 15.11 |
| G4 | Passages and driveway: fill, sand and PCC base sloped 1:60; ramps at both gates; steps as before. | Passages 265 sft and ramps 183 sft: fill, 2" sand, 3" PCC; steps 40 cft. | 16.3-16.9 |
| H1 | Septic tank: 3 chambers at drawing size, liquid depth + freeboard, 3" PCC + 6" RCC base, 9" brick 1:4, baffles, RCC top with 2 openings and covers, ¾" 1:3 WP plaster in and out, T-pipes, vent. | Drawing size read as INTERNAL 4'-10" x 3'-5", 5'-0" deep; all parts included; enlargement in Section 9. | 12.1-12.9 |
| H2 | Manholes as before; name two UPVC brands; CI / heavy covers in porches and driveway. | 9 manholes as Rev 0, with the Rev 0 brickwork arithmetic corrected (table 3.1); Dadex or Beta UPVC; 2 heavy-duty CI covers in the car porches. | 11.1-11.13 |
| H3 | ¾" A.C drains to floor traps with stainless-steel mesh caps. | 200 ft ¾" PVC, 10 caps. | 11.14 |
| I1 | Underground tank at the back, about septic size, at least 10 ft from septic tank and manholes. | Rear car porch, internal 4'-10" x 3'-5" x 5'-6"; about 8½ ft from two rear manholes (Section 11). | 13.1, 13.10 |
| I2 | Watertight: 6" RCC 1:1.5:3 + admixture, PVC water-stop, food-safe coating inside, bitumen outside, lockable cover, top 6"-9" above ground. | All included; top 7" above porch level. | 13.2-13.9 |
| I3 | Society supply → underground tank (float valve) → pump → plastic roof tank. | Float valve; 60 ft supply line; rising main 53 ft of 1" PPR. | 13.14-13.21 |
| I4 | No RCC overhead tank; plastic tank on a platform over walls; recommend capacity. | RCC tank deleted; 5' x 5' RCC platform on two 9" walls over mumty walls; 500-gallon tank recommended. | 13.11-13.13 |
| I5 | PPR PN-20 hot, PN-16 cold; SNGPL-approved gas pipe; brands; update rising main. | As decided (Popular / Dadex / Master). | 13.14-13.21 |
| J1 | 10 A.C points, each on its own 1" conduit. | 10 separate runs, 385 ft. | 11.14, 14.7-14.10, 14.17 |
| J2 | No TV / intercom; data conduits to Wi-Fi APs, CCTV conduits to one NVR point; propose positions. | 5 access points, 7 cameras, 1 NVR point (positions in Section 10). | 14.3-14.5, 14.17 |
| J3 | Solar 15 kW, about 23 x 650 W on terrace and mumty roof; area; RCC plinths with cast-in bolts before the membrane. | 23 panels = 14.95 kWp; 34 plinths, 68 J-bolts (Section 7). | 9.3, 14.21 |
| J4 | Two spare conduits (DC and AC); inverter and sodium-ion battery position on the 2nd floor. | 1¼" DC + 1" AC spares; inverter and battery in the mumty laundry with RCC plinth. | 14.11-14.13, 14.20, 14.22 |
| J5 | Backup sub-DB scheme. | Backup DBs on GF, FF and mumty fed from the inverter. | 14.7-14.13, 14.18 |
| J6 | EV charger conduit for 7.4 kW single-phase or 11 kW three-phase. | 1½" conduit to the front car porch + isolator box. | 14.14-14.15, 14.18 |
| J7 | Separate earth pits; state how many. | 3 pits: house, solar / inverter, lightning / surge. | 14.7-14.10, 14.19 |
| J8 | Other electrical allowances kept; meter at the front. | Lights, fans, sockets, feeders and bells as Rev 0; meter at the front. | 14.1-14.10, 14.14-14.17 |
| K1 | Material only; list exclusions. | No labour anywhere; exclusions in Section 8. | whole BOQ |
| K2 | Wastage: cement 3%, sand 8%, crush 5%, bricks 5%, steel 4%, pipes 7%. | Applied per material (table 2.3; editable on the Excel Rates sheet). | whole BOQ (Rates sheet) |
| K3 | Conversion factors as before; OPC, Chenab and Ravi sand, ¾" Sargodha crush, Grade-60 steel; name brands. | Table 2.2; brands in the material names. | whole BOQ |

# 4. Detailed BOQ by construction stage, with workings

Order of work: levels → excavation → anti-termite → foundations → plinth beam and DPC → filling and sub-floor → masonry → RCC → roof waterproofing and tiles → plaster → sewerage → septic tank → water tank → electrical and solar plinths → boundary walls → external works.

Material columns are NET quantities (before wastage). Each stage closes with its net total and its total with wastage (2.3). Amount = quantity with wastage x rate (Rs, shown to the rupee; subtotals are exact sums, so a column of rounded amounts can differ from its subtotal by a rupee; the closing block is shown to the paisa). Purchase rounding to buying units and trolley loads are in Section 5. Steel is by bar size: #2 = 6 mm, #3 = 10 mm, #4 = 12 mm, #6 = 20 mm.

| Item | Location / description | Dimensions / working | Qty | Unit | Cement (bags) | Chenab sand (cft) | Ravi sand (cft) | Crush (cft) | Khangar (cft) | Bricks (nos) | Steel (kg) | Other materials | Register | Amount (Rs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 Levels and layout |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | Whole plot Level survey and setting out (no materials) | Road crown = ±0; NSL = -1'-6"; FFL +2'-9"; porches +1'-3"; lawn +1'-2" = Take levels at the 4 corners and centre before digging; set out walls from sheets 04/06 (architect's dimensions govern) | 1 | job |  |  |  |  |  |  |  |  | A1, A3 | - |
|  | S1 total - net |  |  |  |  |  |  |  |  |  |  |  |  | - |
| S2 Excavation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | All house walls Excavation of wall trenches to PCC bottom, 2'-6" below NSL (4'-0" below road) | PCC footprint 927.4 sft (3'-0" under 9" walls, 2'-6" under 4½" walls; union of all walls, eccentric on property line) = 927.4 sft x 2.50 ft x 1.10 working space | 2,550.4 | cft |  |  |  |  |  |  |  |  | A1, A3, C1, C2, C4 | - |
| 2.2 | Car-porch front corner Excavation for isolated column footing | 3'-6" x 3'-6" x 2'-6" deep = 3.5 x 3.5 x 2.5 x 1.10 | 33.7 | cft |  |  |  |  |  |  |  |  | C5 | - |
| 2.3 | Porch, lawn and gate edges Excavation for edge / retaining walls (section D-D), PCC bottom 2'-3" below NSL | 42.7 ft x 2'-6" x 2'-3" = 42.71 x 2.5 x 2.25 x 1.10 | 264.3 | cft |  |  |  |  |  |  |  |  | C6 | - |
|  | S2 total - net |  |  |  |  |  |  |  |  |  |  |  |  | - |
| S3 Anti-termite treatment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | House trenches Termiticide on trench bottoms and sides, before PCC | bottoms 927 sft + sides 2 x 370.7 ft x 2'-6" = 2,781 sft = 258.4 m2 x 5 L/m2 = 1,292 L emulsion; ÷ 200 | 6.5 | L |  |  |  |  |  |  |  | Termiticide 6.5 L | B1 | 23,737 |
| 3.2 | House floor + both car porches Termiticide on top of compacted plinth filling, before khangar | rooms 1,363 + front porch 308 + rear porch 143 sft = 1,814 sft = 168.5 m2 x 5 L/m2 = 843 L emulsion; ÷ 200 | 4.2 | L |  |  |  |  |  |  |  | Termiticide 4.2 L | B1 | 15,482 |
| 3.3 | House perimeter Termiticide in soil along outside of plinth walls, after back-filling | outer walls 173.7 ft = 52.9 m x 2.25 L/m = 119 L emulsion; ÷ 200 | 0.6 | L |  |  |  |  |  |  |  | Termiticide 0.6 L | B1 | 2,189 |
|  | S3 total - net |  |  |  |  |  |  |  |  |  |  |  |  | 41,407 |
| S4 Foundations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | All house walls PCC 1:4:8, 3" thick, under brick footings (sheet 05 widths 3'-0" / 2'-6") | 927.4 sft x 3" = 927.4 x 0.25 | 231.9 | cft | 22.4 | 112.0 |  | 224.0 |  |  |  |  | C1, C3 | 69,728 |
| 4.2 | All house walls Brick footing in 1:6: steps 27" / 22½" / 18" under 9" walls, 22½" / 18" / 13½" under 4½" walls, 6" each | union plan areas 719.8 + 604.8 + 485.2 sft x 6" = (719.8 + 604.8 + 485.2) x 0.5 | 904.9 | cft | 41.3 |  | 309.5 |  |  | 12,216 |  |  | C1, C3 | 284,907 |
| 4.3 | All house walls Foundation wall in 1:6 from top of steps (-2'-3") to road level: 13½" under 9" walls, 9" under 4½" walls | plan 361.0 sft x 2'-3" = 361.0 x 2.25 | 812.3 | cft | 37.0 |  | 277.8 |  |  | 10,967 |  |  | C1 | 255,759 |
| 4.4 | All house walls Plinth wall 9" in 1:6 from road level to underside of plinth beam (+1'-10½") | plan 270.6 sft x 1'-10½" = 270.6 x 1.875 | 507.3 | cft | 23.1 |  | 173.5 |  |  | 6,849 |  |  | C1, B4 | 159,726 |
| 4.5 | Car-porch front corner PCC 1:4:8, 3", under isolated column footing | 3'-6" x 3'-6" x 3" = 3.5 x 3.5 x 0.25 | 3.1 | cft | 0.3 | 1.5 |  | 3.0 |  |  |  |  | C5 | 921 |
| 4.6 | Car-porch front corner RCC isolated footing 1:2:4, 3'-0" x 3'-0" x 1'-0", #4 @ 6" both ways (the only RCC footing) | 3' x 3' x 1' = 9.0 cft; #4: 2 ways x 7 bars x 2'-9" | 9.0 | cft | 1.6 | 4.0 |  | 8.1 |  |  | #4 11.7 |  | C5 | 6,989 |
| 4.7 | Porch, lawn and gate edges Edge walls (D-D): PCC 1:4:8, 3" x 2'-6" | 42.7 ft x 2'-6" x 3" = 42.71 x 2.5 x 0.25 | 26.7 | cft | 2.6 | 12.9 |  | 25.8 |  |  |  |  | C6, C3 | 8,028 |
| 4.8 | Porch, lawn and gate edges Edge walls: brick steps 22½" / 18" / 13½" (6" each) in 1:6 | 42.7 ft = 42.71 x (1.875 + 1.5 + 1.125) x 0.5 | 96.1 | cft | 4.4 |  | 32.9 |  |  | 1,297 |  |  | C6 | 30,256 |
| 4.9 | Porch, lawn and gate edges Edge walls: 9" wall in 1:6 from step top (-2'-0") to +1'-1½" (under DPC) | 42.7 ft x 9" x 3'-1½" = 42.71 x 0.75 x 3.125 | 100.1 | cft | 4.6 |  | 34.2 |  |  | 1,351 |  |  | C6, D1 | 31,516 |
| 4.10 | Porch, lawn and gate edges Edge walls: DPC 1½" 1:2:4 with waterproofing compound + 2 coats hot bitumen, top at porch level | 42.7 ft x 9" = 42.71 x 0.75 x 0.125; bitumen 32.0 sft x 0.4 kg | 4.0 | cft | 0.7 | 1.8 |  | 3.6 |  |  |  | WP compound 0.7 kg Bitumen 12.8 kg | B4, C6 | 5,714 |
|  | S4 total - net |  |  |  | 138.0 | 132.2 | 827.9 | 264.4 |  | 32,680 | #4 11.7 |  |  | 853,544 |
|  | S4 total - with wastage |  |  |  | 142.1 | 142.8 | 894.1 | 277.6 |  | 34,314 | #4 12.1 |  |  |  |
| S5 Plinth beam and DPC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | All house walls RCC plinth beam 9" x 9" (1:2:4) at floor level per sheet 05; 2 #4 top + 2 #4 bottom, #2 rings @ 10" | plan 270.6 sft x 9"; centre-line 370.7 ft = concrete 270.6 x 0.75; #4: 4 x (370.7 x 1.05 + 15 ends x 1'); rings (370.7 x 12/10 + 1) x 2'-4" | 202.9 | cft | 36.4 | 91.0 |  | 182.1 |  |  | #2 78.7 #4 490.0 |  | B4 | 241,521 |
| 5.2 | All house walls DPC: continuous 1½" 1:2:4 concrete with integral waterproofing compound, + 2 coats hot bitumen, on top of plinth beam; top of DPC = FFL | plan 270.6 sft x 1½" = 270.6 x 0.125; bitumen 270.6 sft x 2 coats x 0.2 kg | 33.8 | cft | 6.1 | 15.2 |  | 30.3 |  |  |  | WP compound 6.1 kg Bitumen 108.2 kg | B4, B5 | 48,263 |
| 5.3 | Outer plinth walls, fill side Vertical DPC: ¾" 1:3 plaster with waterproofing compound + 2 coats hot bitumen (0.2 kg/sft per coat) | 173.7 ft x 4'-1½" (NSL to top of sub-floor PCC) = 173.7 x 4.125 = 716 sft | 716.5 | sft | 14.9 |  | 55.8 |  |  |  |  | WP compound 14.9 kg Bitumen 286.6 kg | B6 | 115,806 |
|  | S5 total - net |  |  |  | 57.4 | 106.2 | 55.8 | 212.4 |  |  | #2 78.7 #4 490.0 |  |  | 405,590 |
|  | S5 total - with wastage |  |  |  | 59.1 | 114.7 | 60.3 | 223.0 |  |  | #2 81.8 #4 509.5 |  |  |  |
| S6 Filling and sub-floor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | All house walls Back-filling of trenches with excavated earth, watered and rammed in 6" layers | trench 2,550 cft less footing below NSL 1,408 cft = 2,550 - 1,408 | 1,142.8 | cft |  |  |  |  |  |  |  |  | A3, A4 | - |
| 6.2 | Inside house plinth Earth filling from NSL to underside of khangar (+1'-11½"), 6" layers watered and rammed | 1,363 sft x 3.458 ft = 1,362.6 x 3.458 x 1.20 compaction | 5,654.8 | cft |  |  |  |  |  |  |  |  | A3, A4, A5 | - |
| 6.3 | Front car porch (full area) Earth filling from NSL to underside of porch khangar (+5½") | 308 sft (20'-0" x 16'-7½" less lobby notch) x 1.958 ft = 308.5 x 1.958 x 1.20 | 724.9 | cft |  |  |  |  |  |  |  |  | A5 | - |
| 6.4 | Rear car porch (full area) Earth filling from NSL to underside of porch khangar (+5½") | 143 sft (16'-9½" x 8'-6") x 1.958 ft = 142.7 x 1.958 x 1.20 | 335.4 | cft |  |  |  |  |  |  |  |  | A5 | - |
| 6.5 | House floor + both car porches Hand-packed stone ballast (khangar) 4", 1½"-2½" crushed stone (capillary break) | 1,814 sft x 4" = 1,813.8 x 0.333 x 1.10 packing | 665.1 | cft |  |  |  |  | 665.1 |  |  |  | B2 | 62,848 |
| 6.6 | House floor + both car porches Sand blinding 1" over khangar | 1,814 sft x 1" = 1,813.8 x 0.0833 x 1.25 (fills top voids) | 188.9 | cft |  |  | 188.9 |  |  |  |  |  | B2 | 8,162 |
| 6.7 | House floor + both car porches Polythene sheet 0.2 mm (800 gauge) with 6" laps, turned up 4" against walls | 1,814 sft + turn-up 576 ft x 4" = 1,813.8 x 1.10 + 576 x 0.33 = 2,185 sft x 0.0171 kg/sft | 2,185.2 | sft |  |  |  |  |  |  |  | Polythene 37.4 kg | B2, B3 | 21,580 |
| 6.8 | House floor + both car porches PCC 1:4:8, 3", sub-floor over polythene | 1,814 sft x 3" = 1,813.8 x 0.25 | 453.4 | cft | 43.8 | 219.0 |  | 438.1 |  |  |  |  | B2 | 136,372 |
| 6.9 | Whole site Earth (mitti) to buy for filling = fill required - surplus excavated earth re-used | fill required 7,362 cft (loose); surplus excavation 2,542 cft bank x 1.25 = 3,177 cft loose = 7,362 - 3,177 | 4,184.8 | cft |  |  |  |  |  |  |  | Earth (mitti) 4,184.8 cft | A4, A5 | 104,620 |
|  | S6 total - net |  |  |  | 43.8 | 219.0 | 188.9 | 438.1 | 665.1 |  |  |  |  | 333,581 |
|  | S6 total - with wastage |  |  |  | 45.1 | 236.6 | 204.1 | 460.0 | 698.3 |  |  |  |  |  |
| S7 Masonry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Ground floor Ground floor 9" walls in 1:6 | plan 193.5 sft x 11.00 ft = gross 2,128 cft less openings 417, lintels 57, bed plates 5.1 | 1,649.6 | cft | 75.2 |  | 564.2 |  |  | 22,270 |  |  | D1, D2 | 519,383 |
| 7.2 | Ground floor Ground floor 4½" walls in 1:4 | plan 38.8 sft x 11.00 ft = gross 427 cft less openings 54, lintels 8 | 364.9 | cft | 23.3 |  | 116.5 |  |  | 4,926 |  |  | D1, D2 | 125,221 |
| 7.3 | First floor First floor 9" walls in 1:6 | plan 174.8 sft x 11.54 ft = gross 2,018 cft less openings 373, lintels 55, bed plates 6.8 | 1,582.6 | cft | 72.2 |  | 541.3 |  |  | 21,365 |  |  | D1, D2 | 498,277 |
| 7.4 | First floor First floor 4½" walls in 1:4 | plan 63.9 sft x 11.54 ft = gross 738 cft less openings 92, lintels 13 | 632.8 | cft | 40.4 |  | 202.0 |  |  | 8,542 |  |  | D1, D2 | 217,143 |
| 7.5 | Mumty (2nd floor) Mumty (2nd floor) 9" walls in 1:6 | plan 43.5 sft x 9.54 ft = gross 415 cft less openings 51, lintels 9, bed plates 1.1 | 353.7 | cft | 16.1 |  | 121.0 |  |  | 4,775 |  |  | D1, D2 | 111,358 |
| 7.6 | Mumty (2nd floor) Mumty (2nd floor) 4½" walls in 1:4 | plan 24.9 sft x 9.54 ft = gross 237 cft less openings 48, lintels 7 | 181.6 | cft | 11.6 |  | 58.0 |  |  | 2,451 |  |  | D1, D2 | 62,315 |
| 7.7 | Terrace (first-floor roof) Terrace parapets on FF roof (4½", one 9" run) 125.6 ft x 3.04 ft high in 1:4 | 115.8 ft (4½") + 9.8 ft (9") x 3.04 ft high (2'-6" above the finished terrace) = (115.8 x 0.375 + 9.8 x 0.75) x 3.04 = 154.4 cft | 154.4 | cft | 9.9 |  | 49.3 |  |  | 2,084 |  |  | D1, D2 | 52,979 |
| 7.8 | Mumty roof Parapet on mumty roof 4½" 94.2 ft x 2.04 ft in 1:4 | 94.2 ft x 4½" x 2.04 ft high (1'-6" above the finished roof) = 94.2 x 0.375 x 2.04 = 72.1 cft | 72.1 | cft | 4.6 |  | 23.0 |  |  | 974 |  |  | D1, D2 | 24,750 |
| 7.9 | Front elevation Elevation features allowance 5% of superstructure brickwork (frames, fins, piers, profile-light reveals, planters) in 1:4 | = 5% x walls + parapets | 249.6 | cft | 15.9 |  | 79.7 |  |  | 3,369 |  |  | D2 | 85,649 |
|  | S7 total - net |  |  |  | 269.2 |  | 1,754.8 |  |  | 70,757 |  |  |  | 1,697,075 |
|  | S7 total - with wastage |  |  |  | 277.3 |  | 1,895.2 |  |  | 74,295 |  |  |  |  |
| S8 RCC (superstructure) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Car-porch front corner RCC column 13½" x 13½" (1:1.5:3), 4 #6, #3 ties @ 6" | 13½" x 13½" x 16.50 ft (footing top -2'-9" to slab soffit +13'-9") = #6: 4 x (16.5 + 2.5 lap/anchorage); ties (16.5 x 2 + 1) x 4'-0" | 20.9 | cft | 4.8 | 8.9 |  | 17.9 |  |  | #3 23.2 #6 51.8 |  | C5, E1 | 30,785 |
| 8.2 | Ground-floor roof Roof slab 5½" - GF roof: outline 1739.1 + porch cantilever 38.7 - stair opening 72.8 (1705.0 sft) | 1,705.0 sft x 5½" thick = 1,705.0 x 5.5/12 = 781.5 cft | 781.5 | cft | 140.2 | 350.5 |  | 701.1 |  |  |  |  | E1 | 330,882 |
| 8.3 | First-floor roof Roof slab 5½" - FF roof: outline 1725.0 - stair opening 74.3 (excl. open shaft & terrace) (1650.7 sft) | 1,650.7 sft x 5½" thick = 1,650.7 x 5.5/12 = 756.6 cft | 756.6 | cft | 135.8 | 339.4 |  | 678.8 |  |  |  |  | E1 | 320,344 |
| 8.4 | Mumty roof Roof slab 5½" - Mumty roof: 16.85x7.75 + 17.63x12.63 (353.2 sft) | 353.2 sft x 5½" thick = 353.2 x 5.5/12 = 161.9 cft | 161.9 | cft | 29.0 | 72.6 |  | 145.2 |  |  |  |  | E1 | 68,544 |
| 8.5 | Ground-floor roof CB-1 concealed beam 12" x 9½" in GF roof - 5 nos, 81.7 ft (4" upstand above slab; 3#6 bottom, 3#4 top, #3@7" rings) | 5 nos, total 81.7 ft; 12" x 9½" deep, 4" upstand above the 5½" slab (S-09) = upstand 81.7 x 1.0 x 4" = 27.2 cft (rest is inside the slab); #6 + #4: 3 + 3 x (81.7 + 3 ft anchorage per beam) x 1.03; #3 rings (81.7 x 12/7 + 5) x 3.21 ft | 27.2 | cft | 4.9 | 12.2 |  | 24.4 |  |  | #3 79.4 #4 90.6 #6 203.6 |  | E1 | 113,333 |
| 8.6 | First-floor roof CB-1 concealed beam 12" x 9½" in FF roof - 1 nos, 15.5 ft (4" upstand above slab; 3#6 bottom, 3#4 top, #3@7" rings) | 1 nos, total 15.5 ft; 12" x 9½" deep, 4" upstand above the 5½" slab (S-09) = upstand 15.5 x 1.0 x 4" = 5.2 cft (rest is inside the slab); #6 + #4: 3 + 3 x (15.5 + 3 ft anchorage per beam) x 1.03; #3 rings (15.5 x 12/7 + 1) x 3.21 ft | 5.2 | cft | 0.9 | 2.3 |  | 4.6 |  |  | #3 15.1 #4 17.3 #6 38.9 |  | E1 | 21,632 |
| 8.7 | Ground-floor roof CB-2 concealed beam 9" x 9½" in GF roof - 5 nos, 48.5 ft (4" upstand above slab; 2#6 top & bottom, #3@6" rings) | 5 nos, total 48.5 ft; 9" x 9½" deep, 4" upstand above the 5½" slab (S-09) = upstand 48.5 x 0.75 x 4" = 12.1 cft (rest is inside the slab); #6: 4 x (48.5 + 3 ft anchorage per beam) x 1.03; #3 rings (48.5 x 12/6 + 5) x 2.71 ft | 12.1 | cft | 2.2 | 5.4 |  | 10.9 |  |  | #3 47.1 #6 178.1 |  | E1 | 66,502 |
| 8.8 | First-floor roof CB-2 concealed beam 9" x 9½" in FF roof - 7 nos, 102.0 ft (4" upstand above slab; 2#6 top & bottom, #3@6" rings) | 7 nos, total 102.0 ft; 9" x 9½" deep, 4" upstand above the 5½" slab (S-09) = upstand 102.0 x 0.75 x 4" = 25.5 cft (rest is inside the slab); #6: 4 x (102.0 + 3 ft anchorage per beam) x 1.03; #3 rings (102.0 x 12/6 + 7) x 2.71 ft | 25.5 | cft | 4.6 | 11.4 |  | 22.9 |  |  | #3 97.5 #6 345.3 |  | E1 | 131,467 |
| 8.9 | Mumty roof CB-2 concealed beam 9" x 9½" in SF roof - 2 nos, 30.3 ft (4" upstand above slab; 2#6 top & bottom, #3@6" rings) | 2 nos, total 30.3 ft; 9" x 9½" deep, 4" upstand above the 5½" slab (S-09) = upstand 30.3 x 0.75 x 4" = 7.6 cft (rest is inside the slab); #6: 4 x (30.3 + 3 ft anchorage per beam) x 1.03; #3 rings (30.3 x 12/6 + 2) x 2.71 ft | 7.6 | cft | 1.4 | 3.4 |  | 6.8 |  |  | #3 28.9 #6 101.8 |  | E1 | 38,810 |
| 8.10 | Ground-floor roof Door/window/vent lintels GF - 23 nos (DL-1, DL-2, WL-1, WL-2, WL-3, VL-1 per S-05) - includes the 8'-0" DL-1 over the 6'-6" dining opening | DL-1 7 nos / 39.6 ft; DL-2 7 nos / 28.6 ft; WL&DL-1 1 nos / 9.4 ft; WL-1 4 nos / 29.8 ft; WL-2 1 nos / 10.3 ft; VL-1 3 nos / 12.0 ft = sum of width x 9" x length = 64.9 cft; main bars n x (L + 6"), rings (L x 12/spacing + 1) x ring length (S-05) | 64.9 | cft | 11.7 | 29.1 |  | 58.3 |  |  | #2 7.7 #3 95.3 #4 161.4 |  | E1, E2 | 99,602 |
| 8.11 | First-floor roof Door/window/vent lintels FF - 26 nos (DL-1, DL-2, WL-1, WL-2, WL-3, VL-1 per S-05) | DL-1 7 nos / 34.0 ft; DL-2 6 nos / 25.9 ft; WL-1 5 nos / 41.8 ft; WL-2 1 nos / 10.3 ft; WL-3 4 nos / 20.8 ft; VL-1 3 nos / 12.0 ft = sum of width x 9" x length = 68.3 cft; main bars n x (L + 6"), rings (L x 12/spacing + 1) x ring length (S-05) | 68.3 | cft | 12.3 | 30.6 |  | 61.3 |  |  | #2 12.4 #3 92.9 #4 181.4 |  | E1 | 107,140 |
| 8.12 | Mumty roof Door/window/vent lintels SF - 9 nos (DL-1, DL-2, WL-1, WL-2, WL-3, VL-1 per S-05) | DL-1 1 nos / 5.0 ft; DL-2 3 nos / 13.0 ft; WL-1 1 nos / 6.8 ft; WL-3 3 nos / 12.6 ft; VL-1 1 nos / 4.0 ft = sum of width x 9" x length = 16.1 cft; main bars n x (L + 6"), rings (L x 12/spacing + 1) x ring length (S-05) | 16.1 | cft | 2.9 | 7.2 |  | 14.4 |  |  | #2 6.8 #3 16.0 #4 50.2 |  | E1 | 26,774 |
| 8.13 | Ground-floor roof Bed plates 9" x 3'-0" x 3" (1:2:4, 3#4) under beam ends, GF roof - 9 nos | 9 nos x 9" x 3'-0" x 3" = 9 x 0.75 x 3.0 x 0.25 = 5.06 cft; #4: 9 x 3 bars x 2'-9" | 5.1 | cft | 0.9 | 2.3 |  | 4.5 |  |  | #4 22.5 |  | E1 | 8,274 |
| 8.14 | First-floor roof Bed plates 9" x 3'-0" x 3" (1:2:4, 3#4) under beam ends, FF roof - 12 nos | 12 nos x 9" x 3'-0" x 3" = 12 x 0.75 x 3.0 x 0.25 = 6.75 cft; #4: 12 x 3 bars x 2'-9" | 6.8 | cft | 1.2 | 3.0 |  | 6.1 |  |  | #4 30.0 |  | E1 | 11,032 |
| 8.15 | Mumty roof Bed plates 9" x 3'-0" x 3" (1:2:4, 3#4) under beam ends, SF roof - 2 nos | 2 nos x 9" x 3'-0" x 3" = 2 x 0.75 x 3.0 x 0.25 = 1.12 cft; #4: 2 x 3 bars x 2'-9" | 1.1 | cft | 0.2 | 0.5 |  | 1.0 |  |  | #4 5.0 |  | E1 | 1,839 |
| 8.16 | Ground-floor roof Zig-zag stair GF to FF: 6" waist (flights 11.65 + 9.59 ft), 6" landing 8'-3"x3'-6", 21 concrete steps; riser 6.57", tread 10½", width 3'-6" | flights 11.65 + 9.59 ft (on slope) x 3'-6" x 6" waist; landing 8'-3" x 3'-6" x 6"; 21 steps 10½" x 6.57" = waist 37.2 + landing 14.4 + steps 17.6 = 69.2 cft; #4 @ 5" main, #3 @ 9"/12" distribution (S-03) | 69.2 | cft | 12.4 | 31.1 |  | 62.1 |  |  | #3 48.4 #4 235.1 |  | E1 | 106,555 |
| 8.17 | First-floor roof Zig-zag stair FF to 2nd floor: 6" waist (flights 11.65 + 9.59 ft), 6" landing 8'-3"x3'-6", 21 concrete steps; riser 6.57", tread 10½", width 3'-6" | flights 11.65 + 9.59 ft (on slope) x 3'-6" x 6" waist; landing 8'-3" x 3'-6" x 6"; 21 steps 10½" x 6.57" = waist 37.2 + landing 14.4 + steps 17.6 = 69.2 cft; #4 @ 5" main, #3 @ 9"/12" distribution (S-03) | 69.2 | cft | 12.4 | 31.1 |  | 62.1 |  |  | #3 48.4 #4 235.1 |  | E1 | 106,555 |
| 8.18 | Stair Stair base footing + stem (8" slab, #4@6" both ways; stem #4@5") | base 4'-6" x 3'-0" x 8" + stem 3'-6" x 2'-0" x 6" = 4.5 x 3.0 x 0.667 + 3.5 x 2.0 x 0.5 = 12.5 cft; #4: 180.75 ft | 12.5 | cft | 2.2 | 5.6 |  | 11.2 |  |  | #4 54.8 |  | E1 | 20,218 |
| 8.19 | Stair P.C.C 1:4:8 under stair footing | 5'-0" x 3'-6" x 3" = 5.0 x 3.5 x 0.25 = 4.38 cft | 4.4 | cft | 0.4 | 2.1 |  | 4.2 |  |  |  |  | E1 | 1,316 |
| 8.20 | Ground-floor roof R.C.C sunshade (chajja) GF: 12.5 ft long x 1'-3" projection, 4"-3" thick, #3@6" main | 12.5 ft x 1'-3" x 4" (tapering to 3") = 12.5 x 1.25 x 0.333 = 5.21 cft; #3 @ 6" x 2'-9" + 3 distribution bars | 5.2 | cft | 0.9 | 2.3 |  | 4.7 |  |  | #3 18.6 |  | E1 | 7,271 |
| 8.21 | First-floor roof R.C.C sunshade (chajja) FF: 8.0 ft long x 1'-3" projection, 4"-3" thick, #3@6" main | 8.0 ft x 1'-3" x 4" (tapering to 3") = 8.0 x 1.25 x 0.333 = 3.33 cft; #3 @ 6" x 2'-9" + 3 distribution bars | 3.3 | cft | 0.6 | 1.5 |  | 3.0 |  |  | #3 12.1 |  | E1 | 4,699 |
| 8.22 | Mumty roof R.C.C sunshade (chajja) SF: 6.0 ft long x 1'-3" projection, 4"-3" thick, #3@6" main | 6.0 ft x 1'-3" x 4" (tapering to 3") = 6.0 x 1.25 x 0.333 = 2.50 cft; #3 @ 6" x 2'-9" + 3 distribution bars | 2.5 | cft | 0.4 | 1.1 |  | 2.2 |  |  | #3 9.2 |  | E1 | 3,556 |
| 8.23 | Front elevation Provisional: front elevation canopy / RCC fins & projections | provisional allowance (front canopy, fins and projections on the 3D views) = 20 cft concrete + 150 ft of #3 | 20.0 | cft | 3.6 | 9.0 |  | 17.9 |  |  | #3 25.6 |  | E1 | 15,439 |
| 8.24 | All parapets Parapet coping 3" x 9" (1:2:4) with 2#3 | terrace 125.6 ft + mumty 94.2 ft = 219.8 ft x 9" x 3" = 219.8 x 0.75 x 0.25 = 41.2 cft; #3: 2 x 219.8 x 1.05 | 41.2 | cft | 7.4 | 18.5 |  | 37.0 |  |  | #3 78.7 |  | E1 | 38,900 |
| 8.25 | Ground-floor roof Slab steel GF roof (44 bar sets incl. cranked, extra top, trimmer 2#6/2#4 & chairs) - 0.84 kg/sft | 1,705.0 sft of slab; every bar set on S-06 (bar size, spacing, width covered, bar length) = bars = width x 12 / spacing + 1; length + 0.6 ft (cranked) or + 0.3 ft (extra top); trimmers; chairs 1.5 ft per 12 sft = 1,435.4 kg | 1,705.0 | sft |  |  |  |  |  |  | #3 1,168.2 #4 224.4 #6 42.8 |  | E1, E4 | 391,116 |
| 8.26 | First-floor roof Slab steel FF roof (40 bar sets incl. cranked, extra top, trimmer 2#6/2#4 & chairs) - 0.88 kg/sft | 1,650.7 sft of slab; every bar set on S-07 (bar size, spacing, width covered, bar length) = bars = width x 12 / spacing + 1; length + 0.6 ft (cranked) or + 0.3 ft (extra top); trimmers; chairs 1.5 ft per 12 sft = 1,446.9 kg | 1,650.7 | sft |  |  |  |  |  |  | #3 1,078.9 #4 355.7 #6 12.3 |  | E1, E4 | 394,250 |
| 8.27 | Mumty roof Slab steel SF roof (11 bar sets incl. cranked, extra top, trimmer 2#6/2#4 & chairs) - 0.73 kg/sft | 353.2 sft of slab; every bar set on S-08 (bar size, spacing, width covered, bar length) = bars = width x 12 / spacing + 1; length + 0.6 ft (cranked) or + 0.3 ft (extra top); trimmers; chairs 1.5 ft per 12 sft = 259.0 kg | 353.2 | sft |  |  |  |  |  |  | #3 259.0 |  | E1, E4 | 70,564 |
| 8.28 | 3 first-floor baths + mumty bath Sunken slab 6": extra concrete (1:2:4) and steel at the drop (U-bars #3 @ 9" + 2 #3 along each edge) | baths 191 sft, perimeter 111.2 ft; drop 6" x slab 5½" = concrete 111.2 x 0.5 x 0.458; #3: 111.2 x 12/9 x 2.5 + 2 x 111.2 | 25.5 | cft | 4.6 | 11.4 |  | 22.9 |  |  | #3 101.2 |  | E3 | 38,368 |
| 8.29 | 3 first-floor baths + mumty bath Waterproof coating in sunk areas: polymer-modified cementitious, 2 coats (~1.5 kg/m2 per coat) | floor 191 sft + sides 111.2 ft x 6" = 247 sft = 22.9 m2 x 3.0 kg | 246.8 | sft |  |  |  |  |  |  |  | WP coating 68.8 kg | E3 | 32,495 |
| 8.30 | 3 first-floor baths + mumty bath Lightweight filling in sunk areas: brick-bat coba (broken bricks in 1:6 mortar), 9" deep to underside of floor finish | 191 sft x 9" (6" drop + 3" floor base) = 143 cft: bats 0.70 cft/cft (9.5 bricks), 1:6 mortar 0.30 cft/cft | 143.3 | cft | 6.5 |  | 49.0 |  |  | 1,355 |  |  | E3 | 35,378 |
| 8.31 | First floor + mumty floor Floor sub-base: 3" PCC 1:4:8 over first-floor and mumty floors (levels the 4" beam upstands, covers conduits), excluding sunken baths | 1,554 sft x 3" = (1,705 - 245 walls - 165 baths) + (353 - 68 walls - 26 bath) = 1,554 sft | 388.5 | cft | 37.5 | 187.7 |  | 375.4 |  |  |  |  | D2 (Rev 0 assumption kept), E3 | 116,845 |
|  | S8 total - net |  |  |  | 441.9 | 1,180.4 | 49.0 | 2,360.9 |  | 1,355 | #2 26.9 #3 3,343.7 #4 1,663.4 #6 974.6 |  |  | 2,760,483 |
|  | S8 total - with wastage |  |  |  | 455.2 | 1,274.9 | 52.9 | 2,478.9 |  | 1,422 | #2 28.0 #3 3,477.5 #4 1,730.0 #6 1,013.6 |  |  |  |
| S9 Roof waterproofing and tiles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1 | Terrace, mumty roof, first-floor front balcony Slope screed 1:4 cement-sand at 1:100 to the drawn rain-water outlets, min ¾" at outlets | terrace 1,302 sft (avg 2.9", max 5.4"); mumty 319 sft (avg 2.7", max 4.9"); balcony 38 sft (avg 1½") = grid calculation of screed thickness by outlet catchment = 393.3 cft | 393.3 | cft | 83.7 | 418.5 |  |  |  |  |  |  | F3 | 163,863 |
| 9.2 | Terrace, mumty roof, balcony Bitumen primer, ~0.3 L/m2 | 1,659 sft roof + upturns = 2,256 sft = 209.6 m2 x 0.3 L | 62.9 | L |  |  |  |  |  |  |  | Primer 62.9 L | F3 | 59,428 |
| 9.3 | Terrace, mumty roof, balcony 4 mm torch-applied modified-bitumen membrane, 4" laps, turned 12" up parapets and mumty walls into a groove, dressed round solar plinths and tank platform | roof 1,659 sft; upturns 276 ft x 12"; 34 plinths; tank platform = 1,659 x 1.10 + 276 x 1.10 + 34 x 5 ft x 8" + platform = 2,256 sft = 209.6 m2 | 209.6 | m2 |  |  |  |  |  |  |  | Membrane 209.6 m2 | F3, J3 | 154,074 |
| 9.4 | Terrace, mumty roof, balcony LPG for torching membrane (~0.4 kg per m2) | = 209.6 m2 x 0.4 kg | 83.8 | kg |  |  |  |  |  |  |  | LPG 83.8 kg | F3 | 25,155 |
| 9.5 | All parapet and mumty-wall junctions Cement fillet (gola) 4" x 4" in 1:3 at every parapet / wall junction | 276 ft = 276 x 0.0556 cft/ft | 15.3 | cft | 4.1 |  | 15.3 |  |  |  |  |  | F3 | 7,206 |
| 9.6 | Terrace, mumty roof, balcony Bhatta tiles 9" x 4½" x 1½" laid in 1:4 mortar (¾" bed), joints filled | 1,581 sft (less plinths and tank platform) = tiles 1,581 x 3.15/sft; mortar bed 1,581 x 0.0625 + joints = 121.3 cft | 1,580.7 | sft | 25.8 |  | 129.0 |  |  |  |  | Bhatta tiles 4,979.2 nos | F3 | 151,605 |
|  | S9 total - net |  |  |  | 113.6 | 418.5 | 144.3 |  |  |  |  |  |  | 561,331 |
|  | S9 total - with wastage |  |  |  | 117.0 | 452.0 | 155.9 |  |  |  |  |  |  |  |
| S10 Chicken mesh and plaster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | All brick / RCC junctions, both faces Galvanised chicken mesh strips 12" wide over every brick/RCC junction (slab edges, lintels, plinth beam, coping, chajjas, column, stairs) | slab edges 2 x 889 ft; lintels 2 x 316 ft; plinth beam 174; coping 2 x 220; chajjas 26.5; column 22; stairs 59 = 3,131 rft of 12" strip (3 strips per 3-ft-wide roll = 300 rft/roll) | 3,131.5 | rft |  |  |  |  |  |  |  | Chicken mesh 11.5 roll Nails 10.4 kg | D3 | 71,006 |
| 10.2 | All concealed conduit and pipe chases Galvanised chicken mesh strips 9" wide over every concealed chase | electrical chases 909 rft; plumbing / gas / AC-drain chases 896 rft = 1,805 rft of 9" strip (4 strips per roll = 400 rft/roll) | 1,804.8 | rft |  |  |  |  |  |  |  | Chicken mesh 5.0 roll Nails 4.5 kg | D3 | 30,693 |
| 10.3 | Ground floor Internal plaster ½" 1:4 on all walls | walls 361.6 ft x 2 faces x 11.00 ft = 7,955 sft = less outer faces 1,467 and property-line faces 443 = 6,044; less openings 982; plus 0.6 x reveals 285 = 5,234 sft | 5,233.7 | sft | 58.0 |  | 290.0 |  |  |  |  |  | F1, F2 | 105,734 |
| 10.4 | First floor Internal plaster ½" 1:4 on all walls | walls 403.6 ft x 2 faces x 11.54 ft = 9,316 sft = less outer faces 1,639 and property-line faces 416 = 7,262; less openings 1,038; plus 0.6 x reveals 282 = 6,393 sft | 6,393.1 | sft | 70.9 |  | 354.3 |  |  |  |  |  | F1, F2 | 129,159 |
| 10.5 | Mumty Internal plaster ½" 1:4 on all walls | walls 124.2 ft x 2 faces x 9.54 ft = 2,371 sft = less outer faces 779 and property-line faces 121 = 1,472; less openings 265; plus 0.6 x reveals 76 = 1,252 sft | 1,252.1 | sft | 13.9 |  | 69.4 |  |  |  |  |  | F1, F2 | 25,295 |
| 10.6 | Ground floor External plaster ¾" 1:4 on outer faces incl. exposed plinth | outer walls 133.4 ft x 11.00 ft = 1,467 sft + exposed plinth 133.4 ft x 2'-0" (outside levels now +1'-0" to +1'-3"; was 3'-3") = less openings 420; plus 0.4 x reveals 285; plus plinth 266.8 = 1,428 sft | 1,428.5 | sft | 23.7 |  | 118.7 |  |  |  |  |  | F1, F2 | 43,288 |
| 10.7 | First floor External plaster ¾" 1:4 on outer faces | outer walls 142.0 ft x 11.54 ft = 1,639 sft = less openings 448; plus 0.4 x reveals 282 = 1,304 sft | 1,304.0 | sft | 21.7 |  | 108.4 |  |  |  |  |  | F1, F2 | 39,516 |
| 10.8 | Mumty External plaster ¾" 1:4 on outer faces | outer walls 81.6 ft x 9.54 ft = 779 sft = less openings 129; plus 0.4 x reveals 76 = 680 sft | 679.8 | sft | 11.3 |  | 56.5 |  |  |  |  |  | F1, F2 | 20,599 |
| 10.9 | Terrace and mumty parapets Parapet plaster ¾" 1:4, both faces and top | terrace 125.6 ft x (2 x 3.04 + 0.75) + mumty 94.2 ft x (2 x 2.04 + 0.75) = 858 + 455 = 1,314 sft | 1,313.6 | sft | 21.8 |  | 109.2 |  |  |  |  |  | F1 | 39,806 |
| 10.10 | Car-porch slab, entrance canopy, chajjas, stair over rear porch External soffit plaster ⅜" 1:3 (separate line - can be deleted) | porch 102 + canopy 39 + entrance 33 + chajjas 33 + stair 34 sft = 241 sft | 240.8 | sft | 2.5 |  | 9.4 |  |  |  |  |  | F1 | 4,426 |
|  | S10 total - net |  |  |  | 223.8 |  | 1,115.9 |  |  |  |  |  |  | 509,522 |
|  | S10 total - with wastage |  |  |  | 230.5 |  | 1,205.2 |  |  |  |  |  |  |  |
| S11 Sewerage and drainage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 | Manholes (9 nos, sheet 17) Manholes / inspection chambers - 9 nos 2'x2' internal, avg 3'-6" deep: excavation | 9 nos x 4'-6" x 4'-6" x 4'-3" deep (2'-0" inside + 9" walls + 6" working space each side) = 9 x 4.5 x 4.5 x 4.25 = 774.6 cft | 774.6 | cft |  |  |  |  |  |  |  |  | H2 | - |
| 11.2 | Manholes (9 nos, sheet 17) Manholes: P.C.C 1:4:8 4" base | 9 nos x 3'-6" x 3'-6" x 4" = 9 x 3.5 x 3.5 x 0.333 = 36.7 cft | 36.7 | cft | 3.5 | 17.7 |  | 35.5 |  |  |  |  | H2 | 11,041 |
| 11.3 | Manholes (9 nos, sheet 17) Manholes: 9" brick walls 1:4 | 9 nos, 2'-0" x 2'-0" inside, 9" walls, average 3'-6" deep; centre-line 4 x 2'-9" = 11 ft = 9 x 11 x 0.75 x 3.5 = 259.9 cft (Rev 0 took 9 x 11 x 3.5 = 346.5, missing the 9" thickness - corrected) | 259.9 | cft | 16.6 |  | 83.0 |  |  | 3,508 |  |  | H2 | 89,181 |
| 11.4 | Manholes (9 nos, sheet 17) Manholes: benching/channels 1:2:4 | 9 nos x 1.5 cft (benching and half-round channels) = 9 x 1.5 = 13.5 cft | 13.5 | cft | 2.4 | 6.1 |  | 12.1 |  |  |  |  | H2 | 5,716 |
| 11.5 | Manholes (9 nos, sheet 17) Manholes: inside plaster ½" 1:3 | 9 nos x (4 walls x 2'-0" x 3'-6" + benching 4 sft) = 9 x 32 = 288.0 sft | 288.0 | sft | 4.0 |  | 15.0 |  |  |  |  |  | H2 | 7,058 |
| 11.6 | Manholes Back-filling around manholes with excavated earth | = 775 - 9 x 3.5 x 3.5 x 3.0 | 443.8 | cft |  |  |  |  |  |  |  |  | H2 | - |
| 11.7 | 7 manholes in passages and lawn RCC cover slabs 3" (1:2:4), #3 @ 6" both ways | 7 x 2'-9" x 2'-9" x 3" = 7 x 2.75 x 2.75 x 0.25; #3: 7 x 35 ft | 13.2 | cft | 2.4 | 5.9 |  | 11.9 |  |  | #3 41.8 |  | H2 | 16,989 |
| 11.8 | Manholes in front and rear car porches Heavy-duty cast-iron manhole cover with frame, 20" x 20" | 2 nos = 2 | 2 | nos |  |  |  |  |  |  |  | CI cover 2 nos | H2 | 30,000 |
| 11.9 | House, passages, roofs Sewerage (UPVC) 5": Main sewer: rear manholes -> side passage -> septic tank -> road sewer; car-porch MH -> septic tank | = measured on sewerage drawing (GF 135.9 ft) + 20 ft street connection | 155.9 | ft |  |  |  |  |  |  |  | UPVC 5" 155.9 ft | H2 | 70,064 |
| 11.10 | House, passages, roofs Sewerage (UPVC) 4": WC connections GF/FF/2F (horizontal 47.8 ft) + soil stacks 3x13 ft (FF), 1x25 ft (2F) + 4 vent pipes x 4 ft | = measured + vertical stacks | 127.8 | ft |  |  |  |  |  |  |  | UPVC 4" 127.8 ft | H2 | 41,039 |
| 11.11 | House, passages, roofs Sewerage (UPVC) 3": Floor traps, basins, showers, sinks (horizontal 116.1 ft) + waste stacks 4x13 ft + 2x25 ft | = measured + vertical stacks | 218.1 | ft |  |  |  |  |  |  |  | UPVC 3" 218.1 ft | H2 | 46,670 |
| 11.12 | House, passages, roofs Rain water (UPVC) 3": 4 rain-water down pipes in 9"x6" ducts (38+29+29+16 ft) | = roof sumps on roof plans | 112.0 | ft |  |  |  |  |  |  |  | RWP 3" 112.0 ft | H2 | 23,968 |
| 11.13 | Sewer / rain-water system UPVC fittings: 5" (bends 8, Y-tees 6, sockets 9); 4" (WC connectors 8, bends 20, Y-tees 10, sockets 8, vent cowls 4, clean-outs 4); 3" (floor traps 14, gully traps 3, bends 40, Y-tees 20, sockets 15, roof outlets 4, RWP shoes 4) | = counts from sanitary and sewerage sheets | 177 | nos |  |  |  |  |  |  |  | Fittings 5" 23 nos Fittings 4" 54 nos Fittings 3" 100 nos | H2 | 92,250 |
| 11.14 | 10 A.C points to nearest washroom floor trap A.C condensate drains ¾" (20 mm) PVC, concealed; stainless-steel mesh cap on each outlet | 10 x ~20 ft = 200 ft; 10 caps | 200.0 | ft |  |  |  |  |  |  |  | PVC ¾" drain 200.0 ft SS mesh cap 10 nos | H3, J1 | 10,990 |
|  | S11 total - net |  |  |  | 28.9 | 29.7 | 97.9 | 59.5 |  | 3,508 | #3 41.8 |  |  | 444,965 |
|  | S11 total - with wastage |  |  |  | 29.8 | 32.1 | 105.7 | 62.4 |  | 3,684 | #3 43.5 |  |  |  |
| S12 Septic tank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 | Front lawn (sheet 17 position) Excavation for septic tank, 1'-0" working space each side | pit 9.33 x 7.92 x 3'-6" below NSL = (9.33 x 7.92) x 3.5 | 258.6 | cft |  |  |  |  |  |  |  |  | H1 | - |
| 12.2 | Septic tank PCC 1:4:8, 3", under base | 7.33 x 5.92 x 3" | 10.8 | cft | 1.0 | 5.2 |  | 10.5 |  |  |  |  | H1 | 3,262 |
| 12.3 | Septic tank RCC base 6" (1:2:4), #3 @ 6" both ways | 6.83 x 5.42 x 6" = #3: (%d + %d) bars x length | 18.5 | cft | 3.3 | 8.3 |  | 16.6 |  |  | #3 26.7 |  | H1 | 15,109 |
| 12.4 | Septic tank 9" brick walls in 1:4 (5'-0" high) + two 4½" baffle walls (4'-6" high) forming THREE chambers | internal 4'-10" x 3'-5", depth 5'-0" (4'-0" liquid + 1'-0" freeboard) = walls 2 x (5.58 + 4.17) x 0.75 x 5.0 + baffles 2 x 3.42 x 0.375 x 4.5 | 84.7 | cft | 5.4 |  | 27.0 |  |  | 1,143 |  |  | H1 | 29,052 |
| 12.5 | Septic tank RCC top slab 5" (1:2:4) with two 1'-9" x 1'-9" openings, #3 @ 6" both ways + trimmers | 6.33 x 4.92 x 5" | 10.4 | cft | 1.9 | 4.7 |  | 9.4 |  |  | #3 25.6 |  | H1 | 11,384 |
| 12.6 | Septic tank Two RCC covers 2'-0" x 2'-0" x 3" (1:2:4) with lifting hooks | 2 nos | 2.0 | cft | 0.4 | 0.9 |  | 1.8 |  |  | #3 6.8 |  | H1 | 2,706 |
| 12.7 | Septic tank ¾" 1:3 waterproof plaster (with waterproofing compound) inside and outside | inside walls + floor + baffles 161 sft; outside 112 sft | 273.0 | sft | 5.7 |  | 21.3 |  |  |  |  | WP compound 5.7 kg | H1 | 12,021 |
| 12.8 | Septic tank T-pipe inlet and outlet (5" UPVC tees with 2 ft pipe each) and 3" vent pipe 12 ft with cowl | 2 tees 5" + 2 x 2 ft of 5" pipe; vent 3" x 12 ft + cowl = counted | 1 | set |  |  |  |  |  |  |  | Fittings 5" 2 nos UPVC 5" 4.0 ft UPVC 3" 12.0 ft Vent cowl 1 nos | H1 | 6,866 |
| 12.9 | Septic tank Back-filling around tank with excavated earth | = 259 - 115 (tank below NSL) | 143.6 | cft |  |  |  |  |  |  |  |  | H1 | - |
|  | S12 total - net |  |  |  | 17.7 | 19.1 | 48.3 | 38.2 |  | 1,143 | #3 59.1 |  |  | 80,400 |
|  | S12 total - with wastage |  |  |  | 18.2 | 20.6 | 52.2 | 40.1 |  | 1,200 | #3 61.5 |  |  |  |
| S13 Water tanks, water and gas pipes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 | Rear car porch (back of house) Excavation for underground water tank, 1'-0" working space each side | pit 8.83 x 7.42 x 3'-4" below NSL | 218.4 | cft |  |  |  |  |  |  |  |  | I1 | - |
| 13.2 | Underground water tank PCC 1:4:8, 3", under base | 6.83 x 5.42 x 3" | 9.3 | cft | 0.9 | 4.5 |  | 8.9 |  |  |  |  | I2 | 2,783 |
| 13.3 | Underground water tank RCC base 6" (1:1.5:3) with integral waterproofing admixture, #3 @ 6" both ways | 6.33 x 4.92 x 6" | 15.6 | cft | 3.6 | 6.7 |  | 13.3 |  |  | #3 26.8 | WP compound 3.6 kg | I2 | 16,273 |
| 13.4 | Underground water tank RCC walls 6" (1:1.5:3) with integral waterproofing admixture, #3 @ 6" both ways (one central mesh) | centre-line 18.5 ft x 6" x 5.5 ft = #3: verticals 38 x 7.0 + horizontals 12 x 20.5 | 50.9 | cft | 11.6 | 21.8 |  | 43.6 |  |  | #3 87.3 | WP compound 11.6 kg | I2 | 53,089 |
| 13.5 | Underground water tank RCC top slab 5" (1:1.5:3) with one 1'-9" x 1'-9" opening; top 7" above porch level | 5.83 x 4.42 x 5" | 9.5 | cft | 2.2 | 4.1 |  | 8.1 |  |  | #3 20.5 | WP compound 2.2 kg | I2 | 11,023 |
| 13.6 | Underground water tank PVC water-stop 6" (150 mm) at base-to-wall joint | 18.5 ft + laps | 20.0 | ft |  |  |  |  |  |  |  | Water-stop 20.0 ft | I2 | 13,650 |
| 13.7 | Underground water tank Food-safe cementitious waterproof coating inside (walls, floor, soffit), 2 coats ~1.5 kg/m2 each | 124 sft = 11.5 m2 x 3.0 kg | 123.8 | sft |  |  |  |  |  |  |  | Food-safe coating 34.5 kg | I2 | 19,923 |
| 13.8 | Underground water tank Hot bitumen coat on outside of walls below ground (1 coat, 0.2 kg/sft) | 111 sft | 110.7 | sft |  |  |  |  |  |  |  | Bitumen 22.1 kg | I2 | 6,509 |
| 13.9 | Underground water tank Lockable steel / CI manhole cover with frame, 2'-0" x 2'-0" | 1 no | 1 | nos |  |  |  |  |  |  |  | Lockable cover 1 nos | I2 | 12,000 |
| 13.10 | Underground water tank Back-filling around tank with excavated earth | = 218 - 91 | 127.0 | cft |  |  |  |  |  |  |  |  | I1 | - |
| 13.11 | Mumty roof, rear-right corner (sheet 24) Plastic roof tank platform: RCC slab 5'-0" x 5'-0" x 4" (1:2:4), #3 @ 6" both ways, cast before membrane | 5' x 5' x 4" | 8.3 | cft | 1.5 | 3.7 |  | 7.5 |  |  | #3 20.5 |  | I4 | 9,105 |
| 13.12 | Mumty roof Platform supports: two 9" brick walls 1'-6" high in 1:4, placed directly over the mumty walls below | 2 x 5'-0" x 9" x 1'-6" | 11.2 | cft | 0.7 |  | 3.6 |  |  | 152 |  |  | I4 | 3,861 |
| 13.13 | Mumty roof Plaster ¾" 1:4 on platform walls and slab edges | 2 walls x 2 faces x 5'-0" x 1'-6" + slab edges 2 x 5'-0" x 4" = 2 x 2 x 5.0 x 1.5 + 2 x 5.0 x 0.33 = 33.3 sft | 33.3 | sft | 0.6 |  | 2.8 |  |  |  |  |  | I4 | 1,009 |
| 13.14 | House Cold water PPR 1¼" PN-16: tank down-take | = measured on sheets 21-24 + risers (Rev 0), updated for pump at back | 38.0 | ft |  |  |  |  |  |  |  | PPR 1¼" 38.0 ft | I3, I5 | 10,978 |
| 13.15 | House Cold water PPR 1" PN-16: pump rising main from UGT at back to roof tank (53 ft) + floor mains (40 ft) | = measured on sheets 21-24 + risers (Rev 0), updated for pump at back | 93.0 | ft |  |  |  |  |  |  |  | PPR 1" 93.0 ft | I3, I5 | 17,414 |
| 13.16 | House Cold water PPR ¾" PN-16: branch lines (measured 370 ft) + society supply to UGT (60 ft) | = measured on sheets 21-24 + risers (Rev 0), updated for pump at back | 430.0 | ft |  |  |  |  |  |  |  | PPR ¾" 430.0 ft | I3, I5 | 50,611 |
| 13.17 | House Cold water PPR ½" PN-16: fixture drops 35 x 4 ft | = measured on sheets 21-24 + risers (Rev 0), updated for pump at back | 140.0 | ft |  |  |  |  |  |  |  | PPR ½" 140.0 ft | I3, I5 | 10,486 |
| 13.18 | House Hot water PPR ¾" PN-20: hot lines (measured) + geyser risers 36 ft | = measured on sheets 21-24 + risers (Rev 0), updated for pump at back | 360.0 | ft |  |  |  |  |  |  |  | PPR ¾" hot 360.0 ft | I3, I5 | 52,002 |
| 13.19 | House Hot water PPR ½" PN-20: fixture drops 20 x 4 ft | = measured on sheets 21-24 + risers (Rev 0), updated for pump at back | 80.0 | ft |  |  |  |  |  |  |  | PPR ½" hot 80.0 ft | I3, I5 | 7,276 |
| 13.20 | House Gas: SNGPL-approved medium-class GI ½"-¾" to kitchens, geyser, heaters + riser from meter | = measured on sheets 21-24 + risers (Rev 0), updated for pump at back | 189.8 | ft |  |  |  |  |  |  |  | GI gas pipe 189.8 ft | I3, I5 | 52,802 |
| 13.21 | House PPR fittings (elbows ~120, tees ~60, sockets ~80, adaptors ~55, unions ~20), 24 ball/gate valves, ¾" float valve for UGT; GI fittings ~36 | = approximate counts | 1 | set |  |  |  |  |  |  |  | PPR fittings 335 nos Valves 24 nos Float valve 1 nos GI fittings 36 nos | I3, I5 | 73,300 |
|  | S13 total - net |  |  |  | 21.0 | 40.7 | 6.4 | 81.4 |  | 152 | #3 155.1 |  |  | 424,095 |
|  | S13 total - with wastage |  |  |  | 21.6 | 44.0 | 6.9 | 85.5 |  | 159 | #3 161.3 |  |  |  |
| S14 Electrical civil provisions and solar plinths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 | House ¾" PVC conduit: light/fan points 115 x 12 ft | = allowance per point / measured run | 1,380.0 | ft |  |  |  |  |  |  |  | Conduit ¾" 1,380.0 ft | J8 | 36,915 |
| 14.2 | House ¾" PVC conduit: socket points 79 x 8 ft | = allowance per point / measured run | 632.0 | ft |  |  |  |  |  |  |  | Conduit ¾" 632.0 ft | J8 | 16,906 |
| 14.3 | House ¾" PVC conduit: call bells at 2 gates x 40 ft | = allowance per point / measured run | 80.0 | ft |  |  |  |  |  |  |  | Conduit ¾" 80.0 ft | J2, J8 | 2,140 |
| 14.4 | House ¾" PVC conduit: Cat6 data to 5 Wi-Fi access points (2 GF, 2 FF, 1 mumty) to router point | = allowance per point / measured run | 220.0 | ft |  |  |  |  |  |  |  | Conduit ¾" 220.0 ft | J2, J8 | 5,885 |
| 14.5 | House ¾" PVC conduit: Cat6 to 7 CCTV cameras to NVR point (gates, porches, back gali, passages) | = allowance per point / measured run | 350.0 | ft |  |  |  |  |  |  |  | Conduit ¾" 350.0 ft | J2, J8 | 9,362 |
| 14.6 | House 1" PVC conduit: DB to 39 switch-boards x 25 ft | = allowance per point / measured run | 975.0 | ft |  |  |  |  |  |  |  | Conduit 1" 975.0 ft | J8 | 39,644 |
| 14.7 | House 1" PVC conduit: own run to each of 10 A.C points (5 x 45 + 4 x 30 + 1 x 40 ft) | = allowance per point / measured run | 385.0 | ft |  |  |  |  |  |  |  | Conduit 1" 385.0 ft | J1, J5, J7, J8 | 15,654 |
| 14.8 | House 1" PVC conduit: earth conductors from 3 earth pits (house 25, lightning/surge 45, solar 55 ft) | = allowance per point / measured run | 125.0 | ft |  |  |  |  |  |  |  | Conduit 1" 125.0 ft | J1, J5, J7, J8 | 5,082 |
| 14.9 | House 1" PVC conduit: water pump at UGT from backup DB | = allowance per point / measured run | 60.0 | ft |  |  |  |  |  |  |  | Conduit 1" 60.0 ft | J1, J5, J7, J8 | 2,440 |
| 14.10 | House 1" PVC conduit: spare AC conduit, roof to inverter position | = allowance per point / measured run | 30.0 | ft |  |  |  |  |  |  |  | Conduit 1" 30.0 ft | J1, J5, J7, J8 | 1,220 |
| 14.11 | House 1¼" PVC conduit: spare DC conduit, roof to inverter position | = allowance per point / measured run | 30.0 | ft |  |  |  |  |  |  |  | Conduit 1¼" 30.0 ft | J4, J5 | 1,798 |
| 14.12 | House 1¼" PVC conduit: inverter AC-out to GF backup DB (45) + grid to inverter AC-in (45) | = allowance per point / measured run | 90.0 | ft |  |  |  |  |  |  |  | Conduit 1¼" 90.0 ft | J4, J5 | 5,393 |
| 14.13 | House 1¼" PVC conduit: backup risers GF->FF backup DB (20) and ->mumty backup DB (32) | = allowance per point / measured run | 52.0 | ft |  |  |  |  |  |  |  | Conduit 1¼" 52.0 ft | J4, J5 | 3,116 |
| 14.14 | House 1½" PVC conduit: meter to main DB + risers to FF and mumty DBs | = allowance per point / measured run | 85.0 | ft |  |  |  |  |  |  |  | Conduit 1½" 85.0 ft | J6, J8 | 6,821 |
| 14.15 | House 1½" PVC conduit: EV charger at front car porch from main DB (7.4 kW 1-ph / 11 kW 3-ph) | = allowance per point / measured run | 35.0 | ft |  |  |  |  |  |  |  | Conduit 1½" 35.0 ft | J6, J8 | 2,809 |
| 14.16 | House Conduit accessories: one bend and one coupler per 10 ft of conduit; PVC solvent | = by conduit length | 1 | set |  |  |  |  |  |  |  | Bends ¾" 267 nos Couplers ¾" 267 nos Bends 1" 158 nos Couplers 1" 158 nos Bends 1¼" 18 nos Couplers 1¼" 18 nos Bends 1½" 12 nos Couplers 1½" 12 nos Solvent tins 5 nos | J8 | 33,430 |
| 14.17 | House Back boxes: 39 switch-board boxes, 79 socket boxes, 10 A.C boxes, 13 fan boxes with hooks, 110 light junction boxes, 5 access-point boxes, 7 CCTV boxes, 1 NVR/router box, 2 bell boxes | = point count from sheets 13-16 + J2 | 266 | nos |  |  |  |  |  |  |  | Switch-board boxes 39 nos Socket boxes 79 nos A.C boxes 10 nos Fan boxes 13 nos Light boxes 110 nos Wi-Fi AP boxes 5 nos CCTV boxes 7 nos NVR box 1 nos Bell boxes 2 nos | J1, J2, J8 | 32,110 |
| 14.18 | GF, FF, mumty DB enclosures (empty): main 12-way GF, 8-way FF, 4-way mumty; backup (essential-load) 8-way GF, 6-way FF, 4-way mumty; EV isolator box | one main + one backup DB per floor + EV box = 3 main + 3 backup + 1 EV = 7 nos | 7 | nos |  |  |  |  |  |  |  | DB 12-way 1 nos DB 8-way 2 nos DB 6-way 1 nos DB 4-way 2 nos EV isolator box 1 nos | J5, J6 | 20,200 |
| 14.19 | Lawn near front door 3 earth pits (house, solar/inverter, lightning/surge): copper-bonded rod 5/8" x 10 ft, 2 bags earthing compound each, inspection chamber with cover | 3 nos | 3 | nos |  |  |  |  |  |  |  | Earth rods 3 nos Earth compound 6 bag Earth chambers 3 nos | J7 | 57,000 |
| 14.20 | Mumty and roof walls 2" PVC sleeves through walls for DC/AC conduits, pump cable and earth leads (6 nos x 1 ft) | 6 nos x 1 ft = 6 x 1 = 6 ft | 6.0 | ft |  |  |  |  |  |  |  | Sleeve 2" 6.0 ft | J4 | 720 |
| 14.21 | Terrace and mumty roof Solar mounting plinths 15" x 15" x 12" high (1:2:4), 4 #3 starter bars cast into slab + 2 #2 links; 2 cast-in M12 x 250 mm galvanised J-bolts each; cast before membrane | 34 nos (23 panels x 650 W = 14.95 kWp) = 34 x 1.25 x 1.25 x 1.0 | 53.1 | cft | 9.5 | 23.8 |  | 47.7 |  |  | #2 23.2 #3 40.6 | J-bolts M12 68 nos | J3 | 57,063 |
| 14.22 | Mumty laundry (inverter + sodium-ion battery position) Raised RCC plinth 3'-0" x 1'-6" x 6" for battery cabinet (1:2:4) | 3'-0" x 1'-6" x 6" = 3.0 x 1.5 x 0.5 = 2.25 cft | 2.2 | cft | 0.4 | 1.0 |  | 2.0 |  |  |  |  | J4 | 953 |
|  | S14 total - net |  |  |  | 9.9 | 24.8 |  | 49.7 |  |  | #2 23.2 #3 40.6 |  |  | 356,660 |
|  | S14 total - with wastage |  |  |  | 10.2 | 26.8 |  | 52.2 |  |  | #2 24.1 #3 42.2 |  |  |  |
| S15 Boundary walls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15.1 | All boundary walls Excavation to PCC bottom 2'-3" below NSL (into firm soil) | 141.9 ft x 2'-3" x 2'-3" = 141.9 x 2.25 x 2.25 x 1.10 | 790.2 | cft |  |  |  |  |  |  |  |  | G1, G3 | - |
| 15.2 | All boundary walls PCC 1:4:8, 3" x 2'-3" | 141.9 ft | 79.8 | cft | 7.7 | 38.6 |  | 77.1 |  |  |  |  | G3 | 24,004 |
| 15.3 | All boundary walls Brick footing steps 22½" / 18" / 13½" (6" each) in 1:6, eccentric (all projections inside plot) | 141.9 ft = 141.9 x 4.5 x 0.5 | 319.3 | cft | 14.6 |  | 109.2 |  |  | 4,310 |  |  | G3, C4 | 100,518 |
| 15.4 | All boundary walls Wall 9" and pillar bases in 1:6 from step top (-2'-0") to underside of plinth band (+7½") | panels 122.0 ft x 9" x 2'-7½"; pillars | 298.9 | cft | 13.6 |  | 102.2 |  |  | 4,035 |  |  | G3, D1 | 94,113 |
| 15.5 | All boundary walls RCC plinth band 9" x 6" (1:2:4), 4 #3 + #2 rings @ 9" (as before) | 141.9 ft | 53.2 | cft | 9.5 | 23.9 |  | 47.7 |  |  | #2 26.4 #3 101.6 |  | G3 | 57,629 |
| 15.6 | All boundary walls DPC 1½" 1:2:4 with waterproofing compound + 2 coats hot bitumen on plinth band (top at plinth level +1'-3") | 141.9 ft x 9" | 13.3 | cft | 2.4 | 6.0 |  | 11.9 |  |  |  | WP compound 2.4 kg Bitumen 42.6 kg | G3, B4 | 18,983 |
| 15.7 | All boundary walls Boundary wall 9" in 1:6 from DPC (+1'-3") to underside of coping; heights per sheets 11-12, back 7'-0", right 6'-0" / 7'-0" | Front panels: 23.6 ft to 6'-0"; Left short panel at side gate: 3.0 ft to 6'-0"; Left long panel: 43.8 ft to 7'-3"; Back wall on gali, 37'-3": 37.3 ft to 7'-0"; Right side, front stretch: 9.0 ft to 6'-0"; Right side, rear-passage stretch: 5.4 ft to 7'-0" = sum L x 0.75 x (H - 3" coping - 1'-3") | 484.8 | cft | 22.1 |  | 165.8 |  |  | 6,545 |  |  | G1, G2, D1 | 152,651 |
| 15.8 | Gates and corners Brick pillars in 1:4: 7 nos 13½" x 13½" + 3 nos 4'-0" x 13½" feature pillars, DPC to cap (6'-6" above road) | = 7 x 1.266 x 5.0 + 3 x 4.5 x 5.0 | 111.8 | cft | 7.1 |  | 35.7 |  |  | 1,509 |  |  | G2, D1 | 38,365 |
| 15.9 | All boundary walls RCC coping 3" x 12" on panels + 10 pillar caps (1:2:4), 2 #3 | panels 122.0 ft x 12" x 3" + 10 caps 1'-6" x 1'-6" x 3" = 122.0 x 1.0 x 0.25 + 10 x 1.5 x 1.5 x 0.25 = 36.1 cft; #3: 2 x 122.0 x 1.05 | 36.1 | cft | 6.5 | 16.2 |  | 32.4 |  |  | #3 43.7 |  | G3 | 27,206 |
| 15.10 | All boundary walls Plaster ¾" 1:4 on BOTH faces of all boundary walls (all sides), coping and pillars | outside face from road / gali / neighbour level, inside face from passage / porch / lawn level | 2,089.6 | sft | 34.7 |  | 173.7 |  |  |  |  |  | F1 | 63,324 |
| 15.11 | All boundary walls Back-filling with excavated earth | = 790 - 445 | 345.3 | cft |  |  |  |  |  |  |  |  | G3 | - |
|  | S15 total - net |  |  |  | 118.3 | 84.6 | 586.6 | 169.2 |  | 16,400 | #2 26.4 #3 145.3 |  |  | 576,794 |
|  | S15 total - with wastage |  |  |  | 121.9 | 91.4 | 633.6 | 177.7 |  | 17,220 | #2 27.4 #3 151.2 |  |  |  |
| S16 External works |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16.1 | Edge walls Back-filling around edge walls with excavated earth | trench 264.3 cft less PCC, brick steps and wall below NSL = 264.3 - (42.7 x 2.5 x 0.25 + 42.7 x 2.25 + 42.7 x 0.75 x 0.5) = 125.5 cft | 125.5 | cft |  |  |  |  |  |  |  |  | C6 | - |
| 16.2 | Column footing Back-filling around column footing | pit 33.7 cft less PCC, footing and column below NSL = 33.7 - (3.5 x 3.5 x 0.25 + 9.0 + 1.266 x 1.25) = 20.0 cft | 20.0 | cft |  |  |  |  |  |  |  |  | C5 | - |
| 16.3 | Left and rear passages Earth filling to underside of paving (+4"), compacted | 265 sft x 1.833 ft = 265.0 x 1.833 x 1.20 | 583.0 | cft |  |  |  |  |  |  |  |  | A5, G4 | - |
| 16.4 | Left and rear passages Sand 2" under paving base | 265 sft x 2" | 48.6 | cft |  |  | 48.6 |  |  |  |  |  | G4 | 2,099 |
| 16.5 | Left and rear passages PCC 1:4:8, 3" paving base sloped 1:60 away from house to drains (tuff-tile finish in finishing estimate) | 265 sft x 3" | 66.2 | cft | 6.4 | 32.0 |  | 64.0 |  |  |  |  | G4 | 19,924 |
| 16.6 | Outside main and side gates Ramps 1:6 (road to porch +1'-3"): compacted earth fill | 183 sft | 64.2 | cft |  |  |  |  |  |  |  |  | G4 | - |
| 16.7 | Outside main and side gates Ramps: sand 2" | 183 sft | 33.6 | cft |  |  | 33.6 |  |  |  |  |  | G4 | 1,452 |
| 16.8 | Outside main and side gates Ramps: PCC 1:4:8 3" base (finish in finishing estimate) | 183 sft | 45.8 | cft | 4.4 | 22.1 |  | 44.3 |  |  |  |  | G4 | 13,782 |
| 16.9 | Entrance, porch, rear and side doors Steps: brick 1:5 + PCC 1:4:8 tread base (allowance, as before) | entrance, two porch, rear-door and side-door steps = allowance: brickwork 40 cft (1:5) + PCC 20 cft (1:4:8) under treads | 40.0 | cft | 4.1 | 23.0 |  | 19.3 |  | 540 |  |  | G4 | 19,440 |
| 16.10 | Toe walls and rear-porch edges Plaster ¾" 1:4 on exposed face of edge walls | 22.3 ft x 1'-3" | 27.9 | sft | 0.5 |  | 2.3 |  |  |  |  |  | C6, F1 | 845 |
|  | S16 total - net |  |  |  | 15.4 | 77.1 | 84.5 | 127.6 |  | 540 |  |  |  | 57,541 |
|  | S16 total - with wastage |  |  |  | 15.8 | 83.3 | 91.3 | 134.0 |  | 567 |  |  |  |  |
| All stages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All stages - net |  |  |  | 1,498.9 | 2,332.5 | 4,960.4 | 3,801.4 | 665.1 | 126,536 | #2 155.1 #3 3,785.6 #4 2,165.0 #6 974.6 |  |  | 9,102,989.18 |
|  | All stages - with wastage |  |  |  | 1,543.9 | 2,519.1 | 5,357.2 | 3,991.4 | 698.3 | 132,862 | #2 161.3 #3 3,937.1 #4 2,251.6 #6 1,013.6 |  |  |  |
|  | Binding wire, 10 kg per ton of gross steel (E4) |  | 73.6 | kg |  |  |  |  |  |  |  | Binding wire 73.6 kg | E4 | 29,454.40 |
|  | Total of BOQ lines (before purchase rounding) |  |  |  |  |  |  |  |  |  |  |  |  | 9,132,443.58 |
|  | Rounding up to buying units (Section 5.1) |  |  |  |  |  |  |  |  |  |  |  |  | 57,351.42 |
|  | TOTAL MATERIAL COST |  |  |  |  |  |  |  |  |  |  |  |  | 9,189,795.00 |

# 5. Purchase schedule

## 5.1 Whole job - quantities to buy (governs the budget)

| Material | Net | Waste | With wastage | Purchase | Buying unit | No. of units | Trolley loads | Rate (Rs) | Cost (Rs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cement |  |  |  |  |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 1,498.9 | 3% | 1,543.9 | 1,544 bag | bag | 1,544 |  | 1,560 | 2,408,640 |
| Bricks |  |  |  |  |  |  |  |  |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 126,535.5 | 5% | 132,862.3 | 133,000 nos | 1,000 bricks | 133 | 67 | 16 | 2,128,000 |
| Steel |  |  |  |  |  |  |  |  |  |
| Steel Grade 60 deformed, #2 (6 mm) rings | 155.1 | 4% | 161.3 | 170.0 kg | 10 kg | 17 |  | 270 | 45,900 |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | 3,785.6 | 4% | 3,937.1 | 3,940.0 kg | 10 kg | 394 |  | 262 | 1,032,280 |
| Steel Grade 60 deformed, #4 (12 mm) | 2,165.0 | 4% | 2,251.6 | 2,260.0 kg | 10 kg | 226 |  | 262 | 592,120 |
| Steel Grade 60 deformed, #6 (20 mm) | 974.6 | 4% | 1,013.6 | 1,020.0 kg | 10 kg | 102 |  | 262 | 267,240 |
| Binding wire, annealed 18 gauge | 73.6 | 0% | 73.6 | 75.0 kg | 5 kg | 15 |  | 400 | 30,000 |
| Sand |  |  |  |  |  |  |  |  |  |
| Chenab sand (concrete, screed) | 2,332.5 | 8% | 2,519.1 | 2,550.0 cft | 50 cft | 51 | 26 | 65 | 165,750 |
| Ravi sand (mortar, plaster, blinding, fill) | 4,960.4 | 8% | 5,357.2 | 5,400.0 cft | 50 cft | 108 | 54 | 40 | 216,000 |
| Crush |  |  |  |  |  |  |  |  |  |
| Crush ¾" down, Sargodha (bajri) | 3,801.4 | 5% | 3,991.4 | 4,000.0 cft | 50 cft | 80 | 40 | 110 | 440,000 |
| Stone ballast |  |  |  |  |  |  |  |  |  |
| Stone ballast (khangar) 1½"-2½" | 665.1 | 5% | 698.3 | 700.0 cft | 50 cft | 14 | 7 | 90 | 63,000 |
| Earth |  |  |  |  |  |  |  |  |  |
| Earth (mitti) for filling, delivered | 4,184.8 | 0% | 4,184.8 | 4,200.0 cft | 50 cft | 84 | 42 | 25 | 105,000 |
| Waterproofing |  |  |  |  |  |  |  |  |  |
| Polythene sheet 0.2 mm (800 gauge) | 37.4 | 5% | 39.2 | 40.0 kg | kg | 40 |  | 550 | 22,000 |
| Hot bitumen 60/70 (drum) | 472.3 | 5% | 496.0 | 500.0 kg | 10 kg | 50 |  | 280 | 140,000 |
| Integral waterproofing compound (powder), 1 kg per bag of cement | 47.1 | 0% | 47.1 | 48.0 kg | kg | 48 |  | 350 | 16,800 |
| Bitumen primer | 62.9 | 5% | 66.0 | 68.0 L | 4 L tin | 17 |  | 900 | 61,200 |
| 4 mm APP/SBS modified-bitumen membrane (1 m x 10 m roll) | 209.6 | 5% | 220.1 | 230.0 m2 | roll (10 m2) | 23 |  | 700 | 161,000 |
| LPG for torching (11.8 kg cylinder) | 83.8 | 0% | 83.8 | 94.4 kg | cylinder (11.8 kg) | 8 |  | 300 | 28,320 |
| Polymer-modified cementitious waterproof coating (2-component) | 68.8 | 5% | 72.2 | 75.0 kg | 5 kg | 15 |  | 450 | 33,750 |
| Food-safe (potable-water) cementitious coating | 34.5 | 5% | 36.2 | 40.0 kg | 5 kg | 8 |  | 550 | 22,000 |
| PVC water-stop 150 mm | 20.0 | 5% | 21.0 | 21.0 ft | ft | 21 |  | 650 | 13,650 |
| Anti-termite |  |  |  |  |  |  |  |  |  |
| Termiticide: bifenthrin 10% EC (1 L makes 200 L emulsion) | 11.3 | 5% | 11.8 | 12.0 L | L | 12 |  | 3,500 | 42,000 |
| Roof tiles |  |  |  |  |  |  |  |  |  |
| Bhatta (brick) roof tiles 9" x 4½" x 1½" | 4,979.2 | 5% | 5,228.2 | 5,300 nos | 100 tiles | 53 |  | 20 | 106,000 |
| Chicken mesh |  |  |  |  |  |  |  |  |  |
| Galvanised chicken mesh, roll 3 ft x 100 ft | 16.4 | 5% | 17.3 | 18 roll | roll | 18 |  | 5,500 | 99,000 |
| Concrete nails with washers for mesh | 15.0 | 0% | 15.0 | 15.0 kg | kg | 15 |  | 450 | 6,750 |
| Sewer pipes |  |  |  |  |  |  |  |  |  |
| UPVC sewer pipe 5" (Dadex / Beta) | 159.9 | 7% | 171.1 | 180.0 ft | 20-ft length | 9 |  | 420 | 75,600 |
| UPVC sewer pipe 4" | 127.8 | 7% | 136.8 | 140.0 ft | 20-ft length | 7 |  | 300 | 42,000 |
| UPVC pipe 3" (waste / vent) | 230.1 | 7% | 246.2 | 260.0 ft | 20-ft length | 13 |  | 200 | 52,000 |
| UPVC rain-water pipe 3" | 112.0 | 7% | 119.8 | 120.0 ft | 20-ft length | 6 |  | 200 | 24,000 |
| UPVC fittings 5" (bends, tees, sockets) | 25 | 0% | 25 | 25 nos | nos | 25 |  | 950 | 23,750 |
| UPVC fittings 4" | 54 | 0% | 54 | 54 nos | nos | 54 |  | 600 | 32,400 |
| UPVC fittings 3" incl. floor / gully traps, roof outlets | 100 | 0% | 100 | 100 nos | nos | 100 |  | 380 | 38,000 |
| Vent cowl 3" | 1 | 0% | 1 | 1 nos | nos | 1 |  | 600 | 600 |
| PVC drain pipe ¾" (20 mm) for A.C condensate | 200.0 | 7% | 214.0 | 220.0 ft | 10-ft length | 22 |  | 35 | 7,700 |
| Stainless-steel mesh cap for A.C drain outlet | 10 | 0% | 10 | 10 nos | nos | 10 |  | 350 | 3,500 |
| Covers |  |  |  |  |  |  |  |  |  |
| Heavy-duty CI manhole cover with frame 20" x 20" | 2 | 0% | 2 | 2 nos | nos | 2 |  | 15,000 | 30,000 |
| Lockable steel / CI tank cover 2' x 2' | 1 | 0% | 1 | 1 nos | nos | 1 |  | 12,000 | 12,000 |
| Water pipes |  |  |  |  |  |  |  |  |  |
| PPR pipe 1¼" (40 mm) PN-16 (Popular / Dadex / Master) | 38.0 | 7% | 40.7 | 52.0 ft | 13-ft length | 4 |  | 270 | 14,040 |
| PPR pipe 1" (32 mm) PN-16 | 93.0 | 7% | 99.5 | 104.0 ft | 13-ft length | 8 |  | 175 | 18,200 |
| PPR pipe ¾" (25 mm) PN-16, cold | 430.0 | 7% | 460.1 | 468.0 ft | 13-ft length | 36 |  | 110 | 51,480 |
| PPR pipe ½" (20 mm) PN-16, cold | 140.0 | 7% | 149.8 | 156.0 ft | 13-ft length | 12 |  | 70 | 10,920 |
| PPR pipe ¾" (25 mm) PN-20, hot | 360.0 | 7% | 385.2 | 390.0 ft | 13-ft length | 30 |  | 135 | 52,650 |
| PPR pipe ½" (20 mm) PN-20, hot | 80.0 | 7% | 85.6 | 91.0 ft | 13-ft length | 7 |  | 85 | 7,735 |
| PPR fittings (average) | 335 | 0% | 335 | 335 nos | nos | 335 |  | 120 | 40,200 |
| Ball / gate valves (average) | 24 | 0% | 24 | 24 nos | nos | 24 |  | 900 | 21,600 |
| Float valve ¾" for underground tank | 1 | 0% | 1 | 1 nos | nos | 1 |  | 2,500 | 2,500 |
| Gas pipes |  |  |  |  |  |  |  |  |  |
| GI pipe medium class ½"-¾" for gas (SNGPL-approved) | 189.8 | 7% | 203.1 | 220.0 ft | 20-ft length | 11 |  | 260 | 57,200 |
| GI fittings ½"-¾" | 36 | 0% | 36 | 36 nos | nos | 36 |  | 250 | 9,000 |
| Electrical |  |  |  |  |  |  |  |  |  |
| PVC conduit ¾" heavy gauge | 2,662.0 | 7% | 2,848.3 | 2,850.0 ft | 10-ft length | 285 |  | 25 | 71,250 |
| PVC conduit 1" heavy gauge | 1,575.0 | 7% | 1,685.2 | 1,690.0 ft | 10-ft length | 169 |  | 38 | 64,220 |
| PVC conduit 1¼" heavy gauge | 172.0 | 7% | 184.0 | 190.0 ft | 10-ft length | 19 |  | 56 | 10,640 |
| PVC conduit 1½" heavy gauge | 120.0 | 7% | 128.4 | 130.0 ft | 10-ft length | 13 |  | 75 | 9,750 |
| Conduit bend ¾" | 267 | 0% | 267 | 267 nos | nos | 267 |  | 35 | 9,345 |
| Conduit bend 1" | 158 | 0% | 158 | 158 nos | nos | 158 |  | 55 | 8,690 |
| Conduit bend 1¼" | 18 | 0% | 18 | 18 nos | nos | 18 |  | 90 | 1,620 |
| Conduit bend 1½" | 12 | 0% | 12 | 12 nos | nos | 12 |  | 120 | 1,440 |
| Conduit coupler ¾" | 267 | 0% | 267 | 267 nos | nos | 267 |  | 15 | 4,005 |
| Conduit coupler 1" | 158 | 0% | 158 | 158 nos | nos | 158 |  | 25 | 3,950 |
| Conduit coupler 1¼" | 18 | 0% | 18 | 18 nos | nos | 18 |  | 40 | 720 |
| Conduit coupler 1½" | 12 | 0% | 12 | 12 nos | nos | 12 |  | 55 | 660 |
| PVC solvent cement (tin) | 5 | 0% | 5 | 5 nos | nos | 5 |  | 600 | 3,000 |
| Switch-board back box (assorted 3"x3" to 12"x3") | 39 | 0% | 39 | 39 nos | nos | 39 |  | 250 | 9,750 |
| Socket back box | 79 | 0% | 79 | 79 nos | nos | 79 |  | 120 | 9,480 |
| A.C point box | 10 | 0% | 10 | 10 nos | nos | 10 |  | 150 | 1,500 |
| Fan box with hook | 13 | 0% | 13 | 13 nos | nos | 13 |  | 180 | 2,340 |
| Round light junction box | 110 | 0% | 110 | 110 nos | nos | 110 |  | 60 | 6,600 |
| Ceiling box for Wi-Fi access point | 5 | 0% | 5 | 5 nos | nos | 5 |  | 150 | 750 |
| CCTV camera junction box | 7 | 0% | 7 | 7 nos | nos | 7 |  | 150 | 1,050 |
| NVR / router enclosure box | 1 | 0% | 1 | 1 nos | nos | 1 |  | 400 | 400 |
| Bell push box | 2 | 0% | 2 | 2 nos | nos | 2 |  | 120 | 240 |
| DB enclosure 12-way (empty) | 1 | 0% | 1 | 1 nos | nos | 1 |  | 4,500 | 4,500 |
| DB enclosure 8-way (empty) | 2 | 0% | 2 | 2 nos | nos | 2 |  | 3,200 | 6,400 |
| DB enclosure 6-way (empty) | 1 | 0% | 1 | 1 nos | nos | 1 |  | 2,800 | 2,800 |
| DB enclosure 4-way (empty) | 2 | 0% | 2 | 2 nos | nos | 2 |  | 2,000 | 4,000 |
| EV charger isolator enclosure | 1 | 0% | 1 | 1 nos | nos | 1 |  | 2,500 | 2,500 |
| PVC sleeve 2" | 6.0 | 0% | 6.0 | 6.0 ft | ft | 6 |  | 120 | 720 |
| Earthing |  |  |  |  |  |  |  |  |  |
| Copper-bonded earth rod 5/8" x 10 ft | 3 | 0% | 3 | 3 nos | nos | 3 |  | 9,000 | 27,000 |
| Earthing compound, 25 kg bag | 6 | 0% | 6 | 6 bag | bag | 6 |  | 3,500 | 21,000 |
| Earth-pit inspection chamber with cover | 3 | 0% | 3 | 3 nos | nos | 3 |  | 3,000 | 9,000 |
| Solar plinths |  |  |  |  |  |  |  |  |  |
| M12 x 250 mm galvanised J-bolt with nut and washer | 68 | 0% | 68 | 68 nos | nos | 68 |  | 250 | 17,000 |
| TOTAL MATERIAL COST |  |  |  |  |  |  |  |  | 9,189,795 |

Trolley loads: 100 cft per tractor-trolley (sand, crush, khangar, earth), 2,000 bricks per trolley. LPG is bought in 11.8 kg cylinders.

## 5.2 Stage by stage - what to order before each stage

Quantities include wastage and are rounded up to buying units for each stage on its own, so the stage orders add up to slightly more than the whole-job figures in 5.1 (which govern the budget). Order the next stage's cement in time but do not store cement for more than 4-6 weeks.

| Material | Needed (with wastage) | Order | Unit | Buying units | Trolley loads |
| --- | --- | --- | --- | --- | --- |
| S3 Anti-termite treatment (material cost Rs 41,407) |  |  |  |  |  |
| Termiticide: bifenthrin 10% EC (1 L makes 200 L emulsion) | 11.8 | 12.0 | L | 12 |  |
| S4 Foundations (material cost Rs 853,544) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 142.1 | 143 | bag | 143 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 34,314.3 | 35,000 | nos | 35 x 1,000 bricks | 18 |
| Steel Grade 60 deformed, #4 (12 mm) | 12.1 | 20.0 | kg | 2 x 10 kg |  |
| Chenab sand (concrete, screed) | 142.8 | 150.0 | cft | 3 x 50 cft | 2 |
| Ravi sand (mortar, plaster, blinding, fill) | 894.1 | 900.0 | cft | 18 x 50 cft | 9 |
| Crush ¾" down, Sargodha (bajri) | 277.6 | 300.0 | cft | 6 x 50 cft | 3 |
| Hot bitumen 60/70 (drum) | 13.5 | 20.0 | kg | 2 x 10 kg |  |
| Integral waterproofing compound (powder), 1 kg per bag of cement | 0.7 | 1.0 | kg | 1 |  |
| Binding wire, annealed 18 gauge | 0.1 | 5.0 | kg | 1 x 5 kg |  |
| S5 Plinth beam and DPC (material cost Rs 405,590) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 59.1 | 60 | bag | 60 |  |
| Steel Grade 60 deformed, #2 (6 mm) rings | 81.8 | 90.0 | kg | 9 x 10 kg |  |
| Steel Grade 60 deformed, #4 (12 mm) | 509.5 | 510.0 | kg | 51 x 10 kg |  |
| Chenab sand (concrete, screed) | 114.7 | 150.0 | cft | 3 x 50 cft | 2 |
| Ravi sand (mortar, plaster, blinding, fill) | 60.3 | 100.0 | cft | 2 x 50 cft | 1 |
| Crush ¾" down, Sargodha (bajri) | 223.0 | 250.0 | cft | 5 x 50 cft | 3 |
| Hot bitumen 60/70 (drum) | 414.6 | 420.0 | kg | 42 x 10 kg |  |
| Integral waterproofing compound (powder), 1 kg per bag of cement | 21.0 | 21.0 | kg | 21 |  |
| Binding wire, annealed 18 gauge | 5.9 | 10.0 | kg | 2 x 5 kg |  |
| S6 Filling and sub-floor (material cost Rs 333,581) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 45.1 | 46 | bag | 46 |  |
| Chenab sand (concrete, screed) | 236.6 | 250.0 | cft | 5 x 50 cft | 3 |
| Ravi sand (mortar, plaster, blinding, fill) | 204.1 | 250.0 | cft | 5 x 50 cft | 3 |
| Crush ¾" down, Sargodha (bajri) | 460.0 | 500.0 | cft | 10 x 50 cft | 5 |
| Stone ballast (khangar) 1½"-2½" | 698.3 | 700.0 | cft | 14 x 50 cft | 7 |
| Earth (mitti) for filling, delivered | 4,184.8 | 4,200.0 | cft | 84 x 50 cft | 42 |
| Polythene sheet 0.2 mm (800 gauge) | 39.2 | 40.0 | kg | 40 |  |
| S7 Masonry (material cost Rs 1,697,075) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 277.3 | 278 | bag | 278 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 74,295.3 | 75,000 | nos | 75 x 1,000 bricks | 38 |
| Ravi sand (mortar, plaster, blinding, fill) | 1,895.2 | 1,900.0 | cft | 38 x 50 cft | 19 |
| S8 RCC (superstructure) (material cost Rs 2,760,483) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 455.2 | 456 | bag | 456 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 1,422.3 | 2,000 | nos | 2 x 1,000 bricks | 1 |
| Steel Grade 60 deformed, #2 (6 mm) rings | 28.0 | 30.0 | kg | 3 x 10 kg |  |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | 3,477.5 | 3,480.0 | kg | 348 x 10 kg |  |
| Steel Grade 60 deformed, #4 (12 mm) | 1,730.0 | 1,730.0 | kg | 173 x 10 kg |  |
| Steel Grade 60 deformed, #6 (20 mm) | 1,013.6 | 1,020.0 | kg | 102 x 10 kg |  |
| Chenab sand (concrete, screed) | 1,274.9 | 1,300.0 | cft | 26 x 50 cft | 13 |
| Ravi sand (mortar, plaster, blinding, fill) | 52.9 | 100.0 | cft | 2 x 50 cft | 1 |
| Crush ¾" down, Sargodha (bajri) | 2,478.9 | 2,500.0 | cft | 50 x 50 cft | 25 |
| Polymer-modified cementitious waterproof coating (2-component) | 72.2 | 75.0 | kg | 15 x 5 kg |  |
| Binding wire, annealed 18 gauge | 62.5 | 65.0 | kg | 13 x 5 kg |  |
| S9 Roof waterproofing and tiles (material cost Rs 561,331) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 117.0 | 117 | bag | 117 |  |
| Chenab sand (concrete, screed) | 452.0 | 500.0 | cft | 10 x 50 cft | 5 |
| Ravi sand (mortar, plaster, blinding, fill) | 155.9 | 200.0 | cft | 4 x 50 cft | 2 |
| Bitumen primer | 66.0 | 68.0 | L | 17 x 4 L tin |  |
| 4 mm APP/SBS modified-bitumen membrane (1 m x 10 m roll) | 220.1 | 230.0 | m2 | 23 x roll (10 m2) |  |
| LPG for torching (11.8 kg cylinder) | 83.8 | 94.4 | kg | 8 x cylinder (11.8 kg) |  |
| Bhatta (brick) roof tiles 9" x 4½" x 1½" | 5,228.2 | 5,300 | nos | 53 x 100 tiles |  |
| S10 Chicken mesh and plaster (material cost Rs 509,522) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 230.5 | 231 | bag | 231 |  |
| Ravi sand (mortar, plaster, blinding, fill) | 1,205.2 | 1,250.0 | cft | 25 x 50 cft | 13 |
| Galvanised chicken mesh, roll 3 ft x 100 ft | 17.3 | 18 | roll | 18 |  |
| Concrete nails with washers for mesh | 15.0 | 15.0 | kg | 15 |  |
| S11 Sewerage and drainage (material cost Rs 444,965) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 29.8 | 30 | bag | 30 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 3,683.7 | 4,000 | nos | 4 x 1,000 bricks | 2 |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | 43.5 | 50.0 | kg | 5 x 10 kg |  |
| Chenab sand (concrete, screed) | 32.1 | 50.0 | cft | 1 x 50 cft | 1 |
| Ravi sand (mortar, plaster, blinding, fill) | 105.7 | 150.0 | cft | 3 x 50 cft | 2 |
| Crush ¾" down, Sargodha (bajri) | 62.4 | 100.0 | cft | 2 x 50 cft | 1 |
| UPVC sewer pipe 5" (Dadex / Beta) | 166.8 | 180.0 | ft | 9 x 20-ft length |  |
| UPVC sewer pipe 4" | 136.8 | 140.0 | ft | 7 x 20-ft length |  |
| UPVC pipe 3" (waste / vent) | 233.3 | 240.0 | ft | 12 x 20-ft length |  |
| UPVC rain-water pipe 3" | 119.8 | 120.0 | ft | 6 x 20-ft length |  |
| UPVC fittings 5" (bends, tees, sockets) | 23 | 23 | nos | 23 |  |
| UPVC fittings 4" | 54 | 54 | nos | 54 |  |
| UPVC fittings 3" incl. floor / gully traps, roof outlets | 100 | 100 | nos | 100 |  |
| Heavy-duty CI manhole cover with frame 20" x 20" | 2 | 2 | nos | 2 |  |
| PVC drain pipe ¾" (20 mm) for A.C condensate | 214.0 | 220.0 | ft | 22 x 10-ft length |  |
| Stainless-steel mesh cap for A.C drain outlet | 10 | 10 | nos | 10 |  |
| Binding wire, annealed 18 gauge | 0.4 | 5.0 | kg | 1 x 5 kg |  |
| S12 Septic tank (material cost Rs 80,400) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 18.2 | 19 | bag | 19 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 1,200.0 | 2,000 | nos | 2 x 1,000 bricks | 1 |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | 61.5 | 70.0 | kg | 7 x 10 kg |  |
| Chenab sand (concrete, screed) | 20.6 | 50.0 | cft | 1 x 50 cft | 1 |
| Ravi sand (mortar, plaster, blinding, fill) | 52.2 | 100.0 | cft | 2 x 50 cft | 1 |
| Crush ¾" down, Sargodha (bajri) | 40.1 | 50.0 | cft | 1 x 50 cft | 1 |
| Integral waterproofing compound (powder), 1 kg per bag of cement | 5.7 | 6.0 | kg | 6 |  |
| UPVC sewer pipe 5" (Dadex / Beta) | 4.3 | 20.0 | ft | 1 x 20-ft length |  |
| UPVC pipe 3" (waste / vent) | 12.8 | 20.0 | ft | 1 x 20-ft length |  |
| UPVC fittings 5" (bends, tees, sockets) | 2 | 2 | nos | 2 |  |
| Vent cowl 3" | 1 | 1 | nos | 1 |  |
| Binding wire, annealed 18 gauge | 0.6 | 5.0 | kg | 1 x 5 kg |  |
| S13 Water tanks, water and gas pipes (material cost Rs 424,095) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 21.6 | 22 | bag | 22 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 159.5 | 1,000 | nos | 1 x 1,000 bricks | 1 |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | 161.3 | 170.0 | kg | 17 x 10 kg |  |
| Chenab sand (concrete, screed) | 44.0 | 50.0 | cft | 1 x 50 cft | 1 |
| Ravi sand (mortar, plaster, blinding, fill) | 6.9 | 50.0 | cft | 1 x 50 cft | 1 |
| Crush ¾" down, Sargodha (bajri) | 85.5 | 100.0 | cft | 2 x 50 cft | 1 |
| Hot bitumen 60/70 (drum) | 23.2 | 30.0 | kg | 3 x 10 kg |  |
| Integral waterproofing compound (powder), 1 kg per bag of cement | 17.3 | 18.0 | kg | 18 |  |
| Food-safe (potable-water) cementitious coating | 36.2 | 40.0 | kg | 8 x 5 kg |  |
| PVC water-stop 150 mm | 21.0 | 21.0 | ft | 21 |  |
| Lockable steel / CI tank cover 2' x 2' | 1 | 1 | nos | 1 |  |
| PPR pipe 1¼" (40 mm) PN-16 (Popular / Dadex / Master) | 40.7 | 52.0 | ft | 4 x 13-ft length |  |
| PPR pipe 1" (32 mm) PN-16 | 99.5 | 104.0 | ft | 8 x 13-ft length |  |
| PPR pipe ¾" (25 mm) PN-16, cold | 460.1 | 468.0 | ft | 36 x 13-ft length |  |
| PPR pipe ½" (20 mm) PN-16, cold | 149.8 | 156.0 | ft | 12 x 13-ft length |  |
| PPR pipe ¾" (25 mm) PN-20, hot | 385.2 | 390.0 | ft | 30 x 13-ft length |  |
| PPR pipe ½" (20 mm) PN-20, hot | 85.6 | 91.0 | ft | 7 x 13-ft length |  |
| GI pipe medium class ½"-¾" for gas (SNGPL-approved) | 203.1 | 220.0 | ft | 11 x 20-ft length |  |
| PPR fittings (average) | 335 | 335 | nos | 335 |  |
| Ball / gate valves (average) | 24 | 24 | nos | 24 |  |
| Float valve ¾" for underground tank | 1 | 1 | nos | 1 |  |
| GI fittings ½"-¾" | 36 | 36 | nos | 36 |  |
| Binding wire, annealed 18 gauge | 1.6 | 5.0 | kg | 1 x 5 kg |  |
| S14 Electrical civil provisions and solar plinths (material cost Rs 356,660) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 10.2 | 11 | bag | 11 |  |
| Steel Grade 60 deformed, #2 (6 mm) rings | 24.1 | 30.0 | kg | 3 x 10 kg |  |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | 42.2 | 50.0 | kg | 5 x 10 kg |  |
| Chenab sand (concrete, screed) | 26.8 | 50.0 | cft | 1 x 50 cft | 1 |
| Crush ¾" down, Sargodha (bajri) | 52.2 | 100.0 | cft | 2 x 50 cft | 1 |
| PVC conduit ¾" heavy gauge | 2,848.3 | 2,850.0 | ft | 285 x 10-ft length |  |
| PVC conduit 1" heavy gauge | 1,685.2 | 1,690.0 | ft | 169 x 10-ft length |  |
| PVC conduit 1¼" heavy gauge | 184.0 | 190.0 | ft | 19 x 10-ft length |  |
| PVC conduit 1½" heavy gauge | 128.4 | 130.0 | ft | 13 x 10-ft length |  |
| Conduit bend ¾" | 267 | 267 | nos | 267 |  |
| Conduit bend 1" | 158 | 158 | nos | 158 |  |
| Conduit bend 1¼" | 18 | 18 | nos | 18 |  |
| Conduit bend 1½" | 12 | 12 | nos | 12 |  |
| Conduit coupler ¾" | 267 | 267 | nos | 267 |  |
| Conduit coupler 1" | 158 | 158 | nos | 158 |  |
| Conduit coupler 1¼" | 18 | 18 | nos | 18 |  |
| Conduit coupler 1½" | 12 | 12 | nos | 12 |  |
| PVC solvent cement (tin) | 5 | 5 | nos | 5 |  |
| Switch-board back box (assorted 3"x3" to 12"x3") | 39 | 39 | nos | 39 |  |
| Socket back box | 79 | 79 | nos | 79 |  |
| A.C point box | 10 | 10 | nos | 10 |  |
| Fan box with hook | 13 | 13 | nos | 13 |  |
| Round light junction box | 110 | 110 | nos | 110 |  |
| Ceiling box for Wi-Fi access point | 5 | 5 | nos | 5 |  |
| CCTV camera junction box | 7 | 7 | nos | 7 |  |
| NVR / router enclosure box | 1 | 1 | nos | 1 |  |
| Bell push box | 2 | 2 | nos | 2 |  |
| DB enclosure 12-way (empty) | 1 | 1 | nos | 1 |  |
| DB enclosure 8-way (empty) | 2 | 2 | nos | 2 |  |
| DB enclosure 6-way (empty) | 1 | 1 | nos | 1 |  |
| DB enclosure 4-way (empty) | 2 | 2 | nos | 2 |  |
| EV charger isolator enclosure | 1 | 1 | nos | 1 |  |
| Copper-bonded earth rod 5/8" x 10 ft | 3 | 3 | nos | 3 |  |
| Earthing compound, 25 kg bag | 6 | 6 | bag | 6 |  |
| Earth-pit inspection chamber with cover | 3 | 3 | nos | 3 |  |
| PVC sleeve 2" | 6.0 | 6.0 | ft | 6 |  |
| M12 x 250 mm galvanised J-bolt with nut and washer | 68 | 68 | nos | 68 |  |
| Binding wire, annealed 18 gauge | 0.7 | 5.0 | kg | 1 x 5 kg |  |
| S15 Boundary walls (material cost Rs 576,794) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 121.9 | 122 | bag | 122 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 17,220.2 | 18,000 | nos | 18 x 1,000 bricks | 9 |
| Steel Grade 60 deformed, #2 (6 mm) rings | 27.4 | 30.0 | kg | 3 x 10 kg |  |
| Steel Grade 60 deformed, #3 (10 mm) (Amreli / Mughal / FF Steel) | 151.2 | 160.0 | kg | 16 x 10 kg |  |
| Chenab sand (concrete, screed) | 91.4 | 100.0 | cft | 2 x 50 cft | 1 |
| Ravi sand (mortar, plaster, blinding, fill) | 633.6 | 650.0 | cft | 13 x 50 cft | 7 |
| Crush ¾" down, Sargodha (bajri) | 177.7 | 200.0 | cft | 4 x 50 cft | 2 |
| Hot bitumen 60/70 (drum) | 44.7 | 50.0 | kg | 5 x 10 kg |  |
| Integral waterproofing compound (powder), 1 kg per bag of cement | 2.4 | 3.0 | kg | 3 |  |
| Binding wire, annealed 18 gauge | 1.8 | 5.0 | kg | 1 x 5 kg |  |
| S16 External works (material cost Rs 57,541) |  |  |  |  |  |
| Cement OPC, 50 kg bag (Lucky / Bestway / Maple Leaf) | 15.8 | 16 | bag | 16 |  |
| Bricks, first class (awwal), 9" x 4½" x 3" | 567 | 1,000 | nos | 1 x 1,000 bricks | 1 |
| Chenab sand (concrete, screed) | 83.3 | 100.0 | cft | 2 x 50 cft | 1 |
| Ravi sand (mortar, plaster, blinding, fill) | 91.3 | 100.0 | cft | 2 x 50 cft | 1 |
| Crush ¾" down, Sargodha (bajri) | 134.0 | 150.0 | cft | 3 x 50 cft | 2 |

# 6. Steel (sariya) schedule by bar size and member group

| Member group (net kg) | #2 (6 mm) | #3 (10 mm) | #4 (12 mm) | #6 (20 mm) | Total (kg) |
| --- | --- | --- | --- | --- | --- |
| Column footing |  |  | 11.7 |  | 11.7 |
| Plinth beam | 78.7 |  | 490.0 |  | 568.6 |
| Column |  | 23.2 |  | 51.8 | 75.0 |
| Slabs |  | 2,506.1 | 580.1 | 55.0 | 3,141.3 |
| Concealed beams |  | 268.1 | 107.9 | 867.8 | 1,243.7 |
| Lintels | 26.9 | 204.2 | 393.0 |  | 624.1 |
| Bed plates |  |  | 57.5 |  | 57.5 |
| Stairs |  | 96.8 | 525.0 |  | 621.7 |
| Chajjas |  | 39.8 |  |  | 39.8 |
| Elevation RCC |  | 25.6 |  |  | 25.6 |
| Coping |  | 78.7 |  |  | 78.7 |
| Sunken slabs |  | 101.2 |  |  | 101.2 |
| Manhole covers |  | 41.8 |  |  | 41.8 |
| Septic tank |  | 59.1 |  |  | 59.1 |
| Water tank |  | 134.6 |  |  | 134.6 |
| Tank platform |  | 20.5 |  |  | 20.5 |
| Solar plinths | 23.2 | 40.6 |  |  | 63.8 |
| Boundary walls | 26.4 | 145.3 |  |  | 171.7 |
| TOTAL net | 155.1 | 3,785.6 | 2,165.0 | 974.6 | 7,080.4 |
| With 4% wastage | 161.3 | 3,937.1 | 2,251.6 | 1,013.6 | 7,363.6 |
| PURCHASE (rounded to 10 kg) | 170 | 3,940 | 2,260 | 1,020 | 7,390 |
| Cost (Rs) | 45,900 | 1,032,280 | 592,120 | 267,240 | 1,937,540 |

Binding wire: 75 kg (10 kg per ton of gross steel, E4). Laps, hooks, cranks and chairs are included in the net weights (laps as Rev 0). The S-02 RCC strip footing steel (673 kg of #3) is no longer required (C1).

## Steel by construction stage (kg, with wastage)

| Stage | #2 | #3 | #4 | #6 | Total (kg) |
| --- | --- | --- | --- | --- | --- |
| S4 Foundations |  |  | 12.1 |  | 12.1 |
| S5 Plinth beam and DPC | 81.8 |  | 509.5 |  | 591.4 |
| S8 RCC (superstructure) | 28.0 | 3,477.5 | 1,730.0 | 1,013.6 | 6,249.0 |
| S11 Sewerage and drainage |  | 43.5 |  |  | 43.5 |
| S12 Septic tank |  | 61.5 |  |  | 61.5 |
| S13 Water tanks, water and gas pipes |  | 161.3 |  |  | 161.3 |
| S14 Electrical civil provisions and solar plinths | 24.1 | 42.2 |  |  | 66.3 |
| S15 Boundary walls | 27.4 | 151.2 |  |  | 178.6 |
| TOTAL with wastage | 161.3 | 3,937.1 | 2,251.6 | 1,013.6 | 7,363.6 |

Main bar details are in each BOQ description (Section 4): slabs per S-06 to S-08 bar sets; concealed beams CB-1/CB-2 per S-09; lintels per S-05; stair per S-03; plinth beam 2 #4 top + 2 #4 bottom with #2 rings @ 10".

# 7. Solar layout summary (J3, J4)

| Location | Panels | Arrangement | Position | Plinths |
| --- | --- | --- | --- | --- |
| Terrace row 1 (front) | 8 | 4 panels across x 2 high (landscape) | x 1'-6" to 33'-0" from left parapet; 2'-3" behind the front parapet | 10 |
| Terrace row 2 (beside open shaft) | 6 | 3 across x 2 high | clear of the shaft by 1'-6" | 8 |
| Terrace row 3 (rear) | 6 | 3 across x 2 high | 2'-1½" walkway behind, in front of mumty | 8 |
| Mumty roof - room block | 2 | 2 panels portrait, 1 high | west of the water-tank platform | 4 |
| Mumty roof - stair block | 1 | 1 panel landscape |  | 4 |
| Total | 23 | 23 x 650 W = 14.95 kWp |  | 34 |

| Panels | 23 x 650 W (2,384 x 1,303 mm (7'-9¾" x 4'-3¼") bifacial), tilted 15° facing the front (assumed south - confirm north on site). |
| --- | --- |
| Area | Panels cover about 750 sft in plan. Terrace 1,302 sft, mumty roof 319 sft. |
| Spacing | Row pitch 11'-6" (no row-to-row shading at winter noon); 2'-3" behind the front parapet; 1'-6" along the side parapets; 3'-3" clear between rows; 2'-1½" behind the rear row. |
| RCC plinths | 34 nos, 15" x 15" x 12" high, 1:2:4, 4 #3 starter bars + 2 #2 links, 2 cast-in M12 x 250 mm galvanised J-bolts. Cast on the roof slab BEFORE the membrane; the membrane is dressed round each plinth (item 9.3). Item 14.21: 53.1 cft concrete, 40.6 kg #3 + 23.2 kg #2, 68 J-bolts. |
| Conduits (J4) | Spare 1¼" DC and 1" AC conduits from the roof to the inverter (items 14.10-14.11); 2" sleeves through walls (14.20). |
| Inverter and battery | Mumty laundry (2nd floor): wall space for the inverter and a 3'-0" x 1'-6" x 6" RCC plinth for the sodium-ion battery cabinet (14.22). |
| Backup scheme (J5) | Inverter AC-out feeds backup (essential-load) DBs on GF, FF and mumty (items 14.12-14.13, 14.18); grid feeds the main DBs. |
| Earthing (J7) | Separate earth pit for solar / inverter (one of 3 pits, item 14.19). |

# 8. Exclusions and items moved to the finishing estimate

| Category | Items |
| --- | --- |
| Excluded by K1 | Labour, shuttering and formwork, scaffolding, tools, curing water and construction electricity. |
| Finishing estimate | Floor finishes (tiles / marble in house, tuff tiles on porches, passages, driveway strip and ramps); skirting; wall and ceiling finishes; false ceilings; paint. |
| Finishing estimate | Doors, windows, glazing, grills, railings (incl. glass railing on the front balcony), main and side steel gates. |
| Finishing estimate | Plastic roof water tank (500 gal recommended), pump, float switch, geyser, sanitary fixtures and taps, kitchen work. |
| Finishing estimate | Electrical wires and cables (incl. Cat6, PV and earth cables), switches, sockets, breakers, DB internals, inverter, sodium-ion batteries, solar panels and mounting frames, EV charger, CCTV cameras, NVR, Wi-Fi access points. |
| Finishing estimate | Lawn soil and landscaping (lawn filling to +1'-2"), planters, external lights. |
| Not included | Recommended additions in Section 9 (they are not in the total). |
| Deleted from Rev 0 | Ceiling plaster (F1 - false ceilings are finishing work); RCC overhead tank (I4 - replaced by a plastic tank, which is finishing); TV and intercom conduits (J2). |

# 9. Recommended additions (NOT in the total)

These are not decided in the Register, so they are kept out of the total. Each is worth doing for the reason given; approve any of them and add its cost.

| No. | Addition | Reason | Material cost (Rs) |
| --- | --- | --- | --- |
| 1 | Two more terrace outlets (front-right, rear-right) and one mumty-roof spout | with the drawn outlets the slope screed reaches ~5⅜" and the build-up ~7¾" at the far corner (over the 6½" allowed); 4 terrace outlets keep it within 5¾" and save ~97 cft of screed | -31,443 (saving) |
| 2 | Three 13½" pillars in the new 37'-3" back wall (about every 10 ft) | a free-standing 9" wall 7'-0" high and 37 ft long needs pillars for stability against wind and impact | 5,663 |
| 3 | Roof thermal insulation: 1" XPS board under the bhatta tiles (~1,581 sft) | cuts summer heat gain on the top floor; needs the extra outlets above so the build-up still fits | 150,195 |
| 4 | Second (double) mesh of #3 @ 6" in the underground tank walls | better crack control for a water-retaining wall | 23,793 |
| 5 | Enlarge septic tank to a practical 3-chamber size, 7'-0" x 3'-6" internal | the drawing size gives chambers only ~1'-0" long, too small to clean | 30,000 |
| 6 | Vent / overflow pipe 1½" with insect mesh for the underground tank | lets air out while filling; prevents insects | 2,000 |
| 7 | Pump plinth 2'-0" x 1'-6" x 6" RCC at the back | keeps the pump off a wet floor | 635 |
| 8 | ISP fibre / internet entry conduit 1" from front wall to router point (~60 ft) | lets the internet cable come in concealed | 2,770 |
|  | Net effect if all are approved |  | 183,613 |

# 10. Remaining assumptions (each marked ASSUMED)

Where the drawings and the Register are silent, sensible Faisalabad practice was used without over-building. Each item below is an assumption to be confirmed on site or by the designers.

| No. | Area | Assumption | Status |
| --- | --- | --- | --- |
| 1 | General | The filled questionnaire was not attached to this session. The Final Decisions Register (which combines your answers) was used as the only record of your answers. | ASSUMED |
| 2 | Plinth beam | Plinth beam: sheet 05 gives size only (9" x 9"). Steel 2 #4 top + 2 #4 bottom with #2 rings @ 10" and mix 1:2:4 are taken from the engineer's S-02 plinth beam. | ASSUMED |
| 3 | DPC | DPC is 9" wide on top of the plinth beam under ALL house walls, including 4½" walls (sheet 05 section C-C shows the 9" beam under 4½" walls). | ASSUMED |
| 4 | Levels | Finished level of side and rear passages taken as +1'-0" (not drawn), sloping 1:60 to drains; paving finish allowance 3" (finishing estimate). | ASSUMED |
| 5 | Levels | Porch finish taken 1½" thick (as house floor), so porch sub-floor PCC top is +1'-1½". | ASSUMED |
| 6 | Edge walls | Edge walls are founded like the boundary walls (PCC bottom 2'-3" below NSL) and use the D-D section (steps 22½"/18"/13½", 9" wall, DPC at porch level). | ASSUMED |
| 7 | Edge walls | Edge walls identified from the levels: toe wall lawn/porch 9'-3", toe wall across left passage 4'-11", rear porch edges 3'-1" and 5'-0", gate thresholds 12'-9" and 7'-8¼" = 42'-9". | ASSUMED |
| 8 | Ramps | Ramps are built outside the main and side gates on the road verge, slope 1:6, 7'-6" long with 1'-0" flares (gate thresholds stay at porch level +1'-3"). | ASSUMED |
| 9 | Car porch | Front car porch: 20'-0" x 16'-7½" (sheet 04, confirmed by measurement) less the entrance-lobby notch = 308 sft. The 8'-0" "drive way" on the plans lies inside this zone and gets the porch sub-floor. | ASSUMED |
| 10 | Car porch | Rear car porch 16'-9½" x 8'-6" (sheets 06, 17, 21). | ASSUMED |
| 11 | Boundary walls | Right-side boundary walls: front stretch 9'-0" long x 6'-0" high, rear-passage stretch 5'-5" x 7'-0" high. The open patio stretch (4'-9") stays open (D2: no privacy walls). | ASSUMED |
| 12 | Boundary walls | All boundary-wall footings are eccentric (projections inside the plot) because every boundary wall stands on a property line (C4). | ASSUMED |
| 13 | Septic tank | Septic tank: the 4'-10" x 3'-5" rectangle on sheet 17 is read as the INTERNAL size. With two 4½" baffles, three chambers come out about 2'-0", 1'-0" and 1'-0" long - too small to clean. Drawing size used as instructed; enlargement listed in Recommended additions. | ASSUMED |
| 14 | Septic tank | Septic tank liquid depth 4'-0" + freeboard 1'-0" (internal depth 5'-0"); capacity ~410 gal liquid. | ASSUMED |
| 15 | Water tank | Underground tank: internal 4'-10" x 3'-5" (same as septic tank), 5'-0" water + 6" freeboard = about 515 imperial gallons; in the rear car porch, top 7" above porch level. | ASSUMED |
| 16 | Water tank | Underground tank distance: no spot at the back is 10 ft from every manhole, because three manholes run along the rear. The chosen spot (rear car porch, toward the house) is about 8½ ft (centre to centre) from the two nearest rear manholes - see points to confirm. | ASSUMED |
| 17 | Roof tank | Plastic roof tank recommended 500 gallons. Platform 5'-0" x 5'-0" x 4" RCC on two 9" brick walls 1'-6" high, set directly over the mumty walls at the sheet-24 position. | ASSUMED |
| 18 | Solar | Solar: the drawings have no north arrow. The layout assumes the front (road side) faces south; if not, the rows must be re-placed (the plinth count stays about the same). | ASSUMED |
| 19 | Solar | Solar: 650 W panels 2,384 x 1,303 mm, 2-high landscape tables tilted 15°, low edge about 1'-6" above the roof tiles; row pitch 11'-6" (no row-to-row shading at noon on 21 December, sun about 35° high); front row set 2'-3" back from the 3'-0" front parapet so the parapet shadow stays off the panels; walkways 1'-6" along the side parapets. | ASSUMED |
| 20 | Anti-termite | Anti-termite: bifenthrin 10% EC diluted 1:200 (0.05%), 5 L of emulsion per m2 on trench surfaces and plinth fill, 2.25 L per running metre along the perimeter (IS 6313-2 rates). If imidacloprid 30.5% SC is used, dilute 1:400 (about 5.6 L concentrate). | ASSUMED |
| 21 | Roof | Roof build-up: screed 1:4 cement-sand (min ¾"), 4 mm membrane, ¾" mortar bed, 1½" bhatta tiles; membrane laps +10%; LPG 0.4 kg per m2. | ASSUMED |
| 22 | Floors | Upper-floor 3" PCC sub-base kept from Rev 0 (the Register does not mention it; it covers the 4" concealed-beam upstands and conduits). | ASSUMED |
| 23 | Sunken baths | Sunken slabs: 3 first-floor baths + mumty bath (laundry not sunk); filling 9" deep (6" drop + 3" floor base) with brick-bat coba. | ASSUMED |
| 24 | Chicken mesh | Chicken-mesh lengths for chases are allowances (electrical drops per point, 60% of water pipe length, 50% of gas pipe, 80% of A.C drains). | ASSUMED |
| 25 | Electrical | Electrical positions proposed: Wi-Fi access points - GF front lounge/drawing and GF rear lobby, FF lounge and FF rear lobby, mumty stair lobby (also covers the terrace); CCTV - main gate (outside), front porch, side gate / rear porch, back gali (from first-floor rear wall), rear passage, left passage front, left passage rear; NVR/router point in the GF lobby near the stair. | ASSUMED |
| 26 | Electrical | Inverter + sodium-ion battery position: mumty laundry (roofed, out of sun, ventilated on the terrace side) with a 6" RCC plinth for the battery cabinet and wall space for the inverter. | ASSUMED |
| 27 | Earthing | Earth pits: copper-bonded rod 5/8" x 10 ft in an augered bore with 2 bags of earthing compound each; chamber with cover. | ASSUMED |
| 28 | Geometry | Wall, slab, lintel and opening geometry is re-used from Rev 0 (measured from S-01 / S-04 to S-08). The architect's ground-floor master bedroom is 3" longer (14'-3" vs 14'-0"); effect on quantities is under 0.1%. | ASSUMED |
| 29 | Water supply | Society water supply enters at the front; 60 ft of ¾" PPR runs to the underground tank at the back; pump rising main 53 ft. | ASSUMED |
| 30 | Lawn | Lawn filling to +1'-2" is not included (A5); it belongs to landscaping. | ASSUMED |
| 31 | Steps | Steps (entrance, porches, rear and side doors) kept as the Rev 0 allowance: 40 cft brickwork + 20 cft PCC. | ASSUMED |
| 32 | Rates | Many rates are estimates (marked in the Rates sheet). Cement, steel, bricks, sand, crush and PPR rates come from 2026 published reports (sources listed). | ASSUMED |

# 11. Points for the architect and structural engineer to confirm

| No. | For | Point |
| --- | --- | --- |
| 1 | Engineer | Plinth beam steel and mix: sheet 05 shows size only. Confirm 2 #4 top + 2 #4 bottom, #2 @ 10", 1:2:4 (from S-02) for the beam at floor level. |
| 2 | Engineer | Eccentric plain brick footings on the right property line (house wall and powder-room wall) and under all boundary walls: confirm they are acceptable without the RCC footing of S-02, and whether the plinth beam should be tied to them. |
| 3 | Engineer | Car-porch corner column sits on the property line: its 3' x 3' footing must be placed eccentric (edge on the line). Confirm a tie to the plinth beam. |
| 4 | Architect | Roof drainage: the drawn outlets make the slope screed too thick at the far corners (build-up ~7¾" vs 6½"). Approve two more terrace outlets and one mumty spout. |
| 5 | Architect | Underground water tank is about 8½ ft from the two nearest rear manholes (less than 10 ft). Approve, or shift the rear-middle manhole about 2 ft to the right. |
| 6 | Architect + Engineer | Septic tank at the drawing size is too small for three practical chambers. Confirm the drawing size or the enlarged 7'-0" x 3'-6" tank. |
| 7 | Engineer | Sunken bath slabs (6"): ask the engineer to detail the bars at the drop (S-06 / S-07 show no drop). |
| 8 | Engineer | Upper-floor 3" PCC sub-base adds about 37 kg/m2. Confirm, or specify a lighter fill. |
| 9 | Engineer | Solar: confirm the roof slab can take the panels, frames and wind uplift on 34 plinths; place 4 #3 starter bars at each plinth position before the slab is cast. Confirm true north on site. |
| 10 | Architect + Engineer | Roof tank platform: confirm the two support walls sit over mumty walls at the sheet-24 tank position. |
| 11 | Architect | Car-porch size: sheet 04 shows 16'-7½" deep, other sheets 17'-9". Confirm (estimate uses 16'-7½", which matches the plot measurement). |
| 12 | Engineer | Ground-floor master bedroom 14'-3" (architect) vs 14'-0" (structural layout): ask the engineer to update S-01 so setting-out matches. |
| 13 | Architect | Second stair flight passes over the rear car porch with about 7 ft headroom: check car height. |

# Thumb-rule check, double counting and final check

## Thumb-rule sanity check

| Indicator | This estimate | Normal range | Result | Note |
| --- | --- | --- | --- | --- |
| Cement per sft of covered area | 0.38 bags | 0.35-0.45 bags | PASS |  |
| Steel per sft of covered area | 1.80 kg | 1.5-2.5 kg | PASS | load-bearing walls + RCC slabs (framed houses 3-4 kg) |
| Steel per m3 of RCC | 93 kg/m3 | 80-130 kg/m3 | PASS | all RCC members (2,692 cft of RCC) |
| Bricks per sft - house only | 26.1 | 22-30 | PASS | with wastage |
| Bricks per sft - all work | 32.4 | 28-38 | PASS | incl. boundary walls, manholes, tanks |
| Sand per sft (Chenab + Ravi) | 1.94 cft | 1.6-2.4 cft | PASS |  |
| Crush per sft | 0.97 cft | 0.8-1.2 cft | PASS |  |
| Material cost per sft | Rs 2,239 | Rs 2,000-2,700 | PASS | grey structure, material only, Faisalabad late 2026 (estimated range) |

Ranges are common Pakistani rules of thumb for a double-storey load-bearing brick house with RCC slabs, measured on covered area; they are a sanity check, not a design standard.

## Double-counting checks

- Foundation brickwork stops at road level; the plinth wall runs from road level to the underside of the plinth beam; the beam and the DPC are separate layers.

- Earth is counted once: 4,890 cft dug, 2,348 cft back-filled, 3,177 cft (loose) re-used in filling, 4,185 cft bought.

- Sub-floor layers add up exactly to the finished floor: fill to +1'-11½", khangar 4", sand 1", PCC 3", DPC 1½" = +2'-9".

- The upper-floor 3" sub-base excludes the sunken bath areas, which have their own brick-bat fill.

- Ceiling plaster is deleted; soffit plaster is a separate line; parapet and boundary-wall plaster are not inside the house-wall plaster.

- Roof tiles exclude the 34 solar plinths and the tank platform; membrane upturns are counted once.

- Slab steel is measured once by bar set; concealed beams, lintels and bed plates are separate members with their own bars.

- Binding wire is calculated once, on the total gross steel.

- Manhole, septic-tank and water-tank excavations are separate from the house trenches, and their back-filling is subtracted from the surplus earth.

## Final check

| No. | Check | Result | Evidence |
| --- | --- | --- | --- |
| 1 | All Register lines A1-K3 applied and logged | PASS | All 50 lines are in table 3.2 with how each is applied and the BOQ items where it is used; quantity changes are in table 3.1. |
| 2 | Every drawing sheet, room, wall, opening, slab, stair, tank, manhole and boundary wall is in the BOQ | PASS | Table 2.7 maps all 24 architect's sheets (plus 3D views and 5 bath-detail pages) and all 9 structural sheets to BOQ items; table 2.8 lists 3 slabs, 2 stairs, 58 lintels, 9 manholes, septic tank, underground tank, tank platform, 6 boundary-wall stretches and 6 edge walls. |
| 3 | Sheet 05 followed exactly, with only the DPC added | PASS | PCC 3" 1:4:8 at -4'-0"; brick steps and 13½" wall to road level; 9" plinth wall; 9" x 9" RCC plinth beam at floor level. The only addition is the DPC (B4, with the vertical DPC B6). The S-02 RCC strip footing is not used. |
| 4 | Arithmetic correct; Word and Excel totals identical | PASS | Excel recalculated: 4,415 formulas, 0 errors. Excel total (Totals sheet) Rs 9,189,795 = this report Rs 9,189,795 = take-off engine. Every figure in this report is read from the recalculated workbook; stage, steel and line totals reconcile exactly. One Rev 0 slip was found and corrected (manhole brickwork, table 3.1). |
| 5 | Proper construction order | PASS | BOQ stages S1-S16 follow the required order (Section 4); the purchase schedule (5.2) follows the same order. |
| 6 | Thumb-rule check passed | PASS | Cement 0.38 bags/sft, steel 1.80 kg/sft, house bricks 26/sft, Rs 2,239/sft - all within normal ranges (table above). |