House No. 79/K, WAPDA City, Faisalabad

FINISHING estimate - Step B: priced materials, no labour  |  Faisalabad / Lahore rates, Oct 2026

| Finishing materials Rs 18,592,007 12 site stages, wastage included | Per sft covered area Rs 4,530 on 4,104 sft | Grey + finishing Rs 27,781,802 grey Rev 1 Rs 9,189,795 | Combined per sft Rs 6,769 materials only |
| --- | --- | --- | --- |

# 1. Summary: cost by category (in site order)

| Stage | Category | Amount (Rs) | Share | Rs per sft |
| --- | --- | --- | --- | --- |
| 1 | Electrical first fix: wires, DBs and protection | 1,987,810 | 10.7% | 484 |
| 2 | Solar PV and battery storage | 2,058,133 | 11.1% | 501 |
| 3 | False ceilings | 631,000 | 3.4% | 154 |
| 4 | Floor and wall tiles: rooms, lobbies, kitchens, laundry | 1,242,930 | 6.7% | 303 |
| 5 | Marble and granite: stairs, sills, thresholds, external steps | 392,610 | 2.1% | 96 |
| 6 | Bathrooms | 2,229,045 | 12.0% | 543 |
| 7 | Kitchens and wardrobes | 1,929,865 | 10.4% | 470 |
| 8 | Doors | 1,812,950 | 9.8% | 442 |
| 9 | Windows, glazing, fly-nets and grills | 1,353,920 | 7.3% | 330 |
| 10 | Paint and colour scheme | 955,475 | 5.1% | 233 |
| 11 | Electrical second fix: switches, lights, fans, ACs, Wi-Fi, CCTV, EV charger | 3,401,739 | 18.3% | 829 |
| 12 | External works: paving, gates, railings, roof tank and pump | 596,530 | 3.2% | 145 |
|  | GRAND TOTAL - finishing materials | 18,592,007 | 100% | 4,530 |

- Material only: no labour, fabrication, fixing, transport or tools. Retail prices for Faisalabad / Lahore, October 2026; GST is included in the prices.

- Quantities are the Step A measurements with stated cutting/handling wastage, rounded UP to buying units (boxes, coils, bags, gallons, sheets, lengths).

- Every rate shows its source and date; 58% of the total (Rs 10,820,935) rests on rates marked "estimated" - get 2-3 quotations for those before buying.

- All 40 questionnaire answers are applied (Section 16). Optional items are listed in Section 15 and are NOT in the total.

- As instructed on 3 Oct 2026, this estimate is issued as this Word file only (no Excel workbook).

# 2. Basis and site order

## 2.1 Inputs

- Measurements: Finishing Step A (schedules of 35 rooms, 30 doors + arch, 17 windows + 7 ventilators, tiles, paint, false ceilings, electrical points, solar).

- Owner's answers to the 40 Step A questions (answered file of 3 Oct 2026); blank answers = the recommended default.

- Binding grey decisions (Rev 1): floor finish within 1 1/2" over the 3" PCC; no ceiling plaster (all ceilings are false ceilings); roof tiles, conduits, back boxes, earth pits, solar plinths, UPVC floor-trap bodies, PPR and gas pipes are already in the grey estimate and are not repeated here.

- Orientation: front (road) side taken as south, as in the grey solar layout. It drives the AC sizing and the panel direction.

## 2.2 Site order of the 12 stages

| Stage | Work | When on site |
| --- | --- | --- |
| 1 | Electrical first fix: wires, DBs and protection | Wires pulled into the grey conduits, DBs fitted and AC copper pipes run - before false ceilings and putty. |
| 2 | Solar PV and battery storage | Frames and panels on the grey plinths once the roof is clear; inverter and battery wired when the DBs are in; net-metering applied for at the end. |
| 3 | False ceilings | GI frame and gypsum boards after wiring and AC pipes, before wall putty and floor tiles. |
| 4 | Floor and wall tiles: rooms, lobbies, kitchens, laundry | After the ceilings and the first putty coat; skirting goes in with the floor tiles. |
| 5 | Marble and granite: stairs, sills, thresholds, external steps | Stairs and thresholds with the floor tiles; window sills before the aluminium frames. |
| 6 | Bathrooms | Waterproofing and concealed valves first, then tiles; sanitary ware, mirrors and screens after the final paint coats. |
| 7 | Kitchens and wardrobes | Cabinets and counters after the floor tiles; hobs, hoods and sinks last. |
| 8 | Doors | Diyar chowkats are fixed before plaster, so buy them first; shutters, hardware and polish after the floor tiles. |
| 9 | Windows, glazing, fly-nets and grills | Aluminium frames and glass after the first paint coats; grills before the final coats. |
| 10 | Paint and colour scheme | Putty and primer before tiling; final coats after doors, windows and kitchens; exterior and boundary walls last. |
| 11 | Electrical second fix: switches, lights, fans, ACs, Wi-Fi, CCTV, EV charger | After the final paint coats. |
| 12 | External works: paving, gates, railings, roof tank and pump | Last, so heavy deliveries do not damage the paving (tank and pump can be bought early for site water). |

Stages overlap on site. Each one is placed where its first material is needed. Buy the Diyar chowkats (Stage 8) and the DB boards (Stage 1) before the internal plaster, because they are fixed into the walls.

## 2.3 How quantities become purchases

- Net qty = measured quantity. Gross = net x (1 + wastage). Purchase = gross rounded UP to whole buying units. Amount = purchase x rate.

- Tile boxes: 2'x4' (600 x 1200) 2 pcs, 600 x 600 4 pcs and 300 x 600 8 pcs all hold 1.44 m2 = 15.5 sft per box.

- Cutting wastage: large floor tiles 8%, skirting strips, bath and wall tiles 10%, stone 10-20%, wire 10%, paint 5%, Diyar wood 25% (conversion and planing), gypsum boards 7-10%.

- Wire is bought in 90 m coils for 1.5, 2.5 and 4 mm2 (each colour separately); 6, 10 and 16 mm2 are bought as cut lengths.

# 3. Stage 1 - Electrical first fix: wires, DBs and protection

On site: Wires pulled into the grey conduits, DBs fitted and AC copper pipes run - before false ceilings and putty.

Scheme: wires ("taar") in the grey conduits; FESCO 3-phase supply, meter at the front; 4-core 16 mm2 armoured cable to the GF main DB (grid side) with a 63 A 4-pole MCCB, Type 2 surge protector and 30 mA RCCBs. The main DBs feed the ACs, 15 A power points, geysers, EV charger and the hybrid inverter's grid port. The inverter's load (backup) port feeds the GF, FF and mumty backup DBs (lights, fans, 5 A sockets, fridges, pump, Wi-Fi and cameras), as in grey Register J5. A manual 4-pole changeover lets the backup DBs run straight from the grid when the inverter is serviced.

Maximum demand with the AC sizes worked out in Stage 11: 36.5 kW (Step A 34.7 kW) = 58 A per phase at 400 V, 0.9 p.f. The 63 A incomer and 4 x 16 mm2 cable remain adequate. The hybrid geysers' electric elements are a winter load and do not coincide with the AC peak.

## 3.1 Circuit list by DB

| Circuit | No. | Points | Cable (live + neutral, earth) | Breaker | Phase | Route (m) |
| --- | --- | --- | --- | --- | --- | --- |
| GF main DB (grid side) |  |  |  |  |  |  |
| 15 A power points (RCD) | 5 | 9 in total | 4 + 2.5 mm2 earth | 20 A B | B, B, Y, B, Y | 85 |
| AC 1.5 t - Bed-1 | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | B | 15 |
| AC 1.5 t - Master bed | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | Y | 15 |
| AC 1.5 t - Dining | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | R | 15 |
| AC 2 t - TV lounge (less splayed corner 2 sft) | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | Y | 15 |
| AC 1.5 t - Drawing room | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | B | 15 |
| Geyser electric element (hybrid geyser) | 1 | 1 | 4 + 2.5 mm2 earth | 20 A B | Y | 15 |
| EV charger 7.4 kW | 1 | 1 | 6 + 6 mm2 earth | RCBO 40 A Type A | R | 12 |
| GF backup DB (inverter load port) |  |  |  |  |  |  |
| Lighting, fans and exhausts | 7 | 80 in total | 1.5 + 1.5 mm2 earth | 10 A B | R, Y, R, Y, R, Y, B | 366 |
| 5 A sockets (RCD) | 5 | 33 in total | 2.5 + 1.5 mm2 earth | 16 A B | R, Y, B, R, Y | 159 |
| External lighting + porch fan (photocell) | 2 | 27 lights in total | 1.5 + 1.5 mm2 earth | 10 A B | R, Y | 224 |
| External 5 A sockets, weatherproof (RCD) | 1 | 6 | 2.5 + 1.5 mm2 earth | 16 A B | B | 36 |
| Water pump 1 HP (underground tank) | 1 | 1 | 2.5 + 1.5 mm2 earth | 16 A C | B | 20 |
| Wi-Fi router, NVR, cameras | 1 | 1 | 2.5 + 1.5 mm2 earth | 10 A B | B | 15 |
| FF main DB |  |  |  |  |  |  |
| 15 A power points (RCD) | 3 | 6 in total | 4 + 2.5 mm2 earth | 20 A B | R, Y, B | 51 |
| AC 1.5 t - Bed-1 | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | R | 11 |
| AC 2 t - Master bed | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | R | 11 |
| AC 2 t - TV lounge | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | Y | 11 |
| AC 2 t - Front bed (less splayed corner 2 sft) | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | B | 11 |
| Geyser electric element (hybrid geyser) | 1 | 1 | 4 + 2.5 mm2 earth | 20 A B | Y | 15 |
| FF backup DB |  |  |  |  |  |  |
| Lighting, fans and exhausts | 7 | 77 in total | 1.5 + 1.5 mm2 earth | 10 A B | Y, B, Y, B, Y, B, Y | 358 |
| 5 A sockets (RCD) | 4 | 30 in total | 2.5 + 1.5 mm2 earth | 16 A B | R, Y, B, R | 138 |
| Mumty main DB |  |  |  |  |  |  |
| 15 A power point - room | 1 | 1 | 4 + 2.5 mm2 earth | 20 A B | B | 12 |
| 15 A power point - washing machine | 1 | 1 | 4 + 2.5 mm2 earth | 20 A B | B | 8 |
| AC 1.5 t - Room | 1 | 1 | 4 + 2.5 mm2 earth | 20 A C | B | 14 |
| Geyser electric element (hybrid geyser) | 1 | 1 | 4 + 2.5 mm2 earth | 20 A B | B | 15 |
| Mumty backup DB |  |  |  |  |  |  |
| Lighting, fans and exhausts | 2 | 15 in total | 1.5 + 1.5 mm2 earth | 10 A B | Y, Y | 77 |
| 5 A sockets (RCD) | 1 | 5 in total | 2.5 + 1.5 mm2 earth | 16 A B | Y | 27 |
| Sub-mains and feeders |  |  |  |  |  |  |
| GF main DB to FF main DB (3-phase) | 1 | - | 4 x 10 + 6 mm2 earth | 4P MCB 40 A | RYB | 9 |
| GF main DB to inverter grid port (mumty laundry) | 1 | - | 4 x 10 + 6 mm2 earth | 4P MCB 40 A | RYB | 15 |
| Inverter load port to GF backup DB | 1 | - | 4 x 10 + 6 mm2 earth | 4P MCB 40 A (in inverter AC box) | RYB | 15 |
| GF backup DB to FF backup DB (3-phase) | 1 | - | 4 x 6 + 6 mm2 earth | 4P MCB 32 A | RYB | 8 |
| FF main DB to mumty main DB (phase B) | 1 | - | 2 x 6 + 6 mm2 earth | DP MCB 32 A | B | 11 |
| FF backup DB to mumty backup DB (phase Y) | 1 | - | 2 x 4 + 2.5 mm2 earth | DP MCB 20 A | Y | 11 |
| FESCO meter to GF main DB | 1 | - | 4-core 16 mm2 armoured | MCCB 63 A 4-pole | RYB | 20 |
| Earth leads from the 3 grey earth pits | 3 | - | 16 mm2 green/yellow | - | - | 44 |
| Final circuits | 55 |  |  |  |  | 1,765 |

Route lengths: downlights 2.0 m, fans 5 m, exhausts and decorative points 4 m, switch-board loops 6 m, sockets 3 m + 12 m home run per circuit, 15 A circuits 17 m, ACs on the grey conduit runs (GF 45 ft, FF 30 ft, mumty 40 ft), 2-way strappers on stairs and bed-heads. Each route carries live + neutral + earth.

## 3.2 Wire by size and colour, in coils

Recommended brand: Pakistan Cables "registered" single-core PVC copper, 450/750 V (Fast Cables or Newage are equal alternatives, Q28). Colours: phases red / yellow / blue (each circuit keeps the colour of its phase, so a mixed-phase board is visible at once), neutral black, earth green. Quantities include 10% for loops, drops and terminations.

| Size | Red | Yellow | Blue | Black | Green | Total m (gross) | Buy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 mm2 | 324 m 4 coils | 605 m 7 coils | 226 m 3 coils | 1,156 m 13 coils | 1,562 m 18 coils | 3,874 | 45 coils x 90 m |
| 2.5 mm2 | 146 m 2 coils | 138 m 2 coils | 151 m 2 coils | 435 m 5 coils | 379 m 5 coils | 1,248 | 16 coils x 90 m |
| 4 mm2 | 59 m 1 coils | 146 m 2 coils | 174 m 2 coils | 379 m 5 coils | - | 758 | 10 coils x 90 m |
| 6 mm2 | 22 m | 9 m | 21 m | 34 m | 77 m | 160 | 163 m cut |
| 10 mm2 | 43 m | 43 m | 43 m | 43 m | - | 172 | 172 m cut |
| 16 mm2 | - | - | - | - | g/y 49 m | 49 | cut length |

## 3.3 DBs and protection

| DB | Board | Incomer | Earth-leakage protection | Outgoing ways |
| --- | --- | --- | --- | --- |
| GF main (grid side) | TPN 12-way, 36 modules | MCCB 63 A 4P + Type 2 SPD 4P | 4P RCCB 40 A 30 mA on 15 A power | ACs 5, power 5, geyser 1 (SP MCBs); EV RCBO 40 A Type A; 4P 40 A to FF main; 4P 40 A to inverter grid port |
| GF backup (inverter load port) | TPN 12-way, 36 modules | 4P MCB 40 A + 4P changeover 63 A (bypass) | 4P RCCB 40 A 30 mA on sockets | lighting 7, sockets 5, outdoor 3, pump 1, Wi-Fi/CCTV 1; 4P 32 A to FF backup |
| FF main | TPN 8-way, 24 modules | 4P MCB 40 A | 4P RCCB 40 A 30 mA on 15 A power | ACs 4, power 3, geyser 1; DP 32 A to mumty main |
| FF backup | TPN 8-way, 24 modules | 4P MCB 32 A | 4P RCCB 40 A 30 mA on sockets | lighting 7, sockets 4; DP 20 A to mumty backup |
| Mumty main (phase B) | SPN 8-way | DP MCB 32 A | 2P RCCB 40 A 30 mA | AC 1, room power 1, washing machine 1, geyser 1 |
| Mumty backup (phase Y) | SPN 8-way | DP MCB 20 A | 2P RCCB 40 A 30 mA | lighting 2, sockets 1 |

The grey estimate (Rev 1, item 14.D) priced smaller empty enclosures (12/8/4-way main, 8/6/4-way backup). They are too small for this circuit list, so the boards above replace them and the grey allowance is credited in line 1.32. Fix these enclosures before plaster. Breakers: Chint NXB / NXBLE or equal (Q28).

## 3.4 Priced materials - Stage 1

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.01 | Wire ("taar") 1.5 mm2 red - phase of lighting, fans, exhausts, bells (net 295 m) | 294.9 | m | 10% | 4 coils (90 m) = 360 m | 10,850 per coil | 43,400 | Pakistan Cables retail list, 01-09-2026 |
| 1.02 | Wire ("taar") 1.5 mm2 yellow - phase of lighting, fans, exhausts, bells (net 550 m) | 550.4 | m | 10% | 7 coils (90 m) = 630 m | 10,850 per coil | 75,950 | Pakistan Cables retail list, 01-09-2026 |
| 1.03 | Wire ("taar") 1.5 mm2 blue - phase of lighting, fans, exhausts, bells (net 206 m) | 205.7 | m | 10% | 3 coils (90 m) = 270 m | 10,850 per coil | 32,550 | Pakistan Cables retail list, 01-09-2026 |
| 1.04 | Wire ("taar") 1.5 mm2 black - neutral of lighting, fans, exhausts, bells (net 1,051 m) | 1,051 | m | 10% | 13 coils (90 m) = 1,170 m | 10,850 per coil | 141,050 | Pakistan Cables retail list, 01-09-2026 |
| 1.05 | Wire ("taar") 1.5 mm2 green - earth of lighting and socket circuits (net 1,420 m) | 1,420 | m | 10% | 18 coils (90 m) = 1,620 m | 10,850 per coil | 195,300 | Pakistan Cables retail list, 01-09-2026 |
| 1.06 | Wire ("taar") 2.5 mm2 red - phase of sockets, pump, Wi-Fi/CCTV (net 133 m) | 132.6 | m | 10% | 2 coils (90 m) = 180 m | 17,690 per coil | 35,380 | Pakistan Cables retail list, 01-09-2026 |
| 1.07 | Wire ("taar") 2.5 mm2 yellow - phase of sockets, pump, Wi-Fi/CCTV (net 125 m) | 125.1 | m | 10% | 2 coils (90 m) = 180 m | 17,690 per coil | 35,380 | Pakistan Cables retail list, 01-09-2026 |
| 1.08 | Wire ("taar") 2.5 mm2 blue - phase of sockets, pump, Wi-Fi/CCTV (net 137 m) | 137.3 | m | 10% | 2 coils (90 m) = 180 m | 17,690 per coil | 35,380 | Pakistan Cables retail list, 01-09-2026 |
| 1.09 | Wire ("taar") 2.5 mm2 black - neutral of sockets, pump, Wi-Fi/CCTV (net 395 m) | 395 | m | 10% | 5 coils (90 m) = 450 m | 17,690 per coil | 88,450 | Pakistan Cables retail list, 01-09-2026 |
| 1.10 | Wire ("taar") 2.5 mm2 green - earth of 4 mm2 circuits (net 344 m) | 344.3 | m | 10% | 5 coils (90 m) = 450 m | 17,690 per coil | 88,450 | Pakistan Cables retail list, 01-09-2026 |
| 1.11 | Wire ("taar") 4 mm2 red - phase of ACs, 15 A points, geysers, mumty backup feed (net 54 m) | 53.5 | m | 10% | 1 coil (90 m) = 90 m | 27,575 per coil | 27,575 | Pakistan Cables retail list, 01-09-2026 |
| 1.12 | Wire ("taar") 4 mm2 yellow - phase of ACs, 15 A points, geysers, mumty backup feed (net 133 m) | 133.1 | m | 10% | 2 coils (90 m) = 180 m | 27,575 per coil | 55,150 | Pakistan Cables retail list, 01-09-2026 |
| 1.13 | Wire ("taar") 4 mm2 blue - phase of ACs, 15 A points, geysers, mumty backup feed (net 158 m) | 157.8 | m | 10% | 2 coils (90 m) = 180 m | 27,575 per coil | 55,150 | Pakistan Cables retail list, 01-09-2026 |
| 1.14 | Wire ("taar") 4 mm2 black - neutral of ACs, 15 A points, geysers, mumty backup feed (net 344 m) | 344.3 | m | 10% | 5 coils (90 m) = 450 m | 27,575 per coil | 137,875 | Pakistan Cables retail list, 01-09-2026 |
| 1.15 | Wire ("taar") 6 mm2 red - phase of EV charger, backup and mumty sub-mains (net 20 m) | 19.5 | m | 10% | 22 m | 501 per m | 11,022 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.16 | Wire ("taar") 6 mm2 yellow - phase of EV charger, backup and mumty sub-mains (net 8 m) | 7.50 | m | 10% | 9 m | 501 per m | 4,509 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.17 | Wire ("taar") 6 mm2 blue - phase of EV charger, backup and mumty sub-mains (net 18 m) | 18.5 | m | 10% | 21 m | 501 per m | 10,521 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.18 | Wire ("taar") 6 mm2 black - neutral of EV charger, backup and mumty sub-mains (net 30 m) | 30.5 | m | 10% | 34 m | 501 per m | 17,034 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.19 | Wire ("taar") 6 mm2 green - earth of EV charger and sub-mains (net 70 m) | 69.5 | m | 10% | 77 m | 501 per m | 38,577 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.20 | Wire ("taar") 10 mm2 red - phase of 3-phase sub-mains and inverter (net 39 m) | 39 | m | 10% | 43 m | 866 per m | 37,238 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.21 | Wire ("taar") 10 mm2 yellow - phase of 3-phase sub-mains and inverter (net 39 m) | 39 | m | 10% | 43 m | 866 per m | 37,238 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.22 | Wire ("taar") 10 mm2 blue - phase of 3-phase sub-mains and inverter (net 39 m) | 39 | m | 10% | 43 m | 866 per m | 37,238 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.23 | Wire ("taar") 10 mm2 black - neutral of 3-phase sub-mains and inverter (net 39 m) | 39 | m | 10% | 43 m | 866 per m | 37,238 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 1.24 | Earth leads 16 mm2 green/yellow from the 3 grey earth pits to the DB and inverter (net 44 m) | 44.1 | m | 10% | 49 m | 1,385 per m | 67,865 | derived from Pakistan Cables list 01-09-2026 |
| 1.25 | Incomer: FESCO meter (front) to GF main DB, 4-core 16 mm2 armoured | 20 | m | 5% | 21 m | 5,800 per m | 121,800 | estimated (Pakistan Cables per-mm2 price + armour) |
| 1.26 | Cat6 to 4 Wi-Fi nodes and 7 cameras (grey data conduits, +20% loops) - 208 m | 208.5 | m | - | 1 box (305 m) = 305 m | 33,550 per box | 33,550 | w11stop.com, 21-07-2026 |
| 1.27 | RJ45 connectors (cameras, nodes, patching) | 40 | nos | - | 1 pack (100) = 100 nos | 2,800 per pack | 2,800 | estimated |
| 1.28 | Cat6 keystone jack with 1-gang data plate | 6 | nos | - | 6 nos | 650 per nos | 3,900 | estimated |
| 1.29 | Distribution board 3-phase TPN 12-way (36 modules), metal, flush, with busbars and N/E bars | 2 | nos | - | 2 nos | 15,500 per nos | 31,000 | estimated |
| 1.30 | Distribution board 3-phase TPN 8-way (24 modules), metal, flush | 2 | nos | - | 2 nos | 11,500 per nos | 23,000 | estimated |
| 1.31 | Distribution board single-phase SPN 8-way, metal, flush | 2 | nos | - | 2 nos | 3,800 per nos | 7,600 | estimated |
| 1.32 | LESS: empty DB enclosures already priced in grey Rev 1 (item 14.D: 12/8/8/6/4/4-way) - replaced by the boards above | 1 | set | - | 1 set | -17,700 per set | -17,700 | grey estimate Rev 1 rates |
| 1.33 | MCCB 4-pole 63 A, 25 kA (Chint NXM or equal) - main incomer | 1 | nos | - | 1 nos | 14,500 per nos | 14,500 | estimated |
| 1.34 | MCB 4-pole 40 A C-curve (Chint NXB) | 4 | nos | - | 4 nos | 3,400 per nos | 13,600 | estimated (Chint TP Rs 2,500-2,890, w11stop Jul 2026) |
| 1.35 | MCB 4-pole 32 A C-curve (Chint NXB) | 2 | nos | - | 2 nos | 3,300 per nos | 6,600 | estimated (Chint TP Rs 2,500-2,890, w11stop Jul 2026) |
| 1.36 | MCB double-pole 20-32 A (Chint) | 4 | nos | - | 4 nos | 1,720 per nos | 6,880 | w11stop.com, Jul 2026 |
| 1.37 | MCB single-pole 6-20 A, B/C-curve (Chint NXB) | 54 | nos | - | 54 nos | 1,090 per nos | 58,860 | w11stop.com, Jul 2026 |
| 1.38 | RCCB 4-pole 40 A, 30 mA (Chint NXBLE) | 4 | nos | - | 4 nos | 6,200 per nos | 24,800 | estimated (Chint 4P 25 A Rs 4,950, powerhouseexpress 2026) |
| 1.39 | RCCB 2-pole 40 A, 30 mA (Chint) | 2 | nos | - | 2 nos | 3,800 per nos | 7,600 | estimated |
| 1.40 | RCBO 40 A Type A, 30 mA (EV charger circuit) | 1 | nos | - | 1 nos | 7,500 per nos | 7,500 | estimated |
| 1.41 | Surge protector Type 2, 4-pole (3P+N) 40 kA, 385 V AC (Chint NU6-II) | 1 | nos | - | 1 nos | 9,500 per nos | 9,500 | estimated within Chint SPD list range Rs 3,500-11,000 |
| 1.42 | Changeover switch 4-pole 63 A, manual I-0-II (inverter bypass) | 1 | nos | - | 1 nos | 9,000 per nos | 9,000 | estimated |
| 1.43 | Isolator 40 A double-pole for the EV charger (in the grey EV box) | 1 | nos | - | 1 nos | 3,500 per nos | 3,500 | estimated |
| 1.44 | DB sundries: comb busbars, links, DIN rail, blanks, labels, lugs, glands, ferrules, ties | 1 | set | - | 1 set | 18,000 per set | 18,000 | estimated |
| 1.45 | AC pipe set 1.5 t: concealed copper pair with insulation, drain hose, interconnect cable, outdoor bracket (about 15 ft beyond the supplied kit) - 6 units | 6 | set | - | 6 sets | 24,000 per set | 144,000 | estimated |
| 1.46 | AC pipe set 2 t: as above, larger copper pair - 4 units | 4 | set | - | 4 sets | 28,000 per set | 112,000 | estimated |
|  | Stage 1 total (purchase quantities x rates) |  |  |  |  |  | 1,987,810 |  |

# 4. Stage 2 - Solar PV and battery storage

On site: Frames and panels on the grey plinths once the roof is clear; inverter and battery wired when the DBs are in; net-metering applied for at the end.

| Panels | 23 x 650 W N-type bifacial, tier-1 (Longi Hi-MO X10 / Jinko Tiger Neo / JA) = 14.95 kWp; two strings of 12 + 11; facing the road side (south), tilt 15 deg. |
| --- | --- |
| Mounting structure | Galvanised steel tables bolted to the 34 grey RCC plinths (2 cast-in M12 J-bolts each) with base plates; layout as in grey Section 7. |
| Hybrid inverter | 1 x 12 kW three-phase hybrid, 48 V battery, 2 MPPT (Deye SUN-12K-SG04LP3 class). DC/AC ratio 1.25. Grid port to the GF main DB; load port to the backup DBs. |
| Battery (Q33) | Sodium-ion, 48 V class: 2 modules of about 5.3 kWh = about 10.6 kWh now, in a 3-slot rack with busbars sized for 3 modules; add the 3rd module later for about 16 kWh. Sizing (Step A): essential night load 8.07 kWh -> 9.7 kWh needed; with one bedroom AC 12.97 kWh -> 15.7 kWh. |
| DC side | 6 mm2 DC solar cable red/black, MC4 connectors, DC combiner box with 1,000 V isolator, Type 2 DC surge protector and gPV fuses. |
| AC side and protection | Inverter AC box (grid and load 4P 40 A MCBs, AC surge protector); 10 mm2 sub-mains are in Stage 1; frames bonded to the solar earth pit (grey J7). |
| FESCO net-metering | 3-phase bi-directional meter and application/testing allowance. Under the NEPRA Prosumer Regulations 2026 exported units are credited at the net-billing rate (about Rs 10-11 per kWh), so the design favours self-use and battery storage. |

Sodium-ion packs are new in Pakistan and no retail price was found: Rs 55,000 per kWh is used (about 23% above branded LFP). The pack's BMS must be on the inverter's approved list. If no compatible sodium-ion pack is available, a 10 kWh LFP pack (Rs 447,200, w11stop.com, Jul 2026) is the fall-back and saves about Rs 135,800.

## 4.1 Priced materials - Stage 2

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.01 | Solar panel 650 W N-type bifacial, tier-1 (Longi Hi-MO X10 / Jinko / JA) at Rs 44/W - 23 nos = 14.95 kWp | 23 | nos | - | 23 nos | 28,600 per nos | 657,800 | icons.com.pk solar prices, 01-04-2026 (Longi Hi-MO X10 Rs 44-49.25/W) |
| 2.02 | Hybrid inverter 12 kW 3-phase, 48 V battery, 2 MPPT (Deye SUN-12K-SG04LP3 or equal) | 1 | nos | - | 1 nos | 380,000 per nos | 380,000 | paksolarpanelprices.com, 2026 (Rs 320,000-420,000) |
| 2.03 | Sodium-ion battery module 48 V class, about 5.3 kWh, BMS with CAN/RS485 (Rs 55,000/kWh) - 2 now (about 10.6 kWh); a 3rd module later gives about 16 kWh | 2 | nos | - | 2 nos | 291,500 per nos | 583,000 | estimated (no retail listing; LFP 10 kWh Rs 447,200, w11stop 21-07-2026) |
| 2.04 | Battery rack/cabinet for 3 modules, parallel busbars and communication cables | 1 | set | - | 1 set | 25,000 per set | 25,000 | estimated |
| 2.05 | Battery DC breaker 250 A + 2 x 70 mm2 flexible cables with lugs | 1 | set | - | 1 set | 18,000 per set | 18,000 | estimated |
| 2.06 | Mounting frame per panel, galvanised steel, 15 deg tilt (GI frame) | 23 | nos | - | 23 nos | 8,500 per nos | 195,500 | w11stop.com, Jul 2026 (GI mounting frame) |
| 2.07 | Base plate, nuts and washers per plinth (on the grey cast-in J-bolts) | 34 | nos | - | 34 nos | 1,200 per nos | 40,800 | estimated |
| 2.08 | DC solar cable 6 mm2 red - 2 strings x 40 m (net 80 m) | 80 | m | 10% | 1 coil (90 m) = 90 m | 29,500 per coil | 29,500 | w11stop.com, 2026 (GM DC 6 mm2, 90 m coil Rs 29,500) |
| 2.09 | DC solar cable 6 mm2 black - 2 strings x 40 m (net 80 m) | 80 | m | 10% | 1 coil (90 m) = 90 m | 29,500 per coil | 29,500 | w11stop.com, 2026 (GM DC 6 mm2, 90 m coil Rs 29,500) |
| 2.10 | MC4 connector pair (IP67) | 10 | pair | - | 10 pairs | 450 per pair | 4,500 | estimated |
| 2.11 | PV DC combiner box: 1,000 V DC isolator, Type 2 DC surge protector, gPV fuses (2 strings) | 1 | nos | - | 1 nos | 22,000 per nos | 22,000 | estimated (Chint DC SPD Rs 3,500-11,000) |
| 2.12 | Inverter AC box: grid and load 4-pole 40 A MCBs, AC surge protector | 1 | nos | - | 1 nos | 16,000 per nos | 16,000 | estimated |
| 2.13 | Frame bonding 6 mm2 green to the solar earth pit (net 30 m) | 30 | m | 10% | 33 m | 501 per m | 16,533 | Pakistan Cables list 01-09-2026 (per m, +10% cut length) |
| 2.14 | FESCO 3-phase bi-directional (net-billing) meter | 1 | nos | - | 1 nos | 25,000 per nos | 25,000 | FESCO / NEPRA Prosumer Regulations 2026 (meter Rs 10,000-25,000) |
| 2.15 | FESCO net-billing application, meter testing and connection allowance | 1 | set | - | 1 set | 15,000 per set | 15,000 | estimated |
|  | Stage 2 total (purchase quantities x rates) |  |  |  |  |  | 2,058,133 |  |

# 5. Stage 3 - False ceilings

On site: GI frame and gypsum boards after wiring and AC pipes, before wall putty and floor tiles.

System (Q27): 12.5 mm gypsum board on a GI frame (ceiling sections at 600 mm on intermediate channels at 1,200 mm, perimeter channel, hangers to the slab); moisture-resistant (green) board in the 7 baths and the laundry. Cove boxes with LED strips and recessed profile lights only in the drawing room, both TV lounges and both master bedrooms. Ceiling height 10'-0" in GF/FF rooms, 8'-6" in wet areas and the mumty.

| Id | Room | Height | Board | Area (sft) | Cove (rft) | Lights |
| --- | --- | --- | --- | --- | --- | --- |
| Ground floor |  |  |  |  |  |  |
| G01 | Stair lobby (S-T lobby) | 10'-0" | standard | 40.6 | - |  |
| G02 | Lobby (5'-0" wide) | 10'-0" | standard | 77.5 | - |  |
| G03 | Bed-1 | 10'-0" | standard | 169.0 | - |  |
| G04 | Bed-1 dress | 10'-0" | standard | 20.0 | - |  |
| G05 | Bed-1 bath (incl. shower area) | 8'-6" | moisture-resistant | 45.0 | - |  |
| G06 | Master bed | 10'-0" | standard | 171.0 | 52.5 | cove + profiles |
| G07 | Master dress | 10'-0" | standard | 22.5 | - |  |
| G08 | Master bath (incl. shower area) | 8'-6" | moisture-resistant | 67.7 | - |  |
| G09 | Dining | 10'-0" | standard | 86.6 | - |  |
| G10 | Powder room (P-W) | 8'-6" | moisture-resistant | 18.0 | - |  |
| G11 | Open kitchen | 10'-0" | standard | 160.1 | - |  |
| G12 | Raw kitchen | 10'-0" | standard | 77.0 | - |  |
| G13 | TV lounge (less splayed corner 2 sft) | 10'-0" | standard | 177.8 | 54.0 | cove + profiles |
| G14 | Drawing room | 10'-0" | standard | 156.0 | 50.0 | cove + profiles |
| G15 | Entrance foyer | 10'-0" | standard | 37.5 | - |  |
| G16 | Under-stair store (S-A) | 10'-0" | standard | 17.5 | - |  |
| First floor |  |  |  |  |  |  |
| F01 | Stair hall (S-T lobby) | 10'-0" | standard | 51.6 | - |  |
| F02 | Lobby, rear part (5'-0" wide) | 10'-0" | standard | 62.5 | - |  |
| F03 | Lobby, front part (6'-3" wide) | 10'-0" | standard | 75.0 | - |  |
| F04 | Bed-1 | 10'-0" | standard | 156.0 | - |  |
| F05 | Bed-1 dress | 10'-0" | standard | 24.0 | - |  |
| F06 | Bed-1 bath (incl. shower area) | 8'-6" | moisture-resistant | 54.0 | - |  |
| F07 | Master bed | 10'-0" | standard | 199.5 | 56.5 | cove + profiles |
| F08 | Master dress | 10'-0" | standard | 28.7 | - |  |
| F09 | Master bath (incl. shower area) | 8'-6" | moisture-resistant | 51.0 | - |  |
| F10 | TV lounge | 10'-0" | standard | 201.0 | 56.8 | cove + profiles |
| F11 | Store (with wardrobes) | 10'-0" | standard | 78.0 | - |  |
| F12 | Kitchen | 10'-0" | standard | 104.0 | - |  |
| F13 | Front bed (less splayed corner 2 sft) | 10'-0" | standard | 213.1 | - |  |
| F14 | Front bed dress | 10'-0" | standard | 31.5 | - |  |
| F15 | Front bed bath (incl. shower area) | 8'-6" | moisture-resistant | 60.0 | - |  |
| Second floor (mumty) |  |  |  |  |  |  |
| S01 | Stair hall | 8'-6" | standard | 111.1 | - |  |
| S02 | Room | 8'-6" | standard | 132.0 | - |  |
| S03 | Bath | 8'-6" | moisture-resistant | 26.1 | - |  |
| S04 | Laundry (open on terrace side) | 8'-6" | moisture-resistant | 33.3 | - |  |
|  | Standard board ceilings |  |  | 2,681.1 |  |  |
|  | Moisture-resistant board ceilings |  |  | 355.1 |  |  |
|  | All false ceilings (= Step A) |  |  | 3,036.2 | 269.8 |  |

## 5.1 Priced materials - Stage 3

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.01 | Gypsum board 12.5 mm standard: 2,681 sft ceilings + 472 sft cove boxes (270 rft x 1.75 ft) | 98.5 | sheet | 7% | 106 sheets | 2,450 per sheet | 259,700 | estimated |
| 3.02 | Gypsum board 12.5 mm moisture-resistant: baths, powder room, laundry 355 sft | 11.1 | sheet | 10% | 13 sheets | 3,450 per sheet | 44,850 | estimated |
| 3.03 | Ceiling section at 600 mm (1.7 m per m2) + cove framing 2.5 m per m | 685.1 | m | 5% | 197 lengths (3.66 m) = 721.0 m | 520 per length | 102,440 | estimated |
| 3.04 | Intermediate channel at 1,200 mm (0.85 m per m2) | 239.8 | m | 5% | 69 lengths (3.66 m) = 252.5 m | 650 per length | 44,850 | estimated |
| 3.05 | Perimeter channel on all room walls (368 m) | 367.8 | m | 5% | 106 lengths (3.66 m) = 388.0 m | 390 per length | 41,340 | estimated |
| 3.06 | Hangers at 1.2 m x 1.2 m (0.7 per m2); drops 12-30 in | 197.4 | nos | 5% | 208 nos | 140 per nos | 29,120 | estimated |
| 3.07 | Anchor fasteners: hangers + perimeter channel at 600 mm | 822.7 | nos | 5% | 9 boxes (100) = 900 nos | 1,600 per box | 14,400 | estimated |
| 3.08 | Connecting clips | 564.1 | nos | 5% | 6 bags (100) = 600 nos | 1,500 per bag | 9,000 | estimated |
| 3.09 | Board screws, 14 per m2 | 4,562.9 | nos | 10% | 6 boxes (1,000) = 6,000 nos | 1,900 per box | 11,400 | estimated |
| 3.10 | Frame screws, 4 per m2 | 1,128.3 | nos | 10% | 2 boxes (1,000) = 2,000 nos | 1,500 per box | 3,000 | estimated |
| 3.11 | Jointing compound 0.35 kg per m2 | 114.1 | kg | 10% | 6 bags (25 kg) = 150 kg | 3,600 per bag | 21,600 | estimated |
| 3.12 | Joint tape 1.5 m per m2 + 2 m per m of cove | 587.5 | m | 10% | 9 rolls (75 m) = 675 m | 450 per roll | 4,050 | estimated |
| 3.13 | Edge bead on cove lips (2 lines per cove) | 55 | nos | - | 55 nos | 250 per nos | 13,750 | estimated |
| 3.14 | Access panels: 7 baths, laundry, 1 spare (AC / junctions) | 9 | nos | - | 9 nos | 3,500 per nos | 31,500 | estimated |
|  | Stage 3 total (purchase quantities x rates) |  |  |  |  |  | 631,000 |  |

# 6. Stage 4 - Floor and wall tiles: rooms, lobbies, kitchens, laundry

On site: After the ceilings and the first putty coat; skirting goes in with the floor tiles.

Floor ("farsh") tiles (Q1, Q2, Q3, Q6, Q7): one 2'x4' polished light marble-look porcelain everywhere except wet areas; the same series in matt in the 3 kitchens; 4" skirting cut from the floor tile; 600 x 600 matt anti-slip in baths and laundry. Setting: 1" semi-dry 1:4 bed with a polymer-modified C2TE adhesive bond coat (back-buttering on 2'x4'), levelling clips, 2 mm joints with polymer grout. This fits the 1 1/2" floor-finish allowance over the grey PCC.

## 6.1 Tile schedule per area (all tile areas in the house)

| Area | Tile | Net (sft) | Cutting waste | Gross (sft) | sft/box | Boxes | Priced in |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ground floor: rooms, lobbies, dresses, store, hall landing | 2'x4' polished | 976.0 | 8% | 1,054.1 | 15.5 | 69 | Stage 4 |
| Ground floor: 4" skirting 304 rft | 2'x4' polished (strips) | 119.7 | 10% | 131.7 | 15.5 | 9 | Stage 4 |
| First floor: rooms, lobbies, dresses, store, hall landing | 2'x4' polished | 1,122.3 | 8% | 1,212.1 | 15.5 | 79 | Stage 4 |
| First floor: 4" skirting 321 rft | 2'x4' polished (strips) | 126.4 | 10% | 139.0 | 15.5 | 9 | Stage 4 |
| Second floor (mumty): rooms, lobbies, dresses, store, hall landing | 2'x4' polished | 168.8 | 8% | 182.3 | 15.5 | 12 | Stage 4 |
| Second floor (mumty): 4" skirting 52 rft | 2'x4' polished (strips) | 20.3 | 10% | 22.3 | 15.5 | 2 | Stage 4 |
| Kitchen floor - Open kitchen | 2'x4' matt | 160.1 | 8% | 172.9 | 15.5 | 12 | Stage 4 |
| Kitchen floor - Raw kitchen | 2'x4' matt | 77.0 | 8% | 83.2 | 15.5 | 6 | Stage 4 |
| Kitchen floor - Kitchen | 2'x4' matt | 104.0 | 8% | 112.3 | 15.5 | 8 | Stage 4 |
| Kitchen skirting 44 rft | 2'x4' matt (strips) | 17.5 | 10% | 19.2 | 15.5 | 2 | Stage 4 |
| Laundry floor | 600 x 600 matt anti-slip | 33.3 | 10% | 36.6 | 15.5 | 3 | Stage 4 |
| Laundry dado 4 ft | 300 x 600 glazed | 46.5 | 10% | 51.2 | 15.5 | 4 | Stage 4 |
| Bed-1 bath (incl. shower area) - floor | 600 x 600 matt anti-slip | 45.0 | 10% | 49.5 | 15.5 | 4 | Stage 6 |
| Master bath (incl. shower area) - floor | 600 x 600 matt anti-slip | 67.7 | 10% | 74.4 | 15.5 | 5 | Stage 6 |
| Powder room (P-W) - floor | 600 x 600 matt anti-slip | 18.0 | 10% | 19.8 | 15.5 | 2 | Stage 6 |
| Bed-1 bath (incl. shower area) - floor | 600 x 600 matt anti-slip | 54.0 | 10% | 59.4 | 15.5 | 4 | Stage 6 |
| Master bath (incl. shower area) - floor | 600 x 600 matt anti-slip | 51.0 | 10% | 56.1 | 15.5 | 4 | Stage 6 |
| Front bed bath (incl. shower area) - floor | 600 x 600 matt anti-slip | 60.0 | 10% | 66.0 | 15.5 | 5 | Stage 6 |
| Bath - floor | 600 x 600 matt anti-slip | 26.1 | 10% | 28.7 | 15.5 | 2 | Stage 6 |
| Bed-1 bath (incl. shower area) - walls (plain 170 + feature 42) | 300 x 600 glazed | 213.0 | 10% | 234.3 | 15.5 | 16 | Stage 6 |
| Master bath (incl. shower area) - walls (plain 191 + feature 65) | 300 x 600 glazed | 255.5 | 10% | 281.1 | 15.5 | 19 | Stage 6 |
| Powder room (P-W) - walls (plain 88 + feature 34) | 300 x 600 glazed | 121.5 | 10% | 133.7 | 15.5 | 9 | Stage 6 |
| Bed-1 bath (incl. shower area) - walls (plain 179 + feature 51) | 300 x 600 glazed | 230.0 | 10% | 253.0 | 15.5 | 17 | Stage 6 |
| Master bath (incl. shower area) - walls (plain 165 + feature 54) | 300 x 600 glazed | 219.4 | 10% | 241.3 | 15.5 | 16 | Stage 6 |
| Front bed bath (incl. shower area) - walls (plain 196 + feature 51) | 300 x 600 glazed | 247.0 | 10% | 271.7 | 15.5 | 18 | Stage 6 |
| Bath - walls (plain 109 + feature 40) | 300 x 600 glazed | 149.2 | 10% | 164.2 | 15.5 | 11 | Stage 6 |
| Open kitchen - backsplash 2'-0" | 300 x 600 glazed | 23.8 | 10% | 26.1 | 15.5 | 2 | Stage 7 |
| Raw kitchen - backsplash 2'-0" | 300 x 600 glazed | 33.2 | 10% | 36.6 | 15.5 | 3 | Stage 7 |
| Kitchen - backsplash 2'-0" | 300 x 600 glazed | 39.2 | 10% | 43.2 | 15.5 | 3 | Stage 7 |

Box counts per area are for issuing tiles on site; each purchase line rounds the whole quantity once. Skirting: 5 strips of 4" are cut from each 600 x 1200 tile (19.7 rft per tile). Stairs and thresholds are stone (Stage 5); porches, passages and ramps are tuff tiles (Stage 12).

## 6.2 Priced materials - Stage 4

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.01 | 2'x4' polished porcelain - rooms, lobbies, dresses, store, stair-hall landings 2,267.2 sft (8% cutting) | 2,267.2 | sft | 8% | 158 boxes = 2,449 sft | 320 per sft | 783,680 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 4.02 | 2'x4' polished porcelain cut into 4" skirting strips - 677 rft (5 strips per tile = 266.4 sft) | 266.4 | sft | 10% | 19 boxes = 294.5 sft | 320 per sft | 94,240 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 4.03 | 2'x4' matt porcelain - 3 kitchens 341.1 sft | 341.1 | sft | 8% | 24 boxes = 372 sft | 320 per sft | 119,040 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 4.04 | 2'x4' matt porcelain cut into 4" skirting - 44 rft | 17.5 | sft | 10% | 2 boxes = 31 sft | 320 per sft | 9,920 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 4.05 | 600 x 600 matt anti-slip - laundry floor 33.3 sft | 33.3 | sft | 10% | 3 boxes = 46.5 sft | 260 per sft | 12,090 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 4.06 | 300 x 600 glazed - laundry dado 4 ft on two walls 46.5 sft | 46.5 | sft | 10% | 4 boxes = 62 sft | 230 per sft | 14,260 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 4.07 | Cement for 1" semi-dry 1:4 bed under 2,642 sft of floor tiles (1.7 bags per 100 sft) | 44.7 | bag | 5% | 47 bags | 1,560 per bag | 73,320 | grey estimate Rev 1 (ARY News 15-08-2026 and brand list Sep 2026) |
| 4.08 | Ravi sand for the same bed (8.5 cft per 100 sft) | 223.7 | cft | 10% | 247 cft | 40 per cft | 9,880 | grey estimate Rev 1 (estimated) |
| 4.09 | Tile adhesive: floors 0.30 kg/sft (bond coat, back-buttered), skirting and dado 0.35 kg/sft | 908.1 | kg | 5% | 48 bags (20 kg) = 960 kg | 1,450 per bag | 69,600 | estimated |
| 4.10 | Grout 2 mm joints: 0.015 kg/sft (2'x4'), 0.02 (600 x 600), 0.03 (skirting, dado) | 49.7 | kg | 10% | 55 kg | 450 per kg | 24,750 | estimated |
| 4.11 | Levelling clips: 0.4 per sft (2'x4'), 0.5 per sft (600 x 600) | 1,060.0 | nos | 5% | 12 bags (100) = 1,200 nos | 1,200 per bag | 14,400 | estimated |
| 4.12 | Levelling wedges, reusable (2 bags) | 200 | nos | - | 2 bags (100) = 200 nos | 1,500 per bag | 3,000 | estimated |
| 4.13 | Spacers for dado and skirting | 77.9 | nos | - | 1 bag (100) = 100 nos | 150 per bag | 150 | estimated |
| 4.14 | Edge trim on the laundry dado top | 2 | nos | - | 2 nos | 650 per nos | 1,300 | estimated |
| 4.15 | Waterproofing under laundry tiles: floor + 1 ft upturn = 45 sft at 3 kg/m2 (2 coats) | 12.6 | kg | 10% | 1 set (25 kg) = 25 kg | 11,500 per set | 11,500 | estimated |
| 4.16 | Floor-trap grating, laundry | 1 | nos | - | 1 nos | 1,800 per nos | 1,800 | estimated |
|  | Stage 4 total (purchase quantities x rates) |  |  |  |  |  | 1,242,930 |  |

# 7. Stage 5 - Marble and granite: stairs, sills, thresholds, external steps

On site: Stairs and thresholds with the floor tiles; window sills before the aluminium frames.

Q4 and Q38: granite treads with three anti-slip grooves (cut by the stone supplier) and white Ziarat marble risers and stepped skirting; granite 20 mm (Tropical / Black Galaxy class) for window sills, door thresholds and the external steps. Vanity tops (Stage 6) and kitchen counters (Stage 7) are also granite and are listed here for completeness only.

| Item | Stone | Size / members | Net (sft) | Waste | Priced in |
| --- | --- | --- | --- | --- | --- |
| Stair treads, 42 nos | Granite 20 mm, grooved | 11 1/2" x 3'-6" (10 1/2" going + 1" nosing) | 140.9 | 10% | Stage 5 |
| Mid landings, 2 nos | Granite 20 mm | 8'-3" x 3'-6" | 57.8 | 10% | Stage 5 |
| Stair risers, 44 nos | White marble 20 mm | 6.57" x 3'-6" | 84.3 | 10% | Stage 5 |
| Stepped skirting | White marble 20 mm | 85 rft x 4" | 28.3 | 20% | Stage 5 |
| External steps (4 sets) | Granite 20 mm | treads 13" deep + risers 6" | 87.5 | 10% | Stage 5 |
| Window and vent sills, 23 nos | Granite 20 mm | W01, W02, W03, W04, W05, W06, W08 x2, W09, W10, W11, W12, W13, W14 x2, W15, V01, V02, V03, V04, V05, V06, V07 | 98.9 | 15% | Stage 5 |
| Door thresholds, 12 nos | Granite 20 mm | D01, D02, D06, D08, D10, D12, D18, D21, D24, D27, D28, D30 | 28.0 | 15% | Stage 5 |
| Vanity tops, 6 nos | Granite 20 mm | 3'-0" x 1'-10" + 3" upstand | 37.5 | 15% | Stage 6 |
| Kitchen counters + island | Granite 20 mm | 46.2 rft x 2'-1" + island 5'-0" x 2'-6" + edge strips | 118.0 | 15% | Stage 7 |

## 7.1 Priced materials - Stage 5

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.01 | Granite treads with 3 anti-slip grooves (cut by the supplier) - 42 treads 11 1/2" x 3'-6" + 2 mid landings ("paidan") | 198.6 | sft | 10% | 219 sft | 650 per sft | 142,350 | estimated (zameen.com: Jet Black ~Rs 400, Snow White ~Rs 570/sft) |
| 5.02 | White marble risers ("uthan") - 44 risers 6.57" x 3'-6" | 84.3 | sft | 10% | 93 sft | 500 per sft | 46,500 | icons.com.pk marble prices, 10-09-2026 (Rs 480-520/sft) |
| 5.03 | White marble 4" stepped skirting, both sides of both flights - 85 rft | 28.3 | sft | 20% | 34 sft | 500 per sft | 17,000 | icons.com.pk marble prices, 10-09-2026 (Rs 480-520/sft) |
| 5.04 | External steps (4 sets, 14 risers): granite treads with nosing + granite risers | 87.5 | sft | 10% | 97 sft | 650 per sft | 63,050 | estimated (zameen.com: Jet Black ~Rs 400, Snow White ~Rs 570/sft) |
| 5.05 | Window and ventilator sills 11" deep with 2" horns - 23 openings | 98.9 | sft | 15% | 114 sft | 650 per sft | 74,100 | estimated (zameen.com: Jet Black ~Rs 400, Snow White ~Rs 570/sft) |
| 5.06 | Door thresholds ("dehleez"): 7 bath doors, D01, D02, D12, D28 and sliding door D27 | 28.0 | sft | 15% | 33 sft | 650 per sft | 21,450 | estimated (zameen.com: Jet Black ~Rs 400, Snow White ~Rs 570/sft) |
| 5.07 | Cement for 1" 1:4 bedding of all stone (526 sft) | 8.90 | bag | 5% | 10 bags | 1,560 per bag | 15,600 | grey estimate Rev 1 (ARY News 15-08-2026 and brand list Sep 2026) |
| 5.08 | Ravi sand ("ret") for bedding mortar | 44.5 | cft | 10% | 49 cft | 40 per cft | 1,960 | grey estimate Rev 1 (estimated) |
| 5.09 | White cement for stone joints, 40 kg bag | 1 | bag | - | 1 bag | 3,600 per bag | 3,600 | estimated |
| 5.10 | Stone sealer (granite / marble), 1 litre | 2 | litre | - | 2 litres | 3,500 per litre | 7,000 | estimated |
|  | Stage 5 total (purchase quantities x rates) |  |  |  |  |  | 392,610 |  |

# 8. Stage 6 - Bathrooms

On site: Waterproofing and concealed valves first, then tiles; sanitary ware, mirrors and screens after the final paint coats.

Answers applied: floor-mounted one-piece WC in all 7 baths (Q9); concealed diverter mixer + 8" rain shower + hand shower in the 6 baths with showers (Q10); counter-top basin on a granite top with a wall-hung HDHMR cabinet, SS 304 accessories and LED mirror, wall-hung basin in the powder room (Q11); 8 mm toughened fixed glass screens (Q12); 600 x 600 anti-slip floors and 300 x 600 walls to the false ceiling with one feature wall each (Q7, Q8); hybrid gas/electric geysers (Q32); cementitious waterproofing under all bath tiles. Bath exhaust fans are in Stage 11.

| Id | Bath | Floor (sft) | Wall tiles plain / feature (sft) | Waterproofing (sft) | WC | Basin | Shower | Screen | Traps | Vanity, mirror, accessories |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G05 | Bed-1 bath (incl. shower area) | 45.0 | 170 / 42 | 109 | one-piece | counter-top + granite top | concealed mixer, rain + hand shower | 8 mm screen | 2 | HDHMR vanity, LED mirror 900 x 750, 6-pc set |
| G08 | Master bath (incl. shower area) | 67.7 | 191 / 65 | 137 | one-piece | counter-top + granite top | concealed mixer, rain + hand shower | 8 mm screen | 2 | HDHMR vanity, LED mirror 900 x 750, 6-pc set |
| G10 | Powder room (P-W) | 18.0 | 88 / 34 | 35 | one-piece | wall-hung + half pedestal | - | - | 1 | LED mirror 600, 3-pc set |
| F06 | Bed-1 bath (incl. shower area) | 54.0 | 179 / 51 | 120 | one-piece | counter-top + granite top | concealed mixer, rain + hand shower | 8 mm screen | 2 | HDHMR vanity, LED mirror 900 x 750, 6-pc set |
| F09 | Master bath (incl. shower area) | 51.0 | 165 / 54 | 116 | one-piece | counter-top + granite top | concealed mixer, rain + hand shower | 8 mm screen | 2 | HDHMR vanity, LED mirror 900 x 750, 6-pc set |
| F15 | Front bed bath (incl. shower area) | 60.0 | 196 / 51 | 128 | one-piece | counter-top + granite top | concealed mixer, rain + hand shower | 8 mm screen | 2 | HDHMR vanity, LED mirror 900 x 750, 6-pc set |
| S03 | Bath | 26.1 | 109 / 40 | 83 | one-piece | counter-top + granite top | concealed mixer, rain + hand shower | 8 mm screen | 2 | HDHMR vanity, LED mirror 900 x 750, 6-pc set |
|  | Total | 321.8 | 1,098 / 338 | 727 | 7 | 7 | 6 | 6 | 13 |  |

Feature wall = the shorter wall behind the WC and vanity. Geysers: 2 x 35-gallon hybrid (GF and FF) + 1 x 15-gallon hybrid for the mumty bath and laundry; electric elements on their own 20 A circuits (Stage 1). Floor-trap bodies are in grey (item 11.F); only SS gratings are priced here.

## 8.1 Priced materials - Stage 6

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.01 | Waterproofing under tiles, 2 coats (3 kg/m2): floors + 1 ft upturn + shower walls 6 ft high = 727 sft | 203.6 | kg | 10% | 9 sets (25 kg) = 225 kg | 11,500 per set | 103,500 | estimated |
| 6.02 | Reinforcing tape at corners, floor traps and pipe entries (1 roll per bath) | 7 | roll | - | 7 rolls | 1,800 per roll | 12,600 | estimated |
| 6.03 | 600 x 600 matt anti-slip floor tiles - 7 baths 321.8 sft (10% cutting for falls) | 321.8 | sft | 10% | 23 boxes = 356.5 sft | 260 per sft | 92,690 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 6.04 | 300 x 600 glazed wall tiles, plain - to false ceiling 1,097.8 sft | 1,097.8 | sft | 10% | 78 boxes = 1,209 sft | 230 per sft | 278,070 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 6.05 | 300 x 600 feature tiles - one wall per bath 337.9 sft | 337.9 | sft | 10% | 24 boxes = 372 sft | 280 per sft | 104,160 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 6.06 | Cement for 1 1/4" average bed with falls to the traps | 6.81 | bag | 5% | 8 bags | 1,560 per bag | 12,480 | grey estimate Rev 1 (ARY News 15-08-2026 and brand list Sep 2026) |
| 6.07 | Ravi sand ("ret") for bedding mortar | 34.1 | cft | 10% | 38 cft | 40 per cft | 1,520 | grey estimate Rev 1 (estimated) |
| 6.08 | Tile adhesive: floors 0.30, walls 0.35 kg/sft | 599.0 | kg | 5% | 32 bags (20 kg) = 640 kg | 1,450 per bag | 46,400 | estimated |
| 6.09 | Tile grout, polymer cement, colour matched ("joint filler") | 49.5 | kg | 10% | 55 kg | 450 per kg | 24,750 | estimated |
| 6.10 | Tile levelling clips (large-format floors) | 160.9 | nos | 5% | 2 bags (100) = 200 nos | 1,200 per bag | 2,400 | estimated |
| 6.11 | Tile spacers 2 mm (walls) | 861.4 | nos | - | 9 bags (100) = 900 nos | 150 per bag | 1,350 | estimated |
| 6.12 | Edge trims: niches, ventilator reveals and external corners (3 per bath) | 21 | nos | - | 21 nos | 650 per nos | 13,650 | estimated |
| 6.13 | Floor-trap gratings: shower + general in 6 baths, 1 in the powder room | 13 | nos | - | 13 nos | 1,800 per nos | 23,400 | estimated |
| 6.14 | WC one-piece, floor-mounted, with soft-close seat (Porta / Master) | 7 | nos | - | 7 nos | 63,000 per nos | 441,000 | Daraz.pk listing, 2026 (Porta one-piece) |
| 6.15 | Muslim shower (health faucet) with hose, holder and angle valve | 7 | set | - | 7 sets | 3,800 per set | 26,600 | estimated |
| 6.16 | Angle valves: WC cistern + basin hot and cold | 21 | nos | - | 21 nos | 1,250 per nos | 26,250 | estimated |
| 6.17 | Flexible connector 1/2 in x 18 in | 21 | nos | - | 21 nos | 650 per nos | 13,650 | estimated |
| 6.18 | Counter-top (vessel) wash basin, ceramic (Porta / Master) | 6 | nos | - | 6 nos | 12,500 per nos | 75,000 | estimated |
| 6.19 | Wall-hung wash basin with half pedestal (powder room) | 1 | nos | - | 1 nos | 9,500 per nos | 9,500 | estimated |
| 6.20 | Basin mixer, single lever, tall (for counter-top basin) - Sonex / Faisal / Master | 6 | nos | - | 6 nos | 13,500 per nos | 81,000 | estimated |
| 6.21 | Basin mixer, single lever, standard (powder room) | 1 | nos | - | 1 nos | 9,500 per nos | 9,500 | estimated |
| 6.22 | Basin pop-up waste with chrome bottle trap | 7 | set | - | 7 sets | 3,500 per set | 24,500 | estimated |
| 6.23 | Concealed 2-way diverter mixer, body + trim (Sonex / Faisal / Master) | 6 | set | - | 6 sets | 17,500 per set | 105,000 | OLX.com.pk listings, 2026 (Rs 15,000-20,000) |
| 6.24 | Rain shower 8 in with wall arm | 6 | set | - | 6 sets | 6,500 per set | 39,000 | estimated |
| 6.25 | Hand shower with hose and holder | 6 | set | - | 6 sets | 3,500 per set | 21,000 | estimated |
| 6.26 | Shower screens 8 mm toughened, fixed, 3'-0" x 6'-6" in 6 baths (made to size) | 117 | sft | - | 117 sft | 650 per sft | 76,050 | estimated |
| 6.27 | Shower-screen fixings: SS U-channel, wall profile, support bar | 6 | set | - | 6 sets | 7,500 per set | 45,000 | estimated |
| 6.28 | Wall-hung vanity cabinets 2'-6" wide, HDHMR 18 mm - 0.75 sheet each | 4.50 | sheet | 10% | 5 sheets | 9,500 per sheet | 47,500 | estimated |
| 6.29 | Hettich soft-close concealed hinge | 12 | nos | - | 12 nos | 850 per nos | 10,200 | estimated |
| 6.30 | Cabinet handle (aluminium profile) | 6 | nos | - | 6 nos | 600 per nos | 3,600 | estimated |
| 6.31 | Edge band for vanities (8 m each) | 48 | m | - | 1 roll (50 m) = 50 m | 2,500 per roll | 2,500 | estimated |
| 6.32 | Vanity wall-hanging brackets and fixings | 6 | set | - | 6 sets | 1,500 per set | 9,000 | estimated |
| 6.33 | Granite vanity tops 3'-0" x 1'-10" with 3" upstand, basin cut-out (6 nos) | 37.5 | sft | 15% | 44 sft | 650 per sft | 28,600 | estimated (zameen.com: Jet Black ~Rs 400, Snow White ~Rs 570/sft) |
| 6.34 | Bath accessory set, SS 304, 6 pieces (towel rail, ring, 2 robe hooks, paper holder, soap shelf) | 6 | set | - | 6 sets | 25,000 per set | 150,000 | ideal.house price guide, Jan 2026 (mid sets Rs 25,000-35,000) |
| 6.35 | Powder-room accessory set, SS 304, 3 pieces | 1 | set | - | 1 set | 12,000 per set | 12,000 | estimated |
| 6.36 | LED mirror 900 x 750 mm, touch switch and demister | 6 | nos | - | 6 nos | 16,500 per nos | 99,000 | estimated (600 mm LED mirror Rs 10,125, Daraz.pk 2026) |
| 6.37 | LED mirror 600 mm round (powder room) | 1 | nos | - | 1 nos | 10,125 per nos | 10,125 | Daraz.pk listing, 2026 |
| 6.38 | Geyser hybrid gas/electric 35 gallon (Nasgas DEG-35 Super Deluxe) - GF and FF | 2 | nos | - | 2 nos | 48,000 per nos | 96,000 | Nasgas listing, 05-08-2026 |
| 6.39 | Geyser hybrid gas/electric 15 gallon (Nasgas / Canon) - mumty bath and laundry | 1 | nos | - | 1 nos | 34,000 per nos | 34,000 | estimated |
| 6.40 | Geyser fittings: safety and pressure-relief valves, gas cock, flexible connectors, unions | 3 | set | - | 3 sets | 5,500 per set | 16,500 | estimated |
|  | Stage 6 total (purchase quantities x rates) |  |  |  |  |  | 2,229,045 |  |

# 9. Stage 7 - Kitchens and wardrobes

On site: Cabinets and counters after the floor tiles; hobs, hoods and sinks last.

Kitchens (Q13-Q15): HDHMR carcass, high-gloss acrylic shutters, Hettich soft-close hinges and runners; granite counters in all three kitchens; built-in 5-burner hobs with chimney hoods in the open kitchen (hob on the island as drawn, so an island hood) and the FF kitchen; stainless double-bowl sinks with mixers in all three. Wardrobes ("almari", Q39): HDHMR carcass with laminate shutters and Hettich fittings, full height (8'-0" + 2'-0" loft) in the 5 dresses and the FF store as drawn.

| Id | Kitchen | Wall counter runs | Island | Wall cabinets (rft) | Sink | Hob | Hood | Backsplash (sft) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G11 | Open kitchen | 5'-3" rear wall + 6'-0" side wall | island 4'-6" base, top 5'-0" x 2'-6" | 7.9 | double-bowl + mixer | 5-burner | island chimney | 23.8 |
| G12 | Raw kitchen | 7'-0" L-counter + 9'-0" L-counter | - | 11.2 | double-bowl + mixer | none (see assumptions) | exhaust fan | 33.2 |
| F12 | Kitchen | 13'-0" L-counter + 6'-0" L-counter | - | 13.3 | double-bowl + mixer | 5-burner | wall chimney | 39.2 |

| Id | Location | Run | Shape | Height | Shutters (2 ft) |
| --- | --- | --- | --- | --- | --- |
| G04 | Bed-1 dress (GF) | 4'-0" | straight, on the bottom wall | 8'-0" + 2'-0" loft | 2 |
| G07 | Master dress (GF) | 4'-6" | straight | 8'-0" + 2'-0" loft | 3 |
| F05 | Bed-1 dress (FF) | 4'-0" | straight | 8'-0" + 2'-0" loft | 2 |
| F08 | Master dress (FF) | 4'-6" | straight | 8'-0" + 2'-0" loft | 3 |
| F14 | Front-bed dress (FF) | 9'-3" | L-shaped 5'-3" + 6'-0" less corner | 8'-0" + 2'-0" loft | 5 |
| F11 | Store (FF) | 17'-0" | L-shaped 13'-0" + 6'-0" less corner | 8'-0" + 2'-0" loft | 9 |
|  | Total wardrobe run | 43.2 rft |  |  | 22 |

Board take-off per rft: kitchen base 9 sft carcass + 2.8 sft back + 2.6 sft shutter; wall cabinet 6 + 2.5 + 2.4 sft; wardrobe 18 sft carcass + 10 sft back + 10 sft shutter. Wall cabinets over 70% of the wall counter runs (windows and hoods excluded). No wardrobe is drawn in the mumty room.

## 9.1 Priced materials - Stage 7

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.01 | Kitchen carcass HDHMR 18 mm: base 50.8 rft (incl. island) x 9 sft + wall cabinets 32.4 rft x 6 sft = 651 sft | 20.3 | sheet | 10% | 23 sheets | 9,500 per sheet | 218,500 | estimated |
| 7.02 | Kitchen back panels 6 mm: 223 sft | 6.97 | sheet | 10% | 8 sheets | 3,800 per sheet | 30,400 | estimated |
| 7.03 | Acrylic shutters and island side panels: 222 sft (56 shutters) | 6.95 | sheet | 10% | 8 sheets | 18,500 per sheet | 148,000 | estimated |
| 7.04 | Acrylic edge band on shutters | 136.6 | m | 10% | 4 rolls (50 m) = 200 m | 4,500 per roll | 18,000 | estimated |
| 7.05 | PVC edge band on carcass fronts | 101.3 | m | 10% | 3 rolls (50 m) = 150 m | 2,500 per roll | 7,500 | estimated |
| 7.06 | Hettich soft-close hinges, 2 per shutter | 112 | nos | - | 112 nos | 850 per nos | 95,200 | estimated |
| 7.07 | Soft-close drawer runners: 3 stacks of 3 drawers + 6 in the FF kitchen | 15 | pair | - | 15 pairs | 4,800 per pair | 72,000 | estimated |
| 7.08 | Pull-out SS basket (cutlery / plate / bottle) | 6 | nos | - | 6 nos | 6,500 per nos | 39,000 | estimated |
| 7.09 | Cabinet handle (aluminium profile) | 71 | nos | - | 71 nos | 600 per nos | 42,600 | estimated |
| 7.10 | Adjustable cabinet leg | 101.5 | nos | - | 102 nos | 120 per nos | 12,240 | estimated |
| 7.11 | Wall-cabinet hanger, pair | 22 | pair | - | 22 pairs | 450 per pair | 9,900 | estimated |
| 7.12 | Aluminium plinth on wall runs and island | 59.2 | rft | 5% | 63 rft | 350 per rft | 22,050 | estimated |
| 7.13 | Kitchen fixings, screws, glue, silicone (3 kitchens) | 3 | set | - | 3 sets | 3,000 per set | 9,000 | estimated |
| 7.14 | Granite counters 20 mm with 40 mm built-up front edge: 46.2 rft x 2'-1" + island top 5'-0" x 2'-6" | 118.0 | sft | 15% | 136 sft | 650 per sft | 88,400 | estimated (zameen.com: Jet Black ~Rs 400, Snow White ~Rs 570/sft) |
| 7.15 | Stainless-steel double-bowl sink with drain board | 3 | nos | - | 3 nos | 19,900 per nos | 59,700 | OLX.com.pk listing, 2026 |
| 7.16 | Sink mixer, neck type (Sonex) | 3 | nos | - | 3 nos | 19,500 per nos | 58,500 | OLX.com.pk listing, 2026 |
| 7.17 | Double-bowl sink waste kit with traps | 3 | set | - | 3 sets | 3,500 per set | 10,500 | estimated |
| 7.18 | Angle valves under sinks (hot and cold) | 6 | nos | - | 6 nos | 1,250 per nos | 7,500 | estimated |
| 7.19 | Flexible connector 1/2 in x 18 in | 6 | nos | - | 6 nos | 650 per nos | 3,900 | estimated |
| 7.20 | Built-in gas hob 5 burners (Nasgas DG-GN5 555) - open-kitchen island and FF kitchen | 2 | nos | - | 2 nos | 26,250 per nos | 52,500 | Nasgas listing, 2026 |
| 7.21 | Island chimney hood (local make: Nasgas / Canon / Signature) - over the open-kitchen island hob | 1 | nos | - | 1 nos | 52,000 per nos | 52,000 | estimated |
| 7.22 | Wall chimney hood (Nasgas) - FF kitchen | 1 | nos | - | 1 nos | 30,000 per nos | 30,000 | Nasgas listing, 05-08-2026 (Rs 27,300-35,000) |
| 7.23 | Hood duct 6 in aluminium flexible with wall cap | 2 | set | - | 2 sets | 3,500 per set | 7,000 | estimated |
| 7.24 | 300 x 600 backsplash 2'-0" high: open kitchen, raw kitchen, FF kitchen 96.2 sft | 96.2 | sft | 10% | 7 boxes = 108.5 sft | 250 per sft | 27,125 | estimated (market Rs 140-800/sft, Aug-Sep 2026) |
| 7.25 | Tile adhesive, polymer-modified cement C2TE ("tile bond") | 33.7 | kg | 5% | 2 bags (20 kg) = 40 kg | 1,450 per bag | 2,900 | estimated |
| 7.26 | Tile grout, polymer cement, colour matched ("joint filler") | 2.89 | kg | 10% | 4 kg | 450 per kg | 1,800 | estimated |
| 7.27 | Tile spacers 2 mm (walls) | 100 | nos | - | 1 bag (100) = 100 nos | 150 per bag | 150 | estimated |
| 7.28 | Aluminium tile edge trim 10 mm, 2.5 m length | 6 | nos | - | 6 nos | 650 per nos | 3,900 | estimated |
| 7.29 | Wardrobe carcass 18 mm pre-laminated: 43.2 rft x 18 sft (8 ft + 2 ft loft, 2 ft deep) | 24.3 | sheet | 10% | 27 sheets | 9,500 per sheet | 256,500 | estimated |
| 7.30 | Wardrobe back panels 6 mm | 13.5 | sheet | 10% | 15 sheets | 3,800 per sheet | 57,000 | estimated |
| 7.31 | Wardrobe shutter boards 18 mm: 432 sft | 13.5 | sheet | 10% | 15 sheets | 9,500 per sheet | 142,500 | estimated |
| 7.32 | Decorative laminate 1 mm on shutters (outside face) | 13.5 | sheet | 10% | 15 sheets | 4,500 per sheet | 67,500 | estimated |
| 7.33 | Liner laminate on shutters (inside face) | 13.5 | sheet | 10% | 15 sheets | 1,600 per sheet | 24,000 | estimated |
| 7.34 | Hettich soft-close hinges: 22 tall shutters x 4 + 22 loft shutters x 2 | 132 | nos | - | 132 nos | 850 per nos | 112,200 | estimated |
| 7.35 | Wardrobe long profile handle | 22 | nos | - | 22 nos | 900 per nos | 19,800 | estimated |
| 7.36 | Loft shutter handle | 22 | nos | - | 22 nos | 350 per nos | 7,700 | estimated |
| 7.37 | Wardrobe hanging rod (SS oval) with supports | 11 | nos | - | 11 nos | 1,200 per nos | 13,200 | estimated |
| 7.38 | Drawer runners, 2 drawers per wardrobe | 12 | pair | - | 12 pairs | 4,800 per pair | 57,600 | estimated |
| 7.39 | Cupboard lock | 6 | nos | - | 6 nos | 600 per nos | 3,600 | estimated |
| 7.40 | Edge band on shutters and carcass fronts | 326.2 | m | 10% | 8 rolls (50 m) = 400 m | 2,500 per roll | 20,000 | estimated |
| 7.41 | Wardrobe fixings (6 locations) | 6 | set | - | 6 sets | 3,000 per set | 18,000 | estimated |
|  | Stage 7 total (purchase quantities x rates) |  |  |  |  |  | 1,929,865 |  |

# 10. Stage 8 - Doors

On site: Diyar chowkats are fixed before plaster, so buy them first; shutters, hardware and polish after the floor tiles.

Answers applied: solid Diyar main door with 2 1/2" leaves and PU polish (Q16; its glazed fanlight is W07, Stage 9); 17 semi-solid internal doors - Diyar stiles and rails, ply core, veneer both faces, PU polish, solid Diyar chowkat - door frame (Q17); shutters ("palla") as below; WPC doors and frames in the 7 baths (Q18); insulated steel security doors for D02, D12 and D28 (Q19); Yale/Dorma locks on the main and external doors, good local locks with SS hinges inside (Q20); store door D14 included, master-dress opening D09 left open (Q40).

| Ref | Room | Opening | Frame (chowkat) | Shutter and thickness | Wood cft incl. architraves | Hardware | Finish |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D01 | Entrance foyer | 5'-1½" x 8'-0" | Diyar 7" x 3" 3.23 cft | Solid Diyar panelled 2 1/2", 2 leaves 6.49 cft | 10.80 | H1 | PU polish 126 sft |
| D02 | Stair lobby (S-T lobby) to outside | 3'-6" x 8'-0" | Steel frame (with door) | Insulated steel security door 2", powder coated | - | H2 | factory powder coat |
| D03 | Stair lobby (S-T lobby) / Lobby (5'-0" wide) | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D04 | Bed-1 / Lobby (5'-0" wide) | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D05 | Bed-1 dress / Bed-1 | 2'-10½" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.55 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.89 cft), 25 mm ply core, 6 mm ply + veneer both faces | 2.95 | H3 | PU polish 71 sft |
| D06 | Bed-1 bath (incl. shower area) | 2'-6" x 8'-0" | WPC frame (with door) | WPC door 1 1/4", factory finished | - | H4 | factory finish |
| D07 | Master bed / Lobby (5'-0" wide) | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D08 | Master bath (incl. shower area) | 2'-6" x 8'-0" | WPC frame (with door) | WPC door 1 1/4", factory finished | - | H4 | factory finish |
| D09 | Master dress | 2'-6" x 8'-0" | - | No leaf - opening left open (Q40) | - | - | paint |
| D10 | Powder room (P-W) | 2'-3" x 8'-0" | WPC frame (with door) | WPC door 1 1/4", factory finished | - | H4 | factory finish |
| D11 | Drawing room / Entrance foyer | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D12 | Raw kitchen to outside | 3'-0" x 8'-0" | Steel frame (with door) | Insulated steel security door 2", powder coated | - | H2 | factory powder coat |
| D13 | Raw kitchen / Open kitchen | 3'-0" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.56 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.90 cft), 25 mm ply core, 6 mm ply + veneer both faces | 2.98 | H3 | PU polish 73 sft |
| D14 | Under-stair store (S-A) / Lobby (5'-0" wide) | 2'-6" x 6'-6" | Diyar 4 1/2" x 2 1/2" 1.29 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.72 cft), 25 mm ply core, 6 mm ply + veneer both faces | 2.43 | H3 | PU polish 54 sft |
| A01 | Dining / Open kitchen | 6'-6" x 8'-0" | - | Open arch - no door | - | - | paint |
| D15 | Stair hall (S-T lobby) / Lobby, rear part (5'-0" wide) | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D16 | Bed-1 / Lobby, rear part (5'-0" wide) | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D17 | Bed-1 dress / Bed-1 | 2'-10½" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.55 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.89 cft), 25 mm ply core, 6 mm ply + veneer both faces | 2.95 | H3 | PU polish 71 sft |
| D18 | Bed-1 bath (incl. shower area) | 2'-6" x 8'-0" | WPC frame (with door) | WPC door 1 1/4", factory finished | - | H4 | factory finish |
| D19 | Master bed / Lobby, rear part (5'-0" wide) | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D20 | Master dress / Master bed | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D21 | Master bath (incl. shower area) | 2'-6" x 8'-0" | WPC frame (with door) | WPC door 1 1/4", factory finished | - | H4 | factory finish |
| D22 | Front bed (less splayed corner 2 sft) / Lobby, front part (6'-3" wide) | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D23 | Front bed dress / Front bed (less splayed corner 2 sft) | 3'-0" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.56 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.90 cft), 25 mm ply core, 6 mm ply + veneer both faces | 2.98 | H3 | PU polish 73 sft |
| D24 | Front bed bath (incl. shower area) | 2'-6" x 8'-0" | WPC frame (with door) | WPC door 1 1/4", factory finished | - | H4 | factory finish |
| D25 | Store (with wardrobes) / Lobby, front part (6'-3" wide) | 3'-0" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.56 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.90 cft), 25 mm ply core, 6 mm ply + veneer both faces | 2.98 | H3 | PU polish 73 sft |
| D26 | Kitchen / Lobby, front part (6'-3" wide) | 3'-0" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.56 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.90 cft), 25 mm ply core, 6 mm ply + veneer both faces | 2.98 | H3 | PU polish 73 sft |
| D27 | Kitchen | 6'-0" x 8'-0" | Aluminium (Stage 9) | 2-panel sliding, 6 mm toughened glass | - | Stage 9 | powder coat |
| D28 | Stair hall to outside | 3'-6" x 8'-0" | Steel frame (with door) | Insulated steel security door 2", powder coated | - | H2 | factory powder coat |
| D29 | Room / Stair hall | 3'-6" x 8'-0" | Diyar 4 1/2" x 2 1/2" 1.60 cft | Semi-solid 1 1/2": Diyar 4" x 1 1/2" frame (0.97 cft), 25 mm ply core, 6 mm ply + veneer both faces | 3.10 | H3 | PU polish 82 sft |
| D30 | Bath | 2'-6" x 8'-0" | WPC frame (with door) | WPC door 1 1/4", factory finished | - | H4 | factory finish |
|  | Diyar wood, net (architraves included) |  | 29.89 cft | 22.24 cft | 62.01 |  | gross 77.5 cft with 25% |

| Set | Doors | Hardware |
| --- | --- | --- |
| H1 | Main door D01 | Yale/Dorma heavy mortise lock + lever set + euro cylinder; door closer (Dorma TS-68 / Yale); 8 SS ball-bearing hinges 5"; flush bolts; 2 heavy floor stoppers; door viewer |
| H2 | External doors D02, D12, D28 | Yale/Dorma mortise lock + lever set + cylinder; heavy floor stopper (hinges come with the door) |
| H3 | 17 internal doors | Good local mortise lock with lever handles; 3 SS hinges 4"; floor stopper; SS tower bolt 6" |
| H4 | 7 bathroom doors | Privacy lock with thumb-turn and indicator; 3 SS hinges 4"; floor stopper |

Chowkat lengths include 3" horns; internal shutters are 4" smaller than the opening in width and 2" in height. PU polish area covers both faces and edges of the shutters, the exposed chowkat faces and both architraves. Diyar wastage 25% covers sawing and planing.

## 10.1 Priced materials - Stage 8

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.01 | Diyar chowkats: main door 7" x 3" (3.23 cft) + 17 internal doors 4 1/2" x 2 1/2" (26.7 cft) - buy before plaster | 29.9 | cft | 25% | 37.5 cft | 9,500 per cft | 356,250 | Faisalabad/Lahore timber rates 2026 (Rs 8,500-10,000/cft) |
| 8.02 | Diyar for the main-door leaves, solid panelled 2 1/2" (6.49 cft) | 6.49 | cft | 25% | 8.50 cft | 9,500 per cft | 80,750 | Faisalabad/Lahore timber rates 2026 (Rs 8,500-10,000/cft) |
| 8.03 | Diyar stiles and rails 4" x 1 1/2" for 17 semi-solid shutters (15.7 cft) | 15.7 | cft | 25% | 20 cft | 9,500 per cft | 190,000 | Faisalabad/Lahore timber rates 2026 (Rs 8,500-10,000/cft) |
| 8.04 | Diyar architraves both faces: main door 3 1/2" x 1", internal 2 1/2" x 3/4" (9.88 cft) | 9.88 | cft | 25% | 12.5 cft | 9,500 per cft | 118,750 | Faisalabad/Lahore timber rates 2026 (Rs 8,500-10,000/cft) |
| 8.05 | Ply core strips 25 mm at 50% fill, 17 shutters | 4.05 | sheet | 10% | 5 sheets | 9,800 per sheet | 49,000 | estimated |
| 8.06 | Ply skins 6 mm both faces, 17 shutters | 24.1 | sheet | 10% | 27 sheets | 3,000 per sheet | 81,000 | estimated |
| 8.07 | Veneer both faces, 17 shutters | 770.9 | sft | 15% | 887 sft | 150 per sft | 133,050 | estimated |
| 8.08 | Glue, nails, filler: main door + 17 internal doors | 18 | set | - | 18 sets | 1,200 per set | 21,600 | estimated |
| 8.09 | PU polish on shutters, edges, chowkats and architraves (18 doors) 1,438 sft | 1,437.7 | sft | 5% | 1,510 sft | 75 per sft | 113,250 | estimated |
| 8.10 | WPC door with WPC frame, factory finished, 2 ft 6 in x 8 ft class - 7 bathroom doors | 7 | set | - | 7 sets | 28,000 per set | 196,000 | estimated |
| 8.11 | Insulated steel security door with frame, powder coated, lock-prepared, up to 3 ft 6 in x 8 ft - D02, D12, D28 | 3 | nos | - | 3 nos | 68,000 per nos | 204,000 | estimated |
| 8.12 | Yale / Dorma heavy mortise lock, lever handle set and euro cylinder (main door) | 1 | set | - | 1 set | 24,000 per set | 24,000 | estimated |
| 8.13 | Door closer (Dorma TS-68 / Yale), main door | 1 | nos | - | 1 nos | 12,500 per nos | 12,500 | estimated |
| 8.14 | SS ball-bearing hinges, 4 per leaf (main door) | 8 | nos | - | 8 nos | 1,100 per nos | 8,800 | estimated |
| 8.15 | Flush bolt pair, top and bottom (inactive leaf) | 1 | pair | - | 1 pair | 1,600 per pair | 1,600 | estimated |
| 8.16 | Heavy floor stoppers: main door 2 + external doors 3 | 5 | nos | - | 5 nos | 950 per nos | 4,750 | estimated |
| 8.17 | Door viewer (eye) 200 deg | 1 | nos | - | 1 nos | 1,500 per nos | 1,500 | estimated |
| 8.18 | Yale / Dorma mortise lock, lever handle set and cylinder (external doors) | 3 | set | - | 3 sets | 16,500 per set | 49,500 | estimated |
| 8.19 | Mortise lock with lever handles, good local make | 17 | set | - | 17 sets | 4,800 per set | 81,600 | estimated |
| 8.20 | Bathroom privacy lock with thumb-turn/indicator and lever handles | 7 | set | - | 7 sets | 4,200 per set | 29,400 | estimated |
| 8.21 | SS hinges 4", 3 per door (17 internal + 7 bath) | 72 | nos | - | 72 nos | 450 per nos | 32,400 | estimated |
| 8.22 | Floor door stopper | 24 | nos | - | 24 nos | 650 per nos | 15,600 | estimated |
| 8.23 | SS tower bolt 6 in ("chitkani") | 17 | nos | - | 17 nos | 450 per nos | 7,650 | estimated |
|  | Stage 8 total (purchase quantities x rates) |  |  |  |  |  | 1,812,950 |  |

# 11. Stage 9 - Windows, glazing, fly-nets and grills

On site: Aluminium frames and glass after the first paint coats; grills before the final coats.

Windows ("khirki"). Answers applied: local medium aluminium series (frames about 1.2-1.4 mm wall), powder coated charcoal (RAL 7016), 3-track sliding with fly-net sash (Q21); fly-net ("jaali"); single 5 mm clear glass, frosted in the bath ventilators, 6+6 mm laminated only for the tall units W06, W07 and W09 (Q22); MS grills inside all GF windows, GF bath ventilators and stair windows (Q23).

## 11.1 Window schedule: aluminium, glass, fly-nets and grills per window

| Ref | Floor | Room | Size | Type | Nos | Alu kg | Glass | Glass sft | Net sft | Grill sft / kg | Sill |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W01 | GF | Bed-1 | 7'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 23.1 | 5 mm clear | 49.0 | 17.5 | 49 / 74 | granite |
| W02 | GF | Master bed | 7'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 23.1 | 5 mm clear | 49.0 | 17.5 | 49 / 74 | granite |
| W03 | GF | Dining | 3'-3" x 7'-0" | Sliding 3-track + fixed top light | 1 | 15.8 | 5 mm clear | 22.8 | 8.1 | 23 / 34 | granite |
| W04 | GF | Raw kitchen | 3'-0" x 4'-6" | Sliding 3-track | 1 | 11.2 | 5 mm clear | 13.5 | 6.8 | 14 / 20 | granite |
| W05 | GF | Drawing room | 6'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 21.2 | 5 mm clear | 42.0 | 15.0 | 42 / 63 | granite |
| W06 | GF+FF | TV lounge (GF) + Front bed (FF) | 8'-6" x 20'-0" | Fixed storefront (45 mm) | 1 | 60.5 | 6+6 laminated | 170.0 | - | - | granite |
| W07 | GF+FF | Entrance foyer + FF front lobby | 5'-1½" x 12'-6" | Fixed storefront (45 mm) | 1 | 31.8 | 6+6 laminated | 64.1 | - | - | - |
| W08 | FF | Stair hall (S-T lobby) | 3'-0" x 7'-0" | Fixed + top-hung vent | 2 | 15.7 | 5 mm clear | 42.0 | - | 42 / 63 | granite |
| W09 | FF+2F | Stair hall FF + mumty | 2'-0" x 25'-0" | Fixed storefront (45 mm) | 1 | 38.3 | 6+6 laminated | 50.0 | - | - | granite |
| W10 | FF | Bed-1 | 7'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 23.1 | 5 mm clear | 49.0 | 17.5 | - | granite |
| W11 | FF | Master bed | 7'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 23.1 | 5 mm clear | 49.0 | 17.5 | - | granite |
| W12 | FF | Front bed (less splayed corner 2 sft) | 7'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 23.1 | 5 mm clear | 49.0 | 17.5 | - | granite |
| W13 | FF | TV lounge | 8'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 25.1 | 5 mm clear | 56.0 | 20.0 | - | granite |
| W14 | 2F | Stair hall | 3'-0" x 7'-0" | Fixed + top-hung vent | 2 | 15.7 | 5 mm clear | 42.0 | - | 42 / 63 | granite |
| W15 | 2F | Room | 5'-0" x 7'-0" | Sliding 3-track + fixed top light | 1 | 19.2 | 5 mm clear | 35.0 | 12.5 | - | granite |
| V01 | GF | Bed-1 bath (incl. shower area) | 2'-6" x 2'-0" | Top-hung ventilator | 1 | 4.0 | 5 mm frosted | 5.0 | - | 5 / 8 | granite |
| V02 | GF | Master bath (incl. shower area) | 2'-6" x 2'-0" | Top-hung ventilator | 1 | 4.0 | 5 mm frosted | 5.0 | - | 5 / 8 | granite |
| V03 | GF | Powder room (P-W) | 2'-6" x 2'-0" | Top-hung ventilator | 1 | 4.0 | 5 mm frosted | 5.0 | - | 5 / 8 | granite |
| V04 | FF | Bed-1 bath (incl. shower area) | 2'-6" x 2'-0" | Top-hung ventilator | 1 | 4.0 | 5 mm frosted | 5.0 | - | - | granite |
| V05 | FF | Master bath (incl. shower area) | 2'-6" x 2'-0" | Top-hung ventilator | 1 | 4.0 | 5 mm frosted | 5.0 | - | - | granite |
| V06 | FF | Front bed bath (incl. shower area) | 2'-6" x 2'-0" | Top-hung ventilator | 1 | 4.0 | 5 mm frosted | 5.0 | - | - | granite |
| V07 | 2F | Bath | 2'-6" x 2'-0" | Top-hung ventilator | 1 | 4.0 | 5 mm frosted | 5.0 | - | - | granite |
| D27 | FF | Kitchen | 6'-0" x 8'-0" | Sliding door, heavy series | 1 | 29.1 | 6 mm toughened | 48.0 | 24.0 | - | granite |
|  |  | TOTAL |  |  | 25 | 427.2 |  | 865.3 | 173.9 | 275 / 413 |  |

## 11.2 Aluminium section series and gauge

| Openings | Series and wall thickness (gauge) | Section weights used (kg/m) | Aluminium kg |
| --- | --- | --- | --- |
| Sliding windows W01-W05, W10-W13, W15 | Local medium 3-track sliding series, frame 1.2-1.4 mm, sash 1.2 mm | frame 1.10, glass sash 0.65, net sash 0.40, top-light frame 0.80, transom 0.90 | 197.1 |
| Sliding window W04 | Same series without top light | frame 1.10, sash 0.65, net sash 0.40 | 11.2 |
| Stair windows W08, W14 (x2 each) | Fixed frame + top-hung vent, 1.2 mm | frame 0.80, transom 0.90, vent sash 0.70 | 31.3 |
| Bath ventilators V01-V07 | Top-hung ventilator, 1.2 mm | frame 0.75 + sash 0.70 | 27.8 |
| Tall units W06, W07, W09 | 45 mm storefront series, 1.6-2.0 mm, mullions at 4'-6", transoms at 6'-0" | frame and bars 1.80; vents 0.70 | 130.6 |
| Sliding door D27 | Heavy sliding-door series, 1.6 mm | frame 1.60, sash 1.10, net sash 0.50 | 29.1 |
| Total |  |  | 427.2 |

Aluminium weights use the Step A model (sliding frame 1.10 kg/m, glass sash 0.65, net sash 0.40, fixed frame 0.80, transom 0.90, vent frame 0.75 + sash 0.70, storefront 1.80 kg/m). Ask the fabricator for catalogue weights; the material rate is per kg of powder-coated section. Grills: 12 mm square bars at 5" with a 1 1/4" x 1/4" flat frame, about 1.5 kg per sft, painted charcoal (Stage 10).

## 11.3 Priced materials - Stage 9

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.01 | Aluminium sections for 17 windows, 7 ventilators and sliding door D27: 427 kg (Step A weight model) | 427.2 | kg | 5% | 449 kg | 1,450 per kg | 651,050 | estimated |
| 9.02 | Sliding window hardware: rollers, crescent lock, handles, brush seal, gaskets, end caps | 10 | set | - | 10 sets | 3,200 per set | 32,000 | estimated |
| 9.03 | Fixed + top-hung vent hardware: friction stays, handle, gaskets | 4 | set | - | 4 sets | 1,800 per set | 7,200 | estimated |
| 9.04 | Top-hung ventilator hardware: friction stays, handle, gaskets | 11 | set | - | 11 sets | 1,400 per set | 15,400 | estimated |
| 9.05 | Storefront glazing kit: gaskets, glazing beads, screws, anchors | 3 | set | - | 3 sets | 2,500 per set | 7,500 | estimated |
| 9.06 | Sliding door hardware: heavy rollers, lock, handles, seals | 1 | set | - | 1 set | 6,500 per set | 6,500 | estimated |
| 9.07 | Float glass 5 mm clear - clear windows 498.2 sft | 498.2 | sft | 5% | 524 sft | 215 per sft | 112,660 | estimated (zameen.com Mar 2023 Rs 142/sft, escalated to 2026) |
| 9.08 | Float glass 5 mm frosted / obscure (bathroom ventilators) - bathroom ventilators 35.0 sft | 35 | sft | 5% | 37 sft | 250 per sft | 9,250 | estimated (zameen.com Mar 2023 basis, escalated) |
| 9.09 | Laminated glass 6+6 mm clear (12.76 mm, PVB) - tall units W06, W07, W09 284.1 sft | 284.1 | sft | - | 285 sft | 1,050 per sft | 299,250 | estimated |
| 9.10 | Toughened glass 6 mm clear (sliding door D27) - sliding door D27 48.0 sft | 48 | sft | - | 48 sft | 520 per sft | 24,960 | estimated |
| 9.11 | Silicone: outside perimeter of all frames 573 rft (15 rft per cartridge) | 38.2 | nos | 10% | 43 nos | 1,100 per nos | 47,300 | estimated |
| 9.12 | Fly-net sashes on all sliding windows and D27: 173.9 sft | 173.9 | sft | 10% | 192 sft | 55 per sft | 10,560 | estimated |
| 9.13 | MS grills inside GF windows, GF bath vents and stair windows: 275.2 sft x 1.5 kg/sft | 412.9 | kg | 7% | 442 kg | 285 per kg | 125,970 | estimated (steel bars Rs 228-232/kg, Feb 2026, + section premium) |
| 9.14 | Rawl bolts, 6 per grill (12 grills) | 72 | nos | - | 72 nos | 60 per nos | 4,320 | estimated |
|  | Stage 9 total (purchase quantities x rates) |  |  |  |  |  | 1,353,920 |  |

# 12. Stage 10 - Paint and colour scheme

On site: Putty and primer before tiling; final coats after doors, windows and kitchens; exterior and boundary walls last.

Paint ("rang"). Brand and system (Q24): Dulux (AkzoNobel Pakistan) standard range, not the top-premium lines. Inside: 2 coats of wall putty, 1 coat of primer and 2 coats of emulsion; gypsum ceilings get 1 skim coat of putty. Outside: alkali-resistant primer and 2 coats of Weathershield. Texture coat on the slab-edge feature bands and the boundary pillars (Q26). Gates are powder coated (Stage 12); aluminium is factory powder coated; grills get red-oxide primer and 2 coats of enamel.

## 12.1 Paint per area (net areas; gallons are 3.64 L)

| Area | Net sft | Putty | Primer | Finish coats | Colour |
| --- | --- | --- | --- | --- | --- |
| Lobby (5'-0" wide) - walls | 310 | 26 kg = 1.3 bags | 0.9 gal | 1.7 gal (6 L) emulsion | Egyptian Cotton 310 sft |
| Bed-1 - walls | 451 | 38 kg = 1.9 bags | 1.3 gal | 2.5 gal (9 L) emulsion | Tranquil Dawn 130 sft + Jasmine White 321 sft |
| Bed-1 dress - walls | 157 | 13 kg = 0.7 bags | 0.4 gal | 0.9 gal (3 L) emulsion | Jasmine White 157 sft |
| Master bed - walls | 456 | 39 kg = 1.9 bags | 1.3 gal | 2.5 gal (9 L) emulsion | Denim Drift 120 sft + Jasmine White 336 sft |
| Master dress - walls | 170 | 14 kg = 0.7 bags | 0.5 gal | 0.9 gal (3 L) emulsion | Jasmine White 170 sft |
| Dining - walls | 306 | 26 kg = 1.3 bags | 0.9 gal | 1.7 gal (6 L) emulsion | Egyptian Cotton 306 sft |
| Open kitchen - walls | 304 | 26 kg = 1.3 bags | 0.9 gal | 1.7 gal (6 L) emulsion | Jasmine White 304 sft |
| Raw kitchen - walls | 265 | 23 kg = 1.1 bags | 0.8 gal | 1.5 gal (5 L) emulsion | Jasmine White 265 sft |
| TV lounge (less splayed corner 2 sft) - walls | 470 | 40 kg = 2.0 bags | 1.3 gal | 2.6 gal (9 L) emulsion | Chic Shadow 150 sft + Egyptian Cotton 320 sft |
| Drawing room - walls | 438 | 37 kg = 1.9 bags | 1.2 gal | 2.4 gal (9 L) emulsion | Egyptian Cotton 438 sft |
| Entrance foyer - walls | 139 | 12 kg = 0.6 bags | 0.4 gal | 0.8 gal (3 L) emulsion | Egyptian Cotton 139 sft |
| Under-stair store (S-A) - walls | 154 | 13 kg = 0.7 bags | 0.4 gal | 0.9 gal (3 L) emulsion | Jasmine White 154 sft |
| Lobby, rear part (5'-0" wide) - walls | 250 | 21 kg = 1.1 bags | 0.7 gal | 1.4 gal (5 L) emulsion | Egyptian Cotton 250 sft |
| Lobby, front part (6'-3" wide) - walls | 267 | 23 kg = 1.1 bags | 0.8 gal | 1.5 gal (5 L) emulsion | Egyptian Cotton 267 sft |
| Bed-1 - walls | 431 | 37 kg = 1.8 bags | 1.2 gal | 2.4 gal (9 L) emulsion | Thai Magic 3 120 sft + Jasmine White 311 sft |
| Bed-1 dress - walls | 177 | 15 kg = 0.8 bags | 0.5 gal | 1.0 gal (4 L) emulsion | Jasmine White 177 sft |
| Master bed - walls | 496 | 42 kg = 2.1 bags | 1.4 gal | 2.8 gal (10 L) emulsion | Denim Drift 140 sft + Jasmine White 356 sft |
| Master dress - walls | 190 | 16 kg = 0.8 bags | 0.5 gal | 1.1 gal (4 L) emulsion | Jasmine White 190 sft |
| TV lounge - walls | 384 | 33 kg = 1.6 bags | 1.1 gal | 2.1 gal (8 L) emulsion | Chic Shadow 148 sft + Egyptian Cotton 236 sft |
| Store (with wardrobes) - walls | 356 | 30 kg = 1.5 bags | 1.0 gal | 2.0 gal (7 L) emulsion | Jasmine White 356 sft |
| Kitchen - walls | 317 | 27 kg = 1.3 bags | 0.9 gal | 1.8 gal (6 L) emulsion | Jasmine White 317 sft |
| Front bed (less splayed corner 2 sft) - walls | 456 | 39 kg = 1.9 bags | 1.3 gal | 2.5 gal (9 L) emulsion | Tranquil Dawn 140 sft + Jasmine White 316 sft |
| Front bed dress - walls | 201 | 17 kg = 0.9 bags | 0.6 gal | 1.1 gal (4 L) emulsion | Jasmine White 201 sft |
| Room - walls | 335 | 28 kg = 1.4 bags | 1.0 gal | 1.9 gal (7 L) emulsion | Thai Magic 3 94 sft + Jasmine White 242 sft |
| Laundry (open on terrace side) - walls | 108 | 9 kg = 0.5 bags | 0.3 gal | 0.6 gal (2 L) emulsion | Jasmine White 108 sft |
| Stair well walls (3 storeys) | 1,078 | 92 kg = 4.6 bags | 3.1 gal | 6.0 gal (22 L) emulsion | Natural Hessian |
| Stair soffits (flights and landings) | 206 | 18 kg = 0.9 bags | 0.6 gal | 1.1 gal (4 L) emulsion | Brilliant White |
| Ground floor - false ceilings incl. cove boxes | 1,618 | 73 kg = 3.6 bags | 4.6 gal | 9.0 gal (33 L) emulsion | Brilliant White |
| First floor - false ceilings incl. cove boxes | 1,588 | 71 kg = 3.6 bags | 4.5 gal | 8.8 gal (32 L) emulsion | Brilliant White |
| Second floor (mumty) - false ceilings incl. cove boxes | 303 | 14 kg = 0.7 bags | 0.9 gal | 1.7 gal (6 L) emulsion | Brilliant White |
| Front (road) - walls | 490 | - | 1.6 gal | 3.3 gal (12 L) Weathershield | Jasmine White 67YY 88/044 |
| Front (road) - exposed plinth ("kursi") 1'-9" | 58 | - | 0.2 gal | 0.4 gal (1 L) Weathershield | Urban Obsession 00NN 25/000 |
| Front (road) - slab-edge bands 1'-0" | 71 | - | 0.2 gal | 11 kg texture | Urban Obsession 00NN 25/000 |
| Left (side passage) - walls | 929 | - | 3.1 gal | 6.2 gal (23 L) Weathershield | Jasmine White 67YY 88/044 |
| Left (side passage) - exposed plinth ("kursi") 1'-9" | 80 | - | 0.3 gal | 0.5 gal (2 L) Weathershield | Urban Obsession 00NN 25/000 |
| Left (side passage) - slab-edge bands 1'-0" | 96 | - | 0.3 gal | 14 kg texture | Urban Obsession 00NN 25/000 |
| Rear (rear passage / rear porch) - walls | 706 | - | 2.4 gal | 4.7 gal (17 L) Weathershield | Jasmine White 67YY 88/044 |
| Rear (rear passage / rear porch) - exposed plinth ("kursi") 1'-9" | 63 | - | 0.2 gal | 0.4 gal (2 L) Weathershield | Urban Obsession 00NN 25/000 |
| Rear (rear passage / rear porch) - slab-edge bands 1'-0" | 70 | - | 0.2 gal | 10 kg texture | Urban Obsession 00NN 25/000 |
| Right open stretches (patio, open shaft) - walls | 385 | - | 1.3 gal | 2.6 gal (9 L) Weathershield | Jasmine White 67YY 88/044 |
| Right open stretches (patio, open shaft) - exposed plinth ("kursi") 1'-9" | 32 | - | 0.1 gal | 0.2 gal (1 L) Weathershield | Urban Obsession 00NN 25/000 |
| Right open stretches (patio, open shaft) - slab-edge bands 1'-0" | 38 | - | 0.1 gal | 6 kg texture | Urban Obsession 00NN 25/000 |
| Mumty walls above terrace (all sides) - walls | 711 | - | 2.4 gal | 4.7 gal (17 L) Weathershield | Jasmine White 67YY 88/044 |
| Mumty walls above terrace (all sides) - slab-edge bands 1'-0" | 82 | - | 0.3 gal | 12 kg texture | Urban Obsession 00NN 25/000 |
| Parapets - outer face | 591 | - | 2.0 gal | 3.9 gal (14 L) Weathershield | Jasmine White 67YY 88/044 |
| Parapets - inner face and top | 722 | - | 2.4 gal | 4.8 gal (18 L) Weathershield | Night Jewels 5 00NN 62/000 |
| External soffits (porch, canopy, chajjas) | 241 | - | 0.8 gal | 1.6 gal (6 L) Weathershield | Brilliant White |
| Boundary walls - inside faces | 554 | - | 1.8 gal | 3.7 gal (13 L) Weathershield | Night Jewels 5 00NN 62/000 |
| Boundary walls - street and gali faces | 609 | - | 2.0 gal | 4.1 gal (15 L) Weathershield | Night Jewels 5 00NN 62/000 |
| Boundary walls - coping top | 145 | - | 0.5 gal | 1.0 gal (4 L) Weathershield | Urban Obsession 00NN 25/000 |
| Boundary pillars (10) - all faces | 308 | - | 1.0 gal | 46 kg texture | Urban Obsession 00NN 25/000 |
| Grills (both faces of the bars) | 330 | - | 0.8 gal red oxide | 1.7 gal enamel | charcoal to match RAL 7016 |

Coverage used: putty 0.085 kg/sft for 2 coats (0.045 for 1 skim coat), primer 350 sft/gal inside and 300 outside, emulsion 180 sft/gal and Weathershield 150 sft/gal for 2 coats, texture 0.15 kg/sft. Paint areas: interior walls 8,664 sft (Step A, openings deducted by IS 1200), exterior 5,366 sft (Step A incl. parapets and soffits) and boundary walls 1,615 sft (street, gali and inside faces; neighbour faces optional).

## 12.2 Colour scheme - Dulux (AkzoNobel Pakistan) colour names and codes

| Room / surface | Colour | Code | Role |
| --- | --- | --- | --- |
| All false ceilings, stair soffits, porch soffits | Brilliant White | ready-mixed white | one bright ceiling plane throughout |
| Lobbies, foyer, dining, drawing room, both TV lounges | Egyptian Cotton | 40YY 65/061 | warm greige for the living areas |
| Bedrooms (3 walls), dresses, kitchens, store, laundry, mumty room | Jasmine White | 67YY 88/044 | soft warm white, the quiet base |
| Stair well, GF to mumty | Natural Hessian | 14YY 69/082 | slightly deeper warm neutral for the tall wall |
| Accent wall: GF Bed-1 and FF Front bed | Tranquil Dawn | 45GY 55/052 | muted sage |
| Accent wall: GF and FF master bedrooms | Denim Drift | 87BG 27/077 | dusty blue-grey |
| Accent wall: FF Bed-1 and mumty room | Thai Magic 3 | 30YR 41/263 | soft clay |
| TV walls in both lounges | Chic Shadow | 00NN 53/000 | mid grey behind the screen |

| Elevation | Body | Feature bands / pillars | Plinth / coping | Other |
| --- | --- | --- | --- | --- |
| Front (road) | Jasmine White | Urban Obsession texture | Urban Obsession | Brilliant White soffits; main door natural Diyar polish; frames RAL 7016 |
| Left (side passage) | Jasmine White | Urban Obsession texture | Urban Obsession | charcoal grills and frames |
| Rear (rear passage / porch) | Jasmine White | Urban Obsession texture | Urban Obsession | steel doors powder coated to match |
| Right (patio, open shaft) | Jasmine White | Urban Obsession texture | Urban Obsession | - |
| Mumty walls | Jasmine White | Urban Obsession texture | - | parapet inner faces and tops Night Jewels 5 |
| Boundary walls | Night Jewels 5 (light grey) | pillars: Urban Obsession texture | coping: Urban Obsession | gates RAL 7016 anthracite |

How the palette holds together: every wall colour comes from one warm family (yellow-based neutrals Jasmine White, Egyptian Cotton and Natural Hessian, codes 14YY-67YY) under one white ceiling, so rooms flow into each other without breaks. Each bedroom adds only one muted accent wall, and the three accents (sage, dusty blue, clay) share the same soft, greyed strength, so they never compete. One neutral grey line (Night Jewels 5 -> Chic Shadow -> Urban Obsession, all 00NN) links inside and out: TV walls, house bands, boundary walls and pillars, and the charcoal of the windows, grills and gates (RAL 7016). Natural Diyar polish on the doors is the wood-tone accent. Confirm every code on the dealer's Dulux tinting machine and approve a 1 sft sample on site before ordering.

## 12.3 Priced materials - Stage 10

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.01 | Putty: 2 coats on 8,870 sft of walls and stair soffits (0.085 kg/sft), 1 skim coat on 3,508 sft of gypsum ceilings | 911.8 | kg | 5% | 48 bags (20 kg) = 960 kg | 2,850 per bag | 136,800 | Dulux wall putty 20 kg Rs 2,850 (2026 listing) |
| 10.02 | Interior primer 1 coat on 12,378 sft (350 sft/gal) | 35.4 | gal | 5% | 38 gal | 2,650 per gal | 100,700 | shop.berger.com.pk 2026 (Top Super Primer Plus Rs 2,565-2,700) |
| 10.03 | Emulsion Brilliant White - 3,715 sft, 2 coats (180 sft/gal) | 20.6 | gal | 5% | 22 gal | 3,800 per gal | 83,600 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.04 | Emulsion Jasmine White 67YY 88/044 - 4,279 sft, 2 coats (180 sft/gal) | 23.8 | gal | 5% | 25 gal | 3,800 per gal | 95,000 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.05 | Emulsion Egyptian Cotton 40YY 65/061 - 2,266 sft, 2 coats (180 sft/gal) | 12.6 | gal | 5% | 14 gal | 3,800 per gal | 53,200 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.06 | Emulsion Natural Hessian 14YY 69/082 - 1,078 sft, 2 coats (180 sft/gal) | 5.99 | gal | 5% | 7 gal | 3,800 per gal | 26,600 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.07 | Emulsion Tranquil Dawn 45GY 55/052 - 270 sft, 2 coats (180 sft/gal) | 1.50 | gal | 5% | 2 gal | 3,800 per gal | 7,600 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.08 | Emulsion Denim Drift 87BG 27/077 - 260 sft, 2 coats (180 sft/gal) | 1.44 | gal | 5% | 2 gal | 3,800 per gal | 7,600 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.09 | Emulsion Thai Magic 3 30YR 41/263 - 214 sft, 2 coats (180 sft/gal) | 1.19 | gal | 5% | 2 gal | 3,800 per gal | 7,600 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.10 | Emulsion Chic Shadow 00NN 53/000 - 298 sft, 2 coats (180 sft/gal) | 1.65 | gal | 5% | 2 gal | 3,800 per gal | 7,600 | estimated (gallon emulsions Rs 3,600-4,000, shop.berger.com.pk 2026) |
| 10.11 | Exterior primer 1 coat on 6,981 sft (300 sft/gal) | 23.3 | gal | 5% | 25 gal | 3,335 per gal | 83,375 | shop.berger.com.pk 2026 (Elegance Ultra Seal Rs 3,335) |
| 10.12 | Weathershield Brilliant White - 241 sft, 2 coats (150 sft/gal) | 1.61 | gal | 5% | 2 gal | 5,900 per gal | 11,800 | estimated (undated ICI list Rs 4,065, escalated) |
| 10.13 | Weathershield Jasmine White 67YY 88/044 - 3,813 sft, 2 coats (150 sft/gal) | 25.4 | gal | 5% | 27 gal | 5,900 per gal | 159,300 | estimated (undated ICI list Rs 4,065, escalated) |
| 10.14 | Weathershield Night Jewels 5 00NN 62/000 - 1,885 sft, 2 coats (150 sft/gal) | 12.6 | gal | 5% | 14 gal | 5,900 per gal | 82,600 | estimated (undated ICI list Rs 4,065, escalated) |
| 10.15 | Weathershield Urban Obsession 00NN 25/000 - 378 sft, 2 coats (150 sft/gal) | 2.52 | gal | 5% | 3 gal | 5,900 per gal | 17,700 | estimated (undated ICI list Rs 4,065, escalated) |
| 10.16 | Texture coat tinted Urban Obsession 00NN 25/000 on bands and boundary pillars - 665 sft at 0.15 kg/sft | 99.7 | kg | 5% | 6 buckets (20 kg) = 120 kg | 7,800 per bucket | 46,800 | estimated |
| 10.17 | Red-oxide primer on grills 330 sft (400 sft/gal) | 0.83 | gal | 5% | 1 gal | 3,000 per gal | 3,000 | estimated |
| 10.18 | Enamel 2 coats on grills (200 sft/gal for 2 coats) | 1.65 | gal | 5% | 2 gal | 4,800 per gal | 9,600 | icons.com.pk paint prices 29-07-2026 (enamel Rs 1,200-2,000 per litre) |
| 10.19 | Paint sundries: sandpaper, masking tape, filler | 1 | set | - | 1 set | 15,000 per set | 15,000 | estimated |
|  | Stage 10 total (purchase quantities x rates) |  |  |  |  |  | 955,475 |  |

# 13. Stage 11 - Electrical second fix: switches, lights, fans, ACs, Wi-Fi, CCTV, EV charger

On site: After the final paint coats.

## 13.1 Switches and sockets (Q28: good local modular make, no smart switches)

| Floor | 1-way switches | 2-way switches | 5 A sockets | 15 A sockets | DP 20 A (ACs, geysers) | Switch-board plates |
| --- | --- | --- | --- | --- | --- | --- |
| Ground floor | 46 | 5 | 33 | 9 | 6 | 21 |
| First floor | 42 | 8 | 30 | 6 | 5 | 20 |
| Second floor (mumty) | 8 | 3 | 5 | 2 | 2 | 5 |
| External | 8 | 0 | 6 | 0 | 0 | 5 |
| Total | 104 | 16 | 74 | 17 | 13 | 51 |

## 13.2 Light fittings, fans and exhausts, room by room

| Id | Room | 12 W downlights | Cove strip (m) | Profile (m) | Decorative / outdoor fittings | Fan | Exhaust | AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ground floor |  |  |  |  |  |  |  |  |
| G01 | Stair lobby (S-T lobby) | 2 | - | - | - | - | - | - |
| G02 | Lobby (5'-0" wide) | 4 | - | - | chandelier | - | - | - |
| G03 | Bed-1 | 8 | - | - | - | 1 | - | 1.5 t |
| G04 | Bed-1 dress | 1 | - | - | - | - | - | - |
| G05 | Bed-1 bath (incl. shower area) | 2 | - | - | LED mirror (Stage 6) | - | 1 | - |
| G06 | Master bed | 8 | 16.0 | 6.9 | - | 1 | - | 1.5 t |
| G07 | Master dress | 1 | - | - | - | - | - | - |
| G08 | Master bath (incl. shower area) | 3 | - | - | LED mirror (Stage 6) | - | 1 | - |
| G09 | Dining | 4 | - | - | pendant cluster over the table | 1 | - | 1.5 t |
| G10 | Powder room (P-W) | 1 | - | - | LED mirror (Stage 6) | - | 1 | - |
| G11 | Open kitchen | 6 | - | - | under-cabinet LED | - | - | - |
| G12 | Raw kitchen | 3 | - | - | - | 1 | 1 | - |
| G13 | TV lounge (less splayed corner 2 sft) | 8 | 16.5 | 7.3 | TV-wall LED wash | 1 | - | 2 t |
| G14 | Drawing room | 7 | 15.2 | 6.1 | chandelier | 1 | - | 1.5 t |
| G15 | Entrance foyer | 2 | - | - | double-height pendant | - | - | - |
| G16 | Under-stair store (S-A) | 1 | - | - | - | - | - | - |
| First floor |  |  |  |  |  |  |  |  |
| F01 | Stair hall (S-T lobby) | 3 | - | - | 3 stair wall lights | - | - | - |
| F02 | Lobby, rear part (5'-0" wide) | 3 | - | - | - | - | - | - |
| F03 | Lobby, front part (6'-3" wide) | 3 | - | - | pendant at the front glazing | - | - | - |
| F04 | Bed-1 | 7 | - | - | - | 1 | - | 1.5 t |
| F05 | Bed-1 dress | 1 | - | - | - | - | - | - |
| F06 | Bed-1 bath (incl. shower area) | 2 | - | - | LED mirror (Stage 6) | - | 1 | - |
| F07 | Master bed | 9 | 17.2 | 6.9 | - | 1 | - | 2 t |
| F08 | Master dress | 1 | - | - | - | - | - | - |
| F09 | Master bath (incl. shower area) | 2 | - | - | LED mirror (Stage 6) | - | 1 | - |
| F10 | TV lounge | 9 | 17.3 | 7.2 | TV-wall LED wash | 1 | - | 2 t |
| F11 | Store (with wardrobes) | 2 | - | - | - | - | - | - |
| F12 | Kitchen | 5 | - | - | under-cabinet LED | - | 1 | - |
| F13 | Front bed (less splayed corner 2 sft) | 9 | - | - | - | 1 | - | 2 t |
| F14 | Front bed dress | 2 | - | - | - | - | - | - |
| F15 | Front bed bath (incl. shower area) | 2 | - | - | LED mirror (Stage 6) | - | 1 | - |
| Second floor (mumty) |  |  |  |  |  |  |  |  |
| S01 | Stair hall | 3 | - | - | - | - | - | - |
| S02 | Room | 6 | - | - | - | 1 | - | 1.5 t |
| S03 | Bath | 1 | - | - | LED mirror (Stage 6) | - | 1 | - |
| S04 | Laundry (open on terrace side) | 2 | - | - | - | - | - | - |
| External |  |  |  |  |  |  |  |  |
|  | Front car porch | - | - | - | 4 x Porch ceiling light LED 18 W | 1 | - | - |
|  | Rear car porch + rear passage | - | - | - | 3 x Bulkhead light LED 12 W | - | - | - |
|  | Left passage + lawn | - | - | - | 5 x Outdoor wall light LED 12 W | - | - | - |
|  | Left passage + lawn | - | - | - | 3 x Lawn spike light LED 7 W | - | - | - |
|  | Gates and boundary pillars | - | - | - | 4 x Gate-pillar lantern LED | - | - | - |
|  | Terrace and mumty roof | - | - | - | 4 x Bulkhead light LED 12 W | - | - | - |
|  | Front balcony (FF kitchen terrace) | - | - | - | 1 x Outdoor wall light LED 12 W | - | - | - |
|  | Front elevation (sheet 09) | - | - | 9.0 m outdoor | 3 profile runs | - | - | - |
|  | Total | 133 | 82.2 | 34.3 | 12 decorative + 24 outdoor | 12 | 9 | 10 ACs |

Q29: 12 W downlights, 3000 K in bedrooms and 4000 K in living areas, kitchens and baths. Cove (LED strip in the gypsum cove) and recessed aluminium profiles with COB strip only in the drawing room, both TV lounges and both master bedrooms (Q27). Chandeliers and pendants are prime-cost allowances. Q31: DC inverter fans, 35-40 W, with remote.

## 13.3 Air-conditioners: tonnage from room size and orientation (Q30)

Method: base = conditioned area / 125 sft per ton (GF room, 10'-0" false ceiling, north light, 2 people). Additions: roof above +15%; main window or wall facing west +10%, south +5%, east 0, north -5% (half of a positive addition where a passage wall, porch or shaft shades it); second exposed wall +5%; +5% per 25 sft of glass above 50 sft; living, drawing and dining rooms +10%. Open-plan rooms carry 30% of the un-air-conditioned area open to them. Choose the smallest standard size within 10% of the need (a DC inverter unit boosts for the peak hours). Front (road) side = south.

| Room | Area (sft) | Cond. area | Main exposure | Glass sft | Base t | Additions | Need t | Selected | Step A | Reading of the plan |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bed-1 | 169.0 | 169.0 | north (shaded) | 49 | 1.35 | north -5% | 1.28 | 1.5 t | 1.5 t | W01 on the rear (north) wall under the rear-passage shade; east wall is the party wall |
| Master bed | 171.0 | 171.0 | west (shaded) | 49 | 1.37 | west +5% | 1.44 | 1.5 t | 1.5 t | W02 faces the 3'-3" west passage, shaded by the 7 ft boundary wall |
| Dining | 86.6 + 30% of Open kitchen | 134.7 | east (shaded) | 23 | 1.08 | living +10% | 1.18 | 1.5 t | 1 t | small east window W03 on the patio; open to the open kitchen through the 6'-6" arch |
| TV lounge (less splayed corner 2 sft) | 177.8 | 177.8 | south (open) | 81 | 1.42 | south +5%, 2nd wall +5%, glass +6.2%, living +10% | 1.79 | 2 t | 2 t | south double-height glazing W06 (lower part); west wall on the side passage |
| Drawing room | 156.0 | 156.0 | south (shaded) | 42 | 1.25 | south +2.5%, living +10% | 1.40 | 1.5 t | 1.5 t | front window W05 shaded by the car-porch slab; east wall is the party wall |
| Bed-1 | 156.0 | 156.0 | north (shaded) | 49 | 1.25 | roof +15%, north -5% | 1.37 | 1.5 t | 1.5 t | W10 on the north wall under a sunshade; terrace roof above |
| Master bed | 199.5 | 199.5 | west (open) | 49 | 1.60 | roof +15%, west +10% | 2.00 | 2 t | 1.5 t | W11 on the west wall above boundary-wall height; terrace roof above |
| TV lounge | 201.0 + 30% of Lobby, rear part (5'-0" wide) + Lobby, front part (6'-3" wide) | 242.2 | east (shaded) | 56 | 1.94 | roof +15%, glass +1.2%, living +10% | 2.45 | 2 t | 2 t | W13 on the 3'-6" open shaft; one side open to the FF lobby; roof above. Need exceeds 2 t: see note below |
| Front bed (less splayed corner 2 sft) | 213.1 | 213.1 | west (open) | 121 | 1.70 | roof +15%, west +10%, 2nd wall +5%, glass +14.3% | 2.46 | 2 t | 1.5 t | corner room: W12 west + W06 upper part south (72 sft); roof above. Need exceeds 2 t: see note below |
| Room | 132.0 | 132.0 | north (open) | 35 | 1.06 | roof +15%, north -5%, 2nd wall +5% | 1.21 | 1.5 t | 1 t | mumty room on the roof: W15 north, south wall faces the open terrace; roof above |
| Total |  |  |  |  |  |  |  | 6 x 1.5 t + 4 x 2 t |  | Gree Pular DC inverter (Gree / Haier / Orient allowed, Q30) |

Changes from the Step A trial sizes: dining 1 -> 1.5 t (open to the kitchen), mumty room 1 -> 1.5 t (roof and two exposed walls), FF master 1.5 -> 2 t (roof and west window), FF front bed 1.5 -> 2 t (roof, west window and 72 sft of south glass). The FF TV lounge and the FF front bedroom calculate at about 2.4-2.5 t: 2 t is the largest wall-mounted split in this range, so fit blinds on the tall glazing (front bed) and a curtain or partition on the lounge's open side (optional item), or accept 1-2 deg C warmer on peak afternoons. AC pipe sets are in Stage 1.

## 13.4 Wi-Fi mesh, CCTV and EV charger

- Wi-Fi (grey Register J2 positions): TP-Link Deco X50 (Wi-Fi 6) mesh, 4 nodes with wired Cat6 backhaul: GF TV lounge (main node at the router/NVR point), GF lobby, FF lobby, mumty stair hall (covers the terrace). An 8-port gigabit switch at the router position links the nodes.

- CCTV: Hikvision 8-channel PoE NVR with a 2 TB surveillance disk and 7 x 4 MP IP cameras (main gate, side gate, front porch, rear porch, back gali, two passages), powered over the Cat6 runs in the grey conduits. Viewing on phones and the TV; no separate monitor.

- EV charger (Q34): 7.4 kW single-phase Type 2 wall box (RNR) at the front car porch, on its own 6 mm2 circuit with a 40 A Type A RCBO and a 40 A isolator (Stage 1).

## 13.5 Priced materials - Stage 11

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11.01 | Switches 1-way: lights, fans, exhausts, cove and profile circuits + 8 outdoor | 104 | nos | - | 104 nos | 280 per nos | 29,120 | estimated |
| 11.02 | Switches 2-way: stairs and bed-heads | 16 | nos | - | 16 nos | 420 per nos | 6,720 | estimated |
| 11.03 | Sockets 5 A: 68 indoor + 6 outdoor | 74 | nos | - | 74 nos | 450 per nos | 33,300 | estimated |
| 11.04 | Socket 15 A switched (Orange Akoya) | 17 | nos | - | 17 nos | 1,390 per nos | 23,630 | w11stop.com, Jul 2026 |
| 11.05 | DP switches 20 A: 10 ACs + 3 geysers | 13 | nos | - | 13 nos | 1,250 per nos | 16,250 | estimated |
| 11.06 | Switch-board plates: 46 indoor + 5 outdoor boards | 51 | nos | - | 51 nos | 700 per nos | 35,700 | estimated |
| 11.07 | Plate with frame, 1-2 module (single sockets, DP switches) | 53 | nos | - | 53 nos | 300 per nos | 15,900 | estimated |
| 11.08 | Weatherproof IP55 cover for outdoor sockets | 6 | nos | - | 6 nos | 600 per nos | 3,600 | estimated |
| 11.09 | Call bell push with chime | 2 | set | - | 2 sets | 3,500 per set | 7,000 | estimated |
| 11.10 | LED downlights 12 W, 133 nos (warm white 3000 K in bedrooms, 4000 K elsewhere) | 133 | nos | - | 133 nos | 1,220 per nos | 162,260 | w11stop.com, 26-06-2026 |
| 11.11 | Cove LED strip: 82.2 m in 5 rooms (drawing, 2 lounges, 2 master bedrooms) | 16.4 | nos | 5% | 18 nos | 3,200 per nos | 57,600 | estimated |
| 11.12 | Recessed aluminium profiles: 2 lines per cove room, 34.3 m | 17.1 | nos | 5% | 19 nos | 2,600 per nos | 49,400 | estimated |
| 11.13 | COB strip inside the profiles | 6.86 | nos | 5% | 8 nos | 9,100 per nos | 72,800 | Mux LED strip listing, 2026 |
| 11.14 | LED drivers 24 V 100 W: cove (1 per 8 m) + profiles + front elevation | 18 | nos | - | 18 nos | 3,500 per nos | 63,000 | estimated |
| 11.15 | Chandelier - prime-cost allowance (owner to choose) | 2 | nos | - | 2 nos | 35,000 per nos | 70,000 | estimated |
| 11.16 | Double-height pendant - prime-cost allowance | 1 | nos | - | 1 nos | 25,000 per nos | 25,000 | estimated |
| 11.17 | Dining pendant cluster - prime-cost allowance | 1 | nos | - | 1 nos | 15,000 per nos | 15,000 | estimated |
| 11.18 | Pendant light - prime-cost allowance | 1 | nos | - | 1 nos | 12,000 per nos | 12,000 | estimated |
| 11.19 | Stair wall light LED | 3 | nos | - | 3 nos | 4,500 per nos | 13,500 | estimated |
| 11.20 | Under-cabinet LED kit (kitchen) | 2 | set | - | 2 sets | 6,000 per set | 12,000 | estimated |
| 11.21 | TV-wall LED wash kit | 2 | set | - | 2 sets | 5,000 per set | 10,000 | estimated |
| 11.22 | Porch ceiling light LED 18 W, IP54 | 4 | nos | - | 4 nos | 2,000 per nos | 8,000 | estimated |
| 11.23 | Bulkhead light LED 12 W, IP65 | 7 | nos | - | 7 nos | 2,200 per nos | 15,400 | estimated |
| 11.24 | Outdoor wall light LED 12 W, IP65 | 6 | nos | - | 6 nos | 3,200 per nos | 19,200 | estimated |
| 11.25 | Lawn spike light LED 7 W | 3 | nos | - | 3 nos | 2,600 per nos | 7,800 | estimated |
| 11.26 | Gate-pillar lantern LED | 4 | nos | - | 4 nos | 5,500 per nos | 22,000 | estimated |
| 11.27 | Front-elevation LED profiles, 3 runs of 3 m (sheet 09) | 9 | m | - | 9 m | 3,500 per m | 31,500 | estimated |
| 11.28 | Photocell / timer switch for outdoor lights | 2 | nos | - | 2 nos | 2,500 per nos | 5,000 | estimated |
| 11.29 | Ceiling fan DC inverter 56 in, 35-40 W, with remote (Pak Fan Deluxe Eco Max) - 11 rooms + front porch | 12 | nos | - | 12 nos | 11,900 per nos | 142,800 | powerhouseexpress.com.pk, 2026 |
| 11.30 | Exhaust fan 8 in (GFC), bathroom - 6 baths + powder room | 7 | nos | - | 7 nos | 5,400 per nos | 37,800 | GFC listing, 2026 (from Rs 5,400) |
| 11.31 | Exhaust fan 12 in, metal, kitchen - raw kitchen and FF kitchen | 2 | nos | - | 2 nos | 7,500 per nos | 15,000 | estimated |
| 11.32 | Split AC 1.5 t DC inverter, heat and cool (Gree Pular GS-18PIT10W) - 6 rooms | 6 | nos | - | 6 nos | 178,860 per nos | 1,073,160 | Pakistani retailer listings (japanelectronics / w11stop), Jul-Aug 2026 |
| 11.33 | Split AC 2 t DC inverter, heat and cool (Gree Pular GS-24PITH11W) - 4 rooms | 4 | nos | - | 4 nos | 229,900 per nos | 919,600 | japanelectronics.com.pk, Jul-Aug 2026 |
| 11.34 | Wi-Fi 6 mesh TP-Link Deco X50, 3-pack | 1 | set | - | 1 set | 72,000 per set | 72,000 | Pakistani retailer listing, 2026 |
| 11.35 | Wi-Fi 6 mesh TP-Link Deco X50, 1-pack (4th node) | 1 | nos | - | 1 nos | 24,199 per nos | 24,199 | qeemat.com.pk (Daraz), 2026 |
| 11.36 | Gigabit switch 8-port (router / NVR position) | 1 | nos | - | 1 nos | 6,500 per nos | 6,500 | estimated |
| 11.37 | NVR 8-channel with 8 PoE ports, 4K (Hikvision DS-7608NI-Q1/8P) | 1 | nos | - | 1 nos | 36,000 per nos | 36,000 | estimated |
| 11.38 | IP camera 4 MP, IR, IP67 (Hikvision DS-2CD1043G2-I or equal) | 7 | nos | - | 7 nos | 15,000 per nos | 105,000 | w11stop.com Hikvision listings, 2026 (Rs 14,000-17,000) |
| 11.39 | Surveillance hard disk 2 TB | 1 | nos | - | 1 nos | 19,000 per nos | 19,000 | estimated |
| 11.40 | EV wall charger 7.4 kW single-phase, Type 2 (RNR) | 1 | nos | - | 1 nos | 108,000 per nos | 108,000 | w11stop.com, 13-08-2026 |
|  | Stage 11 total (purchase quantities x rates) |  |  |  |  |  | 3,401,739 |  |

# 14. Stage 12 - External works: paving, gates, railings, roof tank and pump

On site: Last, so heavy deliveries do not damage the paving (tank and pump can be bought early for site water).

| Area | Net (sft) | Tuff tile |
| --- | --- | --- |
| Front car porch + driveway strip - car tracks | 154.2 | 80 mm |
| Front car porch - rest | 154.2 | 60 mm |
| Rear car porch - car tracks | 67.2 | 80 mm |
| Rear car porch - rest | 75.5 | 60 mm |
| Ramps at main and side gates (both used by cars) | 183.3 | 80 mm |
| Left and rear passages | 265.0 | 60 mm |
| Patio beside the dining | 14.5 | 60 mm |
| Total paving (= Step A external areas) | 914.0 | 60 mm 509 + 80 mm 405 sft |

| Item | MS weight (kg) / size | Construction | Hardware |
| --- | --- | --- | --- |
| Main gate 12'-9" x 6'-6", 2 leaves with pedestrian wicket | 243 | MS frame, 16 g sheet, powder coated RAL 7016 | bearing hinges, tower bolts, aldrop, stopper, wicket lock |
| Side gate 7'-8 1/4" x 7'-0", 2 leaves | 162 | MS frame, 16 g sheet, powder coated RAL 7016 | bearing hinges, tower bolts, aldrop, stopper |
| Stair-well guard rail at the mumty floor edge | 14 rft x 3'-0" | SS 304 posts at 4 ft + 50 mm top rail, 12 mm toughened glass 2'-6" high | 4 clamps per panel, anchor bolts |
| Front balcony (FF kitchen terrace) | 15 rft x 3'-6" | SS 304 posts at 4 ft + 50 mm top rail, 12 mm toughened glass 3'-0" high | 4 clamps per panel, anchor bolts |

- Paving (Q5): 60 mm tuff tiles with 80 mm under the car tracks (wheel paths in both porches and both ramps, since both gates take cars), laid on 2" of sand over the grey sub-base.

- Gates (Q36): MS frame with 16-gauge sheet, powder coated, pedestrian wicket in the main gate. Railings ("jangla", Q37): SS 304 posts and 12 mm toughened glass at the stair-well edge on the mumty floor and on the FF front balcony.

- Water (Q35): 500-gallon triple-layer plastic roof tank ("tanki") on the grey platform with fittings; Pedrollo 1 HP pump ("motor") at the underground tank with an automatic level controller (float switches + dry-run cut-off). A pressure switch does not suit a pump that fills an open roof tank (see Remaining assumptions).

## 14.1 Priced materials - Stage 12

| Ref | Material, specification and where used | Net qty | Unit | Waste | Buy (buying units) | Rate (Rs) | Amount (Rs) | Rate source and date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12.01 | Tuff tiles 60 mm: porch areas off the tracks, passages, patio 509.3 sft | 509.3 | sft | 5% | 535 sft | 95 per sft | 50,825 | estimated |
| 12.02 | Tuff tiles 80 mm: car tracks and both ramps 404.7 sft | 404.7 | sft | 5% | 425 sft | 120 per sft | 51,000 | estimated |
| 12.03 | Chenab sand: 2" bedding + joint filling | 161.5 | cft | 10% | 178 cft | 65 per cft | 11,570 | grey estimate Rev 1 (toolsfluent.com 2026) |
| 12.04 | MS for gates: main 243 kg + side 162 kg (2"x4" 14 g frames, 1 1/2" 16 g members at 18", 16 g sheet one face) | 404.8 | kg | 7% | 434 kg | 300 per kg | 130,200 | estimated |
| 12.05 | Main-gate hardware: bearing hinges, tower bolts, aldrop, stopper, wicket lock | 1 | set | - | 1 set | 18,000 per set | 18,000 | estimated |
| 12.06 | Side-gate hardware: bearing hinges, tower bolts, aldrop, stopper | 1 | set | - | 1 set | 9,000 per set | 9,000 | estimated |
| 12.07 | Powder coating both faces of both gates, RAL 7016 anthracite - 301 sft | 300.8 | sft | - | 301 sft | 85 per sft | 25,585 | estimated |
| 12.08 | SS 304 posts at 4 ft maximum (10) | 10 | nos | - | 10 nos | 4,500 per nos | 45,000 | estimated |
| 12.09 | SS 304 top rail 50 mm | 29 | rft | 5% | 31 rft | 650 per rft | 20,150 | estimated |
| 12.10 | Toughened glass 12 mm panels made to size: 80.0 sft | 80 | sft | - | 80 sft | 1,150 per sft | 92,000 | estimated |
| 12.11 | Glass clamps, 4 per panel (8 panels) | 32 | nos | - | 32 nos | 900 per nos | 28,800 | estimated |
| 12.12 | Anchor bolts, 4 per post | 40 | nos | - | 40 nos | 60 per nos | 2,400 | estimated |
| 12.13 | Plastic water tank 500 gallon, triple layer (roof "tanki") | 1 | nos | - | 1 nos | 42,000 per nos | 42,000 | estimated |
| 12.14 | Tank fittings: float valve, overflow, bulkhead connectors, gate valves | 1 | set | - | 1 set | 6,000 per set | 6,000 | estimated |
| 12.15 | Water pump Pedrollo 1 HP (PKm 80) ("motor") | 1 | nos | - | 1 nos | 52,000 per nos | 52,000 | estimated |
| 12.16 | Automatic water-level controller with float switches and dry-run cut-off | 1 | set | - | 1 set | 6,500 per set | 6,500 | estimated |
| 12.17 | Pump fittings: non-return valve, gate valves, unions, foot valve | 1 | set | - | 1 set | 5,500 per set | 5,500 | estimated |
|  | Stage 12 total (purchase quantities x rates) |  |  |  |  |  | 596,530 |  |

# 15. Exclusions, optional items and remaining assumptions

## 15.1 Exclusions (not in the total)

- All labour: electrician, tile and stone fixing, false-ceiling fixing, carpentry and polishing, aluminium and gate fabrication and fixing, painting, plumbing second fix. Allow roughly 25-35% of the material cost for fabricated items (aluminium, joinery, gates).

- Transport and cartage, scaffolding, tools, site water and electricity, cleaning, contractor's overheads and profit, contingency.

- FESCO new 3-phase connection charges and security deposit, SNGPL gas connection, internet connection.

- Loose furniture, curtains and blinds, appliances (fridges, oven, microwave, washing machine, TV), water filter, landscaping and lawn soil.

- Everything already priced in the grey estimate Rev 1: conduits, back boxes, earth pits, solar plinths, UPVC floor-trap bodies, PPR and gas pipes, roof tiles, plaster.

## 15.2 Optional items (estimated, NOT in the total)

| No. | Item | Approx. Rs |
| --- | --- | --- |
| 1 | TV feature walls in both lounges (297.5 sft): 2'x4' porcelain slab or WPC fluted panels instead of paint | 137,445 |
| 2 | Front-elevation "wooden tile" cladding on sheet 09 (about 96 sft) | 47,520 |
| 3 | Lockable louvred aluminium door / screen for the laundry (protects the inverter and battery) | 55,000 |
| 4 | Solar water heater 150 litres on the mumty roof (cuts geyser gas and electricity) | 95,000 |
| 5 | Lightning conductor on the mumty roof with Type 1 surge protector (earth pit is in grey) | 45,000 |
| 6 | Paint on the neighbour-facing boundary faces (92 sft) | 4,863 |
| 7 | SS wall-mounted handrail on both stair flights (42 rft) | 50,987 |
| 8 | Curtain track or glass partition at the FF TV lounge open side (helps the 2-ton AC) | 25,000 |
| 9 | Philips / Ledvance downlights instead of good local make (133 nos) | 106,400 |
| 10 | 3-burner built-in hob for the raw kitchen (Canon) | 18,000 |
|  | Saving if a 10 kWh LFP battery replaces the sodium-ion modules | -135,800 |

## 15.3 Remaining assumptions (leftover doubts, flagged)

- Front (road) side taken as south (as in the grey solar layout). If north differs, re-check the AC tonnage of the west-facing rooms and the panel direction.

- AC sizes are worked out here (Stage 11) and differ from the Step A trial sizes: 6 x 1.5 t + 4 x 2 t. The FF TV lounge and FF front bedroom need about 2.4-2.5 t; 2 t units are priced with blinds/curtains advised.

- Maximum demand rises to 36.5 kW (58 A per phase); the 63 A incomer and 4-core 16 mm2 cable stay adequate. FESCO sanctioned load about 30 kW.

- DB boards: the grey Rev 1 empty enclosures are too small; the TPN/SPN boards here replace them (credit applied). They must be fixed before plaster.

- Wire lengths come from average route lengths per point (no wiring route drawing exists), likely within +/-15%. Earth is run to every point, lights included.

- Sodium-ion battery price is estimated (no retail listing). BMS compatibility with the chosen inverter must be confirmed before ordering; LFP is the fall-back.

- Open-kitchen island 4'-6" x 2'-0" with the hob on it, as drawn on sheet 01, so an island hood is priced. The raw kitchen has no built-in hob or hood (Q15 named only the open and FF kitchens); an exhaust fan is provided.

- Geysers: 2 x 35-gallon hybrid (GF, FF) + 1 x 15-gallon hybrid for the mumty bath and laundry, each with its own 20 A circuit.

- Pump control: an automatic level controller with float switches replaces the pressure switch in Q35, because the pump fills an open roof tank.

- Sliding door D27 is a door, not a window, so it gets 6 mm toughened safety glass (Q22 covered windows only).

- Grills are not fitted to the fixed laminated units W06, W07 and W09; the 6+6 laminated glass is the security layer there.

- Wardrobes as drawn on sheets 01-02: 5 dresses + FF store = 43.2 rft, full height 10'-0" with loft; none in the mumty room.

- Bath feature wall = the shorter wall behind the WC and vanity; powder room has no shower or screen.

- Floor bedding: 1" semi-dry 1:4 bed + adhesive bond coat (1 1/4" with falls in baths) fits the 1 1/2" finish allowance; if the PCC is level within 5 mm, adhesive alone can replace the bed.

- 80 mm tuff tiles under the wheel tracks of both porches and on both ramps; 60 mm elsewhere.

- External steps (4 sets) in granite treads and risers (not asked in the questionnaire).

- Main-door fanlight = storefront glazing W07 (Stage 9). Door closer only on the main door.

- Paint coverage rates are typical for good plaster; rough plaster can use 10-15% more putty and paint. Neighbour-facing boundary faces are optional.

- Colour codes are AkzoNobel (Dulux) codes; Thai Magic 3 is used as the clay accent. Confirm each code on the dealer's tinting machine.

- Step A sizes not shown on the drawings remain as assumed there: lobby lengths, foyer depth, store S-A and door D14, W04 sill, main-door leaf height, false-ceiling heights, counter runs, exterior wall lengths per elevation and gate heights.

- Rates marked "estimated" (58% of the total) have no published October 2026 price; prices are retail incl. GST and can move 5-10% between dealers.

# 16. Final check

| Check | Result |  |
| --- | --- | --- |
| Rooms | All 35 Step A rooms appear in the false-ceiling schedule (Stage 3), floor tiles or stone (Stages 4-6), paint or wall tiles (Stages 6, 10) and the light schedule (Stage 11). | PASS |
| Doors | All 30 doors + arch A01 are in the door schedule: 1 main, 17 semi-solid, 7 WPC, 3 steel, D27 aluminium (Stage 9), D09 open, A01 arch. | PASS |
| Windows | 17 windows (W01-W15, W08 and W14 x2) + 7 ventilators + D27 in the window schedule; aluminium 427 kg and glass 865 sft equal Step A. | PASS |
| Tiles | Floor areas 2,267.2 + 341.1 + 321.8 + 33.3 sft, skirting 721 rft, bath walls 1,435.6 sft, backsplash 96.2 sft and laundry dado 46.5 sft all used once. | PASS |
| Stairs, sills | 42 treads, 44 risers, 2 landings, stepped skirting, 4 external step sets, 23 sills and 12 thresholds in Stage 5. | PASS |
| Paint | Interior 8,664 sft + stair soffits 206 + ceilings 3,508; exterior 5,366 (incl. parapets, soffits); boundary 1,615 sft. Equal to Step A. | PASS |
| False ceiling | 3,036.2 sft (standard 2,681.1 + moisture-resistant 355.1) = Step A. | PASS |
| Electrical | 133 downlights, 12 fans, 9 exhausts, 10 ACs, 74 + 17 sockets, 7 cameras, 4 Wi-Fi nodes, EV charger - same point counts as Step A. | PASS |
| Solar | 23 x 650 W on the 34 grey plinths, 12 kW 3-phase hybrid, 10 kWh sodium-ion expandable to 16 kWh (Q33). | PASS |
| Answers | All 40 answers applied, including the 7 changed answers (Q9, Q14, Q20, Q22, Q24, Q28, Q33); see the table below. | PASS |
| Site order | Sections 3-14 follow the site sequence of Section 2.2 (first fix -> solar -> ceilings -> tiles -> stone -> baths -> kitchens -> doors -> windows -> paint -> second fix -> external). | PASS |
| Totals | Summary total Rs 18,592,007 = sum of the 12 stage totals = sum of 295 priced lines (re-added independently). | PASS |
| Word vs Excel | No Excel workbook was produced, on the owner's instruction of 3 Oct 2026; all figures are in this file. | N/A |
| Exclusions | Labour and the optional items are outside the total (Section 15). | PASS |

## 16.1 Questionnaire answers and where they are applied

| Q | Answer | Applied in |
| --- | --- | --- |
| Q1 | Imported Chinese polished porcelain 2'x4', grade A | Stage 4: T24P tile |
| Q2 | Polished light marble-look, one tile throughout | Stage 4: one series in all rooms, lobbies, halls |
| Q3 | 4" skirting cut from the floor tile | Stage 4: skirting strips, 5 per tile |
| Q4 | Granite treads with anti-slip grooves + white marble risers | Stage 5 |
| Q5 | 60 mm tuff tiles, 80 mm under car tracks | Stage 12 paving split |
| Q6 | Kitchen floors same 2'x4' tile, matt | Stage 4: T24M |
| Q7 | Baths 600 x 600 matt anti-slip | Stage 6 (laundry too, Stage 4) |
| Q8 | 300 x 600 glazed walls, one feature wall per bath | Stage 6 |
| Q9 | CHANGED: floor-mounted one-piece WC in all 7 baths | Stage 6: 7 one-piece WCs, no concealed cisterns |
| Q10 | Local premium brass concealed mixer + rain + hand shower | Stage 6: 6 shower sets |
| Q11 | Stone-top basin + HDHMR vanity, SS accessories, LED mirror; wall-hung basin in powder room | Stages 6 and 5 |
| Q12 | 8 mm toughened fixed glass screen | Stage 6: 6 screens |
| Q13 | Kitchen cabinets in scope: HDHMR, acrylic, Hettich | Stage 7 |
| Q14 | CHANGED: granite in all three kitchens | Stage 7: granite counters, no quartz |
| Q15 | Local hob + hood in open and FF kitchens; SS sinks with mixer | Stage 7 |
| Q16 | Solid Diyar main door, polish, 2 1/2" leaf, glazed fanlight | Stage 8 (fanlight W07 in Stage 9) |
| Q17 | Semi-solid internal doors, Diyar chowkat, veneer, PU | Stage 8: 17 doors |
| Q18 | WPC bath doors and frames | Stage 8: 7 sets |
| Q19 | Insulated steel security doors D02, D12, D28 | Stage 8 |
| Q20 | CHANGED: Dorma/Yale on main + 3 external doors; local hardware + SS hinges inside | Stage 8: hardware sets H1-H4 |
| Q21 | Local medium aluminium, charcoal, 3-track with fly-net | Stage 9 |
| Q22 | CHANGED: single 5 mm glass (frosted in baths); 6+6 laminated only W06, W07, W09 | Stage 9 glass lines |
| Q23 | MS grills on all GF and stair windows | Stage 9 (not on fixed laminated units) |
| Q24 | CHANGED: Dulux/Berger standard range; 2 putty + primer + 2 emulsion; Weathershield 2 coats | Stage 10 |
| Q25 | Recommended colour scheme | Stage 10 colour scheme, Dulux codes |
| Q26 | Texture coat on feature bands and pillars | Stage 10 texture lines |
| Q27 | Gypsum 12.5 mm on GI frame, MR in wet areas; cove/profile in drawing, lounges, master beds | Stage 3 + Stage 11 |
| Q28 | CHANGED: Pakistan Cables / Fast / Newage; Chint breakers; good local switches; no smart switches | Stages 1 and 11 |
| Q29 | 12 W LED downlights 3000-4000 K, cove and profile strips | Stage 11 |
| Q30 | ACs in scope: DC inverter Gree / Haier / Orient | Stage 11: tonnage worked out, Gree Pular |
| Q31 | DC inverter fans 35-40 W | Stage 11: 12 fans |
| Q32 | Hybrid gas/electric geysers | Stage 6: 2 x 35 gal + 1 x 15 gal |
| Q33 | CHANGED: 15 kWp + 12 kW 3-phase hybrid + 10 kWh sodium-ion, expandable to 16 kWh | Stage 2 |
| Q34 | 7.4 kW single-phase EV wall box | Stage 11 + circuit in Stage 1 |
| Q35 | Pedrollo 1 HP + pressure switch; 500-gal triple-layer tank | Stage 12 (level controller - see assumptions) |
| Q36 | MS + 16 g sheet gates, powder coated, wicket | Stage 12 |
| Q37 | SS 304 + 12 mm toughened glass railings | Stage 12 |
| Q38 | Granite 20 mm sills and thresholds | Stage 5 (+ vanity tops Stage 6) |
| Q39 | Wardrobes in scope: HDHMR + laminate, Hettich | Stage 7 |
| Q40 | Store door only; master-dress opening open | Stage 8: D14 priced, D09 no leaf |

## 16.2 Your request list and where each item is

| Requested | Where |
| --- | --- |
| 1. One-page summary | Section 1 |
| 2. Electrical: wire, DBs, MCBs, RCBO/RCD, surge, changeover | Section 3 (Stage 1) |
| 2. Electrical: switches, lights room by room, fans, ACs with tonnage, Wi-Fi, CCTV, EV | Section 13 (Stage 11) |
| 3. Solar and storage | Section 4 (Stage 2) |
| 4. Tiles per area with boxes and wastage | Section 6 (Stage 4) schedule for all tile areas |
| 5. Bathrooms | Section 8 (Stage 6) |
| 6. Kitchen | Section 9 (Stage 7) |
| 7. Doors per door | Section 10 (Stage 8) |
| 8. Windows per window | Section 11 (Stage 9) |
| 9. Paint and colour scheme | Section 12 (Stage 10) |
| 10. False ceilings | Section 5 (Stage 3) |
| 11. Marble / granite | Section 7 (Stage 5) |
| 12. Railings, gates, tank, pump, fixtures | Section 14 (Stage 12) |
| 13. Exclusions, optional items, remaining assumptions | Section 15 |
| Final check | Section 16 |